The  Virtual  Channel  Router  Version  2.0k

PUMA:035/VCR2/ADVT  Revision 1.8

Mark Debbage                      Mark Hill

November 11, 1992

1      Introduction

The Virtual Channel Router (VCR) is a software package developed at the University of

Southampton within the ESPRIT P2701 PU MA project to provide unrestricted channel

communication across networks of T4- and T8-series transputers.  It allows distributed

transputer programs to be written, compiled and configured in a topology-independent

format and then bound to a topology-dependent routing kernel at run-time.

The  system  is  based  on  a  virtual  occam  compiler/configurer  developed  by  Inmos  for

T9000/C104  simulation.   VCR  allows  virtual  channel  programming  techniques  to  be

employed on current generation hardware, easing the learning gap between today's and

tomorrows's technologies.

Version 2.0 of this popular package enhances earlier releases.  It automatically supports

arbitrary  target  topologies,  has  a  library  of  dynamic  primitives  accessible  from  occam

and C, and is fully integrated into the new Inmos occam Dx205 and C Dx214 Toolsets

(where x is 4, 5, 6 or 7 depending on the host machine).  These Toolsets use a different

object file format (TCOFF) to the Inmos D705b occam Toolset and 3L compilers (LFF);

support for code generated by these systems is limited to the Inmos conversion program

icvlink.  Version 1.8 should be used if additional compatibility is required.

2      VCR  Features

o  VCR provides occam-like channel communications over networks of T4- and T8-

series transputers.

o  The VCR routing kernel is deadlock-free and places no restrictions on message size.

o  The communications interface from occam is pure occam _ virtual outputs by !,

virtual inputs by ?  and virtual channel alternation by ALT. In addition, a library

permits the dynamic creation of virtual channels and remote procedure calls.

o  Configuration-level language is a simplification of the occam D705B Toolset stand-

ard with link placement eliminated and the processor's valency limit removed.  In

addition, a channel array constructor allows arbitrary channel slices to be passed

into user code simplifying the expression of regular topologies.

1

o  VCR user programs are completely independent of the target VCR network_the

user configuration level can be written without knowledge of the machine topology

or the target number of processors.

o  VCR applications binaries are truly portable (memory sufficing) between all ma-

chines  running  the  same  release  of  the  VCR  kernel,  provided  that  the  binaries

are processor type independent.  For example, multi-processor applications can be

developed  and  tested  on  a  single  processor  and  the  same  binary  then  run  on  a

network.

o  Latency and processor consumption are comparable to the fastest transputer routing

kernels, but no other matches the simplicity of VCR's user interface.

3      Version  2.0  Features

o  Automatic reconfiguration onto arbitrary networks.  The user generates a descrip-

tion  of  the  target  network  either  as  a  wiring  diagram  or  as  an  Inmos  checkout

file.   This  is  analysed  offline  by  a  deadlock-free  routing  generator  to  produce  a

network configuration file, which is passed into the VCR run-time as a command-

line parameter.  The routing kernel is loaded across the network as defined by this

file (dynamic loading).

o  The routing kernel can also boot up statically as a conventional multi-processor C

bootable.  In this mode compilation steps are required per target topology.

o  Full  support  for  the  new  Inmos  occam  and  C  Toolsets.   A  library  of  dynamic

primitives is supplied with occam and C bindings to permit the dynamic creation of

virtual channels and remote procedure calls (the unit of RPC is an occam separately-

compiled unit).

o  Limited support for old Toolset and other LFF code using icvlink.

o  Support for prototyping message-passing environments through ParaPET, a Parallel

Programming Environment Toolkit. This interface can be used to construct SPMD-

like environments based on library calls from C.

o  An updated virtual occam configurer from Inmos is included.

o  This  release  fixes  the  virtual  processor  and  2D  block  move  limitations  of  earlier

versions.

4      Version  1.8  Features

o  VCR  version  1.8  forms  an  extension  to  the  Inmos  D705B  occam  Toolset  and  is

available for Sun- and PC-hosted transputer networks.

o  This  release  of  VCR  is  targeted  at  intrinsically  deadlock-free  networks.   A  large

number  of  VCR  topologies  are  provided  for  both  fixed  link  and  reconfigurable

machines.  VCR can be configured for other deadlock-free networks by modifying

the router and recompiling.

2

o  The 3L and Inmos/3L scientific languages (C, Pascal and FORTRAN) can be bound

into VCR programs using library calls and an occam harness.  (Only compatibility

with v2.0 of these languages has been tested).

o  Limited source-level debugging is supported using the standard tools.

5      Availability

The Virtual Channel Router,  the accompanying documentation and the virtual occam

compiler are freely available and distributed at media cost. Hence, the system is supplied

with no warranty and support is entirely discretional.

VCR can be obtained by anonymous FTP from ftp.ecs.soton.ac.uk (152.78.64.201)

and is located in pub/transputer/vcr.

It is also distributed through,

University of Southampton

Parallel Applications Centre

2 Venture Road

Chilworth Research Centre

Southampton

SO1 7NP

United Kingdom

Tel.:  0703-760834

Fax.:  0703-760833

A media charge of $ 50.00 + VAT will be made.  This fee covers version 2.0k of the VCR

system including system software, source and documentation. Those interested in Version

1.8 should contact the Parallel Application Centre directly for further details.

3