[This is a summary of the slides presented by Peter Thompson of

INMOS Ltd, Bristol, UK at the T9000 systems workshop given at the

University of Kent on the 14th and 15th of September 1992 presented

by the World occam and Transputer User Group (WoTUG) in association

with COMETT and the Software Environments for Parallel Computing

(SEPC) Initiative.

We have edited the slides slightly to make them more understandable

without the pictures and corrected a few typographical errors.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by David Beckett (djb1@ukc.ac.uk) on 21st October 1992

]

[1]

T9000 - Communications Architecture

An Introduction

Peter Thompson - INMOS Ltd Bristol

[2]

The CSP/occam programming model is of concurrent processes connected

by synchronous channels.

This model can be used in many ways, for many different applications.

Typical ways of using it include:

o  Pipelines

o  Farms

o  Arrays

[3]

Example of a pipeline and a farm:

{Figure of a pipeline of processes}

{Figure of a farm of processes - many process "workers" connected

to one controller "farmer"}

[4]

Mapping processes onto transputers

The processes in an occam program can be executed either by:

o  A single transputer (with process communication performed via

the local memory)

o  A collection of transputers connected in a network (with process

communication performed via links)

Three different ways of using transputers to execute the component

processes of a typical occam program are:

{Figure showing mapping from (1) five processes on one processor, (2)

three processors split 1:3:1 and (3) five processors}

[5]

Current transputers implement point-to-point channels between

processes on the same processor with memory locations, and between

processors on different processors with point-to-point links.

SNAG: Only four links!

[6] & [7]

{Figure of an array of processes [i,j] computing results a[i,j] all

in parallel.  The mapping of processes to processors is done using a

2x2 square of processes to one transputer, requiring 8 links per

processor}

[8]

This means occam-style programs must be written for a specific

network, and so are not portable.

Portable software requires hiding the details of the network

topology.

Even programs for specific networks may need more channels.

[9]

One solution: introduce multiplexing processes.

This can be done automatically by the compiler.

BUT multiplexing software steals processor time and reduces link

utilisation.

[10]

SO:

In addition to its CPU, the T9000 has a hardware VIRTUAL CHANNEL

PROCESSOR (VCP) to multiplex and number of VIRTUAL links on each

physical link.

[11]

Packets:

Each (arbitrary length) message is split into a sequence of packets

which can then be interleaved on each physical link.

Each packet has the form:

(Header)(0-32 bytes of data)(EOP/EOM)

The header is a tag which distinguishes the virtual links.

Every packet ends with and end-of-packet marker (EOP) except the last

packet of a message which ends with an end-of-message marker (EOM).

[12]

How do the packets get from one transputer to another?

o  Point-to-point connection to a neighbouring transputer

o  Via a packet routing network implemented by C104s

[13] [14]

{Figures of example routing networks using C104s}

Small crossbar network

{Figure of 32 T9000s each connected directly to one C104}

32 T9000s + 1 C104: 	Diameter		1

Low-load latency	~0.5us

Bisection bandwidth	320 Mbytes/sec

{Figure of 32 T9000s each connected directly to two C104s}

32 T9000s + 2 C104s: 	Diameter		1

Low-load latency	~0.5us

Bisection bandwidth	640 Mbytes/sec

[15]

Routers

Using routers, communication channels can be established between ANY

two processes, even if they are not on directly connected

transputers.

This simplifies programming: processes can be allocated to

transputers after the program is written in order to optimise

performance.

For general purpose message-passing machines, processes can be

allocated to transputers by a compiler:

o  Removes configuration details from the program

o  Increases software portability

[16]

Message Routing

Where system-wide message routing is required, a collection of

transputers can be interconnected by a routing network constructed

from routers.

In some cases, it is convenient to construct a network from routers

and attach some transputers to its terminal links as in the clos

network:

[17]

{Figure of 16 T9000s surrounding a 4x4 square grid of 16 C104s}

16 T9000s + 16 C104s: 	Diameter		7

Low-load latency	~3.5us

Bisection bandwidth	1.2 Gbytes/sec [???]

[18]

3-Stage Clos network

{Figure of 3-stage clos-network.  32 C104s on the left connect to 16

C104s on the right.  The C104s on the left connect to 512 terminals

(T9000s)}

48 x C104:	Terminals		512

Diameter		3

Low-load latency	~1us

Bisection bandwidth	5 Gbytes/sec

[19]

Message Routing

An alternative is to construct a network such as a hypercube or an

array from a number of nodes, each node consisting of a transputer

and a router:

{Figure of one T9000 with all 4 links connected to a C104 and the

router providing 7 links North, 7 South, 7 East and 7 West out of the

node}

[20]

{Figure of a 10 dimensional hypercube.  Each corner is a node made

from one C104 plus one T9000.  Each edge of the hypercube is composed

of 2 links.  Each corner has 8 unused links (i.e. could connect 2

more T9000s to each node).}

1024 T9000s (would be 3072) with 1024 C104s

Diameter		10 [???]

Low-load latency	~5us [???]

Bisection Bandwidth	40 Gbytes/sec [???]

(would be 120Gbytes/sec [???])

{P.Thompson adds: Note that this is not an efficient use of C104s;

the same number of T9000s could be interconnected just as well with a

smaller number of C104s.}

[21]

Routers

Use of two separate chips, one to perform computing (the transputer)

and one to perform communication (the router) has advantages:

o  Transputers can be directly connected without routers in systems

which do not require message routing, so avoiding the silicon

cost and routing delays.

o  Routers can have many links (eg 32) which allow large networks

to be constructed from a small number of routers, minimising the

delay through the network.  For example, 48 such routers can

connect 512 terminals with only 3 routing delays.

o  Messages do not flow through the transputer, reducing the total

throughput of the chip interface.  This reduces the pin count,

power consumption and package costs of the transputer.

o  It supports scalable architectures in which communication

throughput must be balanced with processing throughput.  In such

architectures, overall  communication capacity must grow faster

than the total number of processors - a large machine must have

proportionately more routers.

[22]

Balance of T9000 vs T805 (1)

-------------------------------------------------------------------------

|			| T805 (20Mhz)	| T9000 (50Mhz)	| Ratio T9/T8	|

-------------------------------------------------------------------------

|MIPS (Peak)		|	20	|	200	|	10	|

|MIPS (Sustained) 	|	10	|	100	|	10	|

|                    	|		|		|		|

|MFLOPS (Peak)		|	2.86	|	25	|	8.74	|

|MFLOPS (Sustained)	|	1.5	|	15	|	10	|

|			|		|		|		|

|MBYTES/S (One link,	|	1.74	|	9.55	|	5.49	|

|Unidirectional)	|		|		|		|

|MBYTES/S (One link,	|	2.96	|	17.48	|	5.9	|

|bidirectional)		|		|		|		|

|MBYTES/S (All links,	|	11.8	|	69.9	|	5.9	|

|bidirectional)		|		|		|		|

-------------------------------------------------------------------------

|BYTES/FLOPS (All	|	7.87	|	4.66	|	0.59	|

|links,sustained)	|		|		|		|

-------------------------------------------------------------------------

[23]

Balance of T9000 vs T805 (2)

The T9000's links look slow!  But there are compensating factors:

- Efficient variable-length communications

- Hardware channel multiplexing

- Global connectivity

The lack of these costs a lot of bandwidth in T805 systems.

[24]

Efficient Variable-Length Communications

T805			T9000

[length]BYTE array:	ldl channel.address	ldl array.base

INT slice.size:		ldl slice.address	ldl channel.address

c! slice.size::array	outword			ldl slice.size

<deschedule>		vout

ldl slice.size		<deschedule>

cj .end

ldl array.base

ldl channel.address

ldl slice.size

out

<deschedule>

.end

[26]

Efficient Variable-Length Communication

T805			T9000

[length]BYTE array:	ldlp slice.size		ldl array.base

INT slice.size:		ldl channel.address	ldl channel.address

c? silce.size::array	ldc bytes.per.word	ldc channel.address

in			vin

<deschedule>		<deschedule>

ldl slice.size		ldcnt

cj .end			stl slice.length

ldl array.base

ldl channel.address

ldl slice.size

in

<deschedule>

.end

[25]

Hardware Channel Multiplexing

{Figure of T805 multiplexing all link traffic in software using

multiplexing processes.}

{Figure of T9000 doing the same with the VCP (i.e. in hardware - no

extra software processes).}

[27]

Global Connectivity

{Figure of 3x3 T805 network showing that non-local communications

needs extra through routing}

Non-local communication requires complex software, and consumes

processor time on intermediate nodes and bandwidth on intermediate

links.

{Figure of T9/C104 network - 8 T9000s are connected to one C104 in

the center}

Non-local communications require a C104!  Nothing else!

[28]

Decoupling Software From Hardware (1)

What we really want: Any program to run on any machine

{Figure of arbitrary (irregular) network of processes.  This is

mapped to a regular topology (not the same shape as the "arbitrary"

program) of processors (not T9000s!)}

(NB: all mappings are subject to there being enough memory!)

[29]

Decoupling Software From Hardware (2)

What we have (T805 etc): any program which runs on several

transputers will run on ONE transputer.

{Figure of 3x3 grid of transputers with one process per transputer

mapping to a 3x3 grid of processes placed on a single transputer}

[30]

Decoupling Software From Hardware (3)

What we get (T9000/C104): any program which runs on several

transputers will run on FEWER transputers.

{Figure of 6 T9000s connected to a C104 network with one process per

T9000}

{Figure of 3 T9000s connected to a C104 network with processes split

2:2:2}

[31]

Decoupling Software From Hardware (4)

What we get (T9000/C104): a program which runs on several transputers

will run on ANY network with as many transputers.

{Figure of T9000 program using 8 processors/processes in a cube, with

a process(or) at each corner}

{Figure of T9000 program using 8 processors arranged in a tree using

routers, with the leaves being the processors}

Program on 8 T9000s [cube]	Same program on 8 T9000s [tree]

^					 ^

\					/

-> No dependence on network topology <-

[32]

When NOT to use a C104 (1)

When the system is very small: up to 5 T9000s can be directly

connected.

{Figure of 5 T9000s in a pentagon, with each of the four links on an

processor connected to all the other processors}

But even then a C104 gives better bandwidth:

{Figure of 5 T9000s around a C104 with all 4 links from each T9000

connected to the C104 in the center}

[33]

When NOT to use a C104 (2)

When the system is dedicated to one program, when bandwidth and

connectivity are satisfied by direct connections.

{Figure of 3x3 grid of processors with links at edges wrapping round

to form a torus.}

But even then:	How do you debug this system?

There is the need for at least one channel to any

processor for interactive debugging!

[Conclusion: Even then, use a router!]