[This is a summary of the slides presented by Roger Shepherd of

INMOS Ltd, Bristol, UK at the T9000 systems workshop given at the

University of Kent on the 14th and 15th of September 1992 presented

by the World occam and Transputer User Group (WoTUG) in association

with COMETT and the Software Environments for Parallel Computing

(SEPC) Initiative.

We have edited the slides slightly to make them more understandable

without the pictures and corrected a few typographical errors.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by David Beckett (djb1@ukc.ac.uk) on 21st October 1992

]

[1]

T9000

An Introduction

Roger Shepherd - INMOS Ltd. Bristol

[2]

T9000 - Designed for concurrency.

Topics:	Virtual Channels

Memory Systems

Processor Pipeline

Process Model

Configuration

Control Links

[3]

T9000 - Virtual Channels

The links of the T2/T4/T8 generation products provide 2 channels per

link, one in each direction. Thus:

1. A single T805 can be directly connected to at most 4

others.

2. At most 8 processes on a T805 may use a channel

implemented by a link.

Even when the PHYSICAL (processor to processor) connectivity is

sufficient, the LOGICAL (process to process) connectivity may not be.

[4]

Example of an array:

{Figure of a square grid of processes, each process computing a

single element of the array of results}

{P.Thompson adds: The point here is that if we want to allocate, say,

four processes to each processor then each processor needs 8 links!}

[5]

T9000 - Virtual Channels - packetisation

The T9000 implements 64K virtual links; each virtual link provides a

virtual channel in each direction between two T9000 transputers.

-----------------------

| HEADER | DATA | EOX |

-----------------------

Packets for different channels may be interleaved over a physical

link, allowing concurrent operation of many virtual channels.

To the programmer, a virtual channel appears just like any other

channel.

[6]

T9000 - Virtual Channels - VCP

The Virtual Channels are implemented by a second processor, the

Virtual Channel Processor (VCP).

The VCP performs packetisation etc whilst the CPU continues with

other processing.

The T9000 has a separate scheduler which schedules processes which

have completed external communications on behalf of the VCP.  The

scheduler operates concurrently with the VCP and the CPU.

[7]

T9000 - Virtual Channels - Routing

In addition to being directly connected, T9000 links may be connected

via a C104 packet router.

{Figure of many T9000s connected to a single C104}

The C104 uses packet headers to route packets to their destination.

Using a network of C104s, channels may be established between

unlimited numbers of T9000s.

[8]

T9000 - Virtual Channels - Performance

At 100 Mbit/s the T9000 links achieve the following data rates:

with 1 byte headers:	    9.55 MBytes/second unidirectional

2 x 8.74 Mbytes/second bidirectional

with 2 byte headers:	    9.26 MBytes/second unidirectional

2 x 8.27 Mbytes/second bidirectional

[9]

T9000 - Memory hierarchy

-------   -------------   -----------

| VCP |   | SCHEDULER |   |   CPU   |

-------   -------------   -----------

|            |         |   |    |

|            |         ^   v    ^

|            |      ----------- |

|            |      |Workspace| |

|            |      |Cache    | |

|            |      ----------- |

|            |         |   |    |

|            |         ^   v    |

|            |         |   |    |

-------------------------------------

|          MAIN   CACHE             |

-------------------------------------

|

-------------------------------------

|       EXTERNAL  MEMORY            |

-------------------------------------

[10]

T9000 - Memory access capabilities of subsystems

CPU       - up to 4 accesses per cycle

VCP       - up to 3 accesses per cycle

Scheduler - up to 1 access per cycle

[11]

T9000 - Multibanked memory system

(Physical) address space is divided into 4 banks:

bank 0 is selected by bits 5 and 4 being 00

bank 1 is selected by bits 5 and 4 being 01

bank 2 is selected by bits 5 and 4 being 10

bank 3 is selected by bits 5 and 4 being 11

Each bank contains 4Kbytes of cache memory

All banks may operate concurrently.

[12]

T9000 - Memory architecture diagram

|          |          |          |

-----------|------o----------o----------o----------o-

|CPU       |------o----------o----------o----------o-

|          |------o----------o----------o----------o-

-----------|------o----------o----------o----------o-

|          |          |          |

-----------|      |          |          |          |

|Scheduler |------o----------o----------o----------o-

-----------|      |          |          |          |

|          |          |          |

-----------|------o----------o----------o----------o-

|VCP       |------o----------o----------o----------o-

-----------|------o----------o----------o----------o-

|          |          |          |

-----------|      |          |          |          |

|External  |------o----------o----------o----------o-

|Memory    |      |          |          |          |

-----------|      |          |          |          |

---------  ---------  ---------  ---------

|Cache  |  |Cache  |  |Cache  |  |Cache  |

|Memory |  |Memory |  |Memory |  |Memory |

---------  ---------  ---------  ---------

Bank 0     Bank 1     Bank 2     Bank 3

[13]

T9000 - Processor Pipeline

Processor operates by forming "groups" of instructions e.g.

ldl; ldnl; ldl; ldnl; add

and executing these on a pipeline.

Fetch &  Local    Address  Main     ALU/FPU  Write

Group    Cache             Cache

-------  -------  -------  -------  ------   ------

|     |  |     |->|     |->|     |->|     |  |    |

|     |->|-----|  |-----|  |-|---|  |     |->|  | |

|     |  |     |->|     |->| |   |->|     |  |  | |

-------  -------  -------  -----|-  -------  ---|--

|                         |  |               |

^  Groups                 ^  ^               V

|                         |  |               |

Read                    Read  Read          Write

[14]

T9000 - Processor performance

Speed achieved via grouping / pipelining and fast functional units.

eg. fpadd 2 cycles, fpmul 2 cycles, shsl/shr 1 cycles, mul 2-5

Peak 200 MIPS		- limited by instruction fetch rate

25 MFLOPS		- limited by add/mul time

In practice, we can achieve >50% of these figures.

[15]

T9000 - Process model enhancements

0 - Virtual Channels

1 - Per-process error handling

2 - Resource channels and Semaphores

3 - P-Processes - protection and memory management

4 - Improved access to and control of transputer scheduler

[16]

T9000 Configuration

The T9000 is highly configurable - often "soft" configurable:

External Memory Interface - bank address, width and timing

VCP - header size, number of virtual channels and link speeds

Cache - cache/RAM split

This is achieved via "configuration registers" which are accessed via

the control link or the processor executing ldconfig / stconfig

instructions.

[17]

T9000/C104 Control Links

Replacement for/enhancement to the T2/T4/T8 Reset/Error/Analyse.

Allows programming of C104 packet routers.

--------------

| Controller |-----\ /---------\ /---------\

--------------     | |         | |         |

___V_V___   ___V_V___   ___V_____

Daisy        | | | | |   | | | | |   | | | | |

chain        | ----- |   | ----- |   | ----- |

diagram      | T9000 |   | T9000 |   | T9000 |

|       |   |       |   |       |

---------   ---------   ---------

The controller sees a virtual link to each T9000.

[18]

T9000 Control Links

The T9000 Control link permits:

Peek and poke to memory (as T2/T4/T8)

Peek and poke to configuration registers

Loading of blocks of code

Start processor executing

Start VCP executing

Stop processor/VCP (analyse)

Reset

Reporting of errors