[This is a summary of the slides presented by Peter Thompson of INMOS

Ltd, Bristol, UK at the T9000 systems workshop given at the

University of Kent on the 14th and 15th of September 1992 presented

by the World occam and Transputer User Group (WoTUG) in association

with COMETT and the Software Environments for Parallel Computing

(SEPC) Initiative.

We have edited the slides slightly to make them more understandable

without the pictures and corrected a few typographical errors.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by David Beckett (djb1@ukc.ac.uk) on 21st October 1992

]

[0]

T9000 Communications

Peter Thompson - INMOS Ltd. Bristol

[1-2]

[Peter covered the T9000's communications in great detail, explaining

the layered protocol that it uses.]

MESSAGES: are exchanged between PROCESSES and may be of any length.

They are segmented into PACKETS for transmission.

PACKETS: are exchanged between PROCESSORS.  They are transmitted as

a series of TOKENS.

TOKENS: are exchanged between DEVICES and convey DATA or FLOW CONTROL

information.  Tokens are encoded for transmission.

PHYSICAL: is the transport of tokens coded for transmission over

copper wire or fibre.

[DEVICES: are the DS-Link handlers on the PROCESSORS (e.g. T9000) or

on the ROUTERS (e.g. the C104) or others (e.g. the C100 protocol

converter)]

[3]

The packet structure consists of a header followed by data and either

an end of packet token or a end of message token.

[4-5]

Packet transmission is bidirectional using synchronisation with ACK

packets flowing the other way to the data.

[6]

Virtual Links

The message is transmitted as a sequence of packets, each of which is

restricted in length to a maximum of 32 data bytes.

Each packet of the message starts with a HEADER, which is used to:

o  Route the packet to an inputting process on a remote transputer.

o  Identify the control block of the virtual link used by the

remote inputting process.

A virtual link is created by setting up a control block in each of

the two transputers: the header in each control block is set so as to

cause packets to be routed via the other control block.

[7]

Virtual Links

Each packet of a message is transferred directly

o  from the outputting process to the physical link

o  from the physical link to the inputting process (provided

that a process is waiting when the packet arrives)

When the first packet of a message starts to arrive on a virtual

link, it is possible that no process is waiting to input the messge.

In this case, the packet must be stored temporarily in a packet

buffer associated with the virtual link control block (VLCB).

This means that communication via other virtual links sharing the

same physical link is not delayed.

[8]

Packet Header Decode

o Virtual Link Control Blocks

o Buffers for process that is not ready

Memory Space

-----------------

|               |

|----------------

| User Process  |<----

|----------------    |

|               |    |

|----------------    |

| Link Buffer   |<-- |

|----------------  | |

|               |  | |

|------------------- |

>| VLCB          |-----

Header Upper   / |----------------

--------    /  | VLCB          |

Link   Header     | 105  |   /   |----------------

--------   --------  /    | VLCB          |

------>|  95  |           -/     |----------------

--------   --------       | VLCB          |

|  92  |       |----------------

--------       | Hard &        |

Header Lower     | Event Channel |

| control words |

-----------------

address = v.base + ((95-92) << v.shift)

[The VLCB size and address for each processor is configurable.

v.base and v.shift can be changed at configuration time.  The numbers

94 and 105 are the intervals for labelling the network.  See later.]

[9]

Virtual Link Control Blocks

o  8 word block used by VCP

o  Two virtual channels

o  64K virtual links require 4Mbytes memory

DataQueueLink	Packets to send

AckQueueLink	Acknowledges to send

OutputWDesc	Outputting process

InputWDesc	Inputting process

OutputLimit	Sender's data upper bound

InputLimit	Receiver's upper bound

HeaderCtrl	Packet header format

BufferPointer	Address of the 32 byte buffer

[10]

Virtual Links

Provided that there is a process waiting to input the message, an

acknowledgement packet is dispatched back along the virtual link as

soon as each packet starts to arrive.

In this way, transmission of acknowledge packets can overlap

transmission of message packets.

At the outputting end of the virtual link, the process will be

rescheduled after the last acknowledgement packet has been received.

[11-15]

T9000 Communications - Token Layer

[A description in detail of the token layer including the flow

control method]

[16-27]

DS-Link Bandwidth

DS-Links operate at a raw speed of 100 Mbits/s. This bandwidth is not

all available at the user level for several reasons:

o Each byte of information (that is, either data or header), is

encoded to 10 bits for transmission on the link;

o One outbound flow control token (4 bits) is sent for each eight

data tokens received on the inbound link;

o Each packet has a header and a terminator;

o For each packet transmitted, an ACK packet is returned.

[Calculations for the raw DS-Link bandwidth that in theory could be

achieved giving]

Unidirectional Bandwidth:

If there are S data tokens in the header of each ACK and one in the

terminator, in the asymptotic case the data rate in Mbits/sec is

D = 51200 / ( 649 + 21 * S )

Bidirectional Bandwidth:

D = 25600 / ( 345 + 21 * S )

giving theoretical results of:

S	Unidirectional	Bidirectional

-------------------------------------

1	9.55		8.74

2	9.26		8.27

[28-31]

DS-Link Bandwidth

{Graphs of the theoretical throughput of data versus message size for

different size headers (one or two bytes) and for uni- and

bi-directional communications.}

[32]

T9000 - Layered Communications Architecture

[33]

Message Routing

Each router has a number of communications links and operates as

follows:

o  It uses the header of each packet arriving on a link to

determine the link to be used to output the packet

o  It arbitrates between two (or more) packets which must both be

output through the same link and causes them to be output one

after the other.

o  It starts to output each packet as early as possible

(immediately after the output link is determined, provided that

the output link is not already in use for another packet)

[34]

Wormhole routing is a form of dynamic circuit switching:

o  The address of the packet creates a temporary circuit (the

'wormhole') through which the data flows.

o  As the tail of the packet is pulled through, the circuit

.vanishes

[35]

This means:

o  A packet can be passing through several nodes at the same time.

o  The head of the packet may be received before the whole packet

has been transmitted.

o  Thus latency is minimised and transmission can be continuous.

[36]

Low Latency Forwarding

o  Only the header is read

o  No external memory is used

o  Uses Wormhole Routing

[37]

Addressing

Every packet must carry with it the address of its destination; this

might be the address of a transputer or the address of one of a

number of a virtual channels forming input channels to a transputer.

The address must be short:

o  It is used by each router to determine the outgoing link; the

delay through the router depends on the address length.

o  It must itself be transmitted through the network and therefore

consumes network bandwidth.

It is also important that the destination link can be derived from

the address quickly and with minimal hardware.

[38]

Interval Labelling

An INTERVAL LABELLING SCHEME assigns a distinct label to each

transputer in a network.

For simplicity, the labels for an N transputer network can be numbers

in the range [0, 1, ..., N-1].

At each router in the network, each output link has an associated

INTERVAL - a set of consecutive labels.

The intervals associated with the links on a router are

non-overlapping and every label will occur in exactly one interval.

As a packet arrives at a router, the address is examined to determine

which interval contains a matching label; the packet is the forwarded

along the associated output link.

[See Slide 8 too]

[39-43]

Labelling arbitrary networks

Labellings for hypercubes and arrays provide optimal routings; each

packet takes one of the shortest paths to its destination.

Any network can be labelled so as to provide deadlock free routing;

it is only necessary to construct (and label) a spanning tree.  (But

this usually produces a non-optimal routing which cannot exploit all

of the links present in the network as a whole.)

Optimal labellings are known for the networks below:

trees

hypercubes

arrays

clos networks

rings

{Examples of interval labelling for simple networks, trees,

hypercubes and arrays.}

[44]

Header Deletion

The main disadvantages of the interval labelling system are that:

o  It does not permit arbitray routes through a network.

o  It does not allow a message to be routed through a series of

networks.

These problems can be overcome by a simple extension: HEADER DELETION.

Each outgoing link can be set to delete the header on every packet

which passes through it: the data immediately following becomes the

new header as the packet enters the next node.

[45]

Header Deletion

Header deletion can be used to minimise delays in the routing

network.

An initial header is used to route the packet to a destination

transputer; this header is deleted as it leaves the final router and

enters the transputer.

A second header is then used to identify the virtual link within the

destination transputer.

As the number of transputers is normally much less than the number of

virtual links, the initial header can be short, minimising the delay

through each router.

[46-48]

Header Deletion

Another important use of header deletion is in the construction of

hierarchical netoworks.  In a 2-dimensional array, each transputer

could be replaced with a local network of transputers.

{Figure describing hierarchical network decomposition - a grid of

C104/sub-net nodes is formed with four links North, South, East, West

and to the sub-net.  The C104 header deletes for the sub-net and then

passes the packet on.  The sub-net consists of another router

connected directly to many T9000s.  The final header is used to route

to the right processor.}

Headers are deleted as packets leave or enter a local network.  A

single header can be used to route a packet withing a local network,

whilst three headers are needed to route via the 2-dimensional array.

[49]

Univeral Routing

To eliminate the network HOT-SPOTS which commonly arise when packets

from many different sources collide at a link in a sparse network,

two phase routing is employed.

Every packet is first dispatched to a randomly chosen intermediate

destination; from the intermediate destination it continues to its

final destination.

This is a distributed algorithm - it does not require any central

coordination - so it is straightforward to implement and scales

easily.

Randomisation does not GUARANTEE a delivery time less than O(log(p))

- but it gives a sufficiently high probability to achieve the

universality result.  The processors will occasionally be held up for

a late message but not often enought to noticeably affect

performance.

[50]

Addition and Absorbtion of Random Headers

To perform the first stage routing, as a packet arrives in a

universal netowrk (e.g. from an attached transputer), a

randomly-generated header is put on the fornt.

The packet is then routed with this header until it arrives at the

routing switch with the corresponding identifier.

The header is then absorbed and the packet is routed again with its

original header.

[51]

Message Delay Distributed at Full Load

(Simulated Hypercube with Universal Routing)

{Graph presented here as an approximate table}

Time	% remaining

(usec)

-------------------

0	100

10	 30

20	 10

30	  4

40	  2

50	  1

60	 <1

70	 <1

80	 <1

90	 <1

100	 <1

[52]

{Graph with no label!}

[53-54]

{More interval labelling examples}

[55]

Conclusions

Transputers and routers are building blocks for concurrent machines.

Transputers can be connected via links to form dedicated processing

systems in which communication takes place only between directly

connected transputers.

Routers can be used to provide system-wide inter-process

communications; this:

o  simplifies programming by removing the constraints of physical

links.

o  allows processes to be allocated to transputers after a program

is written in order to optimise performance or minimise cost.

o  ensures that programs will be portable between different

machines.

[Questions and Answers]