[This is a summary of the slides presented by Peter Thompson of INMOS

Ltd, Bristol, UK at the T9000 systems workshop given at the

University of Kent on the 14th and 15th of September 1992 presented

by the World occam and Transputer User Group (WoTUG) in association

with COMETT and the Software Environments for Parallel Computing

(SEPC) Initiative.

We have edited the slides slightly to make them more understandable

without the pictures and corrected a few typographical errors.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by David Beckett (djb1@ukc.ac.uk) on 21st October 1992

]

[0]

Errors in Systems

Peter Thompson - INMOS Ltd, Bristol.

[1]

Errors in Systems (1)

In some systems it is important to know that there are no errors

e.g. a banking system

In some systems it is important to be tolerant to errors

e.g. safety critical systems

In other systems errors may be unimportant

e.g a radar system (algorithm must be robust agains bad data anyway)

or a graphics system (one wrong pixel does not make much difference)

SO system design must consider errors!

[2]

Errors in Systems (2)

For each error the designer must consider

- Probably frequency

Can this be reduced?  At what cost?

- Consequences

Can this be reduced?  At what cost?

Then minimise

Total Cost =	Cost of reducing frequency +

Cost of reducing consequences +

frequency * cost of consequences

[3]

Errors in Systems (3)

Errors for which it is not economic to pay the cost of reducing the

consequences - for example if they are infrequent - are called

"unanticipated errors".  When one does occur, the consequences may be

sever (e.g. system crashes).

Errors which are sufficiently frequent to make it worthwhile reducing

their consequences are called "anticipated errors".  The system

design must include some means of dealing with the consequences of

such errors.

[4]

T9000 Errors (1)

In the T9000, some types of errors are clearly traceable to a

particular process; these are called "attributable errors".  In the

T9000 the consequences of attributable errors can be restricted to

the process responsible.

Examples: Overflow, illegal instruction, mis-aligned access.

Other errors may be difficult to relate to any particular process;

these are called "unattributable errors".  Such errors can be either

managed by software or else cause the T9000 to halt.

Examples: Invalid external memory accesses, communications errors.

[5]

T9000 Errors (2)

The T9000 specifically detects the following errors:

1. Attributable errors:

Illegal instruction

Explicitly checked errors (e.g. csub0)

Implicitly checked errors (e.g. ldcnt)

Integer overflow

Bad alignment

Floating point exceptions

Privileged instruction } P-Process only

Access violation       } P-Process only

2. Unattributable errors

Link parity or disconnection errors

Invalid packet header

Packet length error

Invalid external memory access

[6]

Containing Software Errors (1)

The T9000 has a special type of process called a "protected process"

(P-process). Each such process is run under the control of a

supervisor process.

The consequence of an error in a P-process is simply a return of

control to the supervisor.

What would the (serious) consequences otherwise be ?

[7]

Containing Software Errors (2)

One serious consequence of an error in software is if it changes

memory belonging to a different process, or to a system function

(e.g. VLCBs).

A P-process may only access regions of memory defined by its

supervisor, so this consequence is prevented.

[8]

Containing Software Errors (3)

Another serious consequence of a software error would be an

unauthorised modification of the device configuration, or of global

data structures such as run queues, or of another processes's state

(e.g. by output).

A P-process is not permitted to execute instructions which affect the

overall state of the T9000; any attempt to do so causes a return to

the supervisor.

The supervisor can execute the privileged instruction on behalf of

the P-process if it so chooses.

[9]

Containing Software Errors(4)

The last possibly serious consequence of software error would be an

infinite loop which consumed all of the processor's resources.

A P-process is not permitted to execute for more than two timeslice

periods, regardless of what instruction it performs.

[10]

Errors in Systems Example: ATM-Switch

ATM calls arrive on lines from the outside world.  The switch accepts

them and routes them out on other lines.

Such a switch must be reliable!  In particular, a fault on one line

must not stop the other lines from working.

Thus the design must anticipate errors on the incoming lines to

ensure that the cost of their consequences is low enough.

[11]

ATM

Generic Fast Packet Switching Architecture

-----   ---------------------------   -----

-->|AT/|   |			       |-->| /A|

|M/ |-->|			       |   |/TM|-->

-----   |			       |   -----

I            |			       |            O

N    -----   |	      Switching	       |   -----    U

P -->|AT/|   |	       Network	       |-->| /A|    T

U    |M/ |-->|	      		       |   |/TM|--> P

T    -----   |          \   /  	       |   -----    U

S      .     |           \ /  	       |     .      T

.     |	 	  o	       |     .      S

.     |		 / \	       |     .

-----   |	        /   \	       |   -----

-->|AT/|   |			       |-->| /A|

|M/ |-->|			       |   |/TM|-->

-----   ---------------------------   -----

[12]

Generic ATM Switch (<100 Mbits/s)

e.g. SMDS Applications

To implement a switch, we might internally use C104s:

{Figure of a T9000/C104 ATM switch.  There is a switch consisting of

one or more C104s providing routing.  The switch connects to a single

T9000 as a controller (on it's left) and several line interfaces on

the right. The line interfaces consist of a T9000 with two links

connected to the switch and two links connected to a DS-Link to ATM

converter unit which is connected to the outside world.}

[13]

Errors in Systems Example: ATM-Switch (2)

When we consider errors on the links inside the switch, it is cheaper

to reduce their frequency (by careful engineering) than to pay an

overhead on every communication to cope with the consequences of it

failing.

[14]

Errors in Systems Example: ATM-Switch (1)

Another source of possible failure might well be the software.

One possibility is to reduce the frequency (probability) of an error

by software engineering (e.g. use a secure language such as occam).

The other possibility is to accept a high probability of an error

(e.g. use C!) but take steps to reduct its consequences.

Use the former to make a secure kernel and the latter to run a

X-Windows display of switch activity.