[This is a summary of the slides presented by Roger Shepherd of INMOS

Ltd, Bristol, UK at the T9000 Systems Workshop given at the

University of Kent on the 14th and 15th of September 1992 presented

by the World occam and Transputer User Group (WoTUG) in association

with COMETT and the Software Environments for Parallel Computing

(SEPC) Initiative.

We have edited the slides slightly to make them more understandable

without the pictures and corrected a few typographical errors.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by David Beckett (djb1@ukc.ac.uk) on 21st October 1992

]

[1]

T9000 Pipeline and Memory System

Roger Shepherd - INMOS Ltd. Bristol

[2]

T9000 Goals

Very significantly faster than T805

Aim:	5x - 10x T805-20

Achieve via:	clock speed (2.5x)

fast functional units

architecture (2x - 4x)

use pipelining

multiple instruction issue (superscalar)

Result:	200 MIPS, 25 MFLOP

In practice we can manage > 50% of these figures

[3]

Review of Pipelining

Example: very simple 3 address register machine

Instruction		Pipeline stage	      c1  c2  c3

------------------	--------------------  ----------

I1:  r1 := r2 + r3	1. Read from reg (2)  I1  I2  I3

I2:  r2 := r3 + r4	2. ALU (1)		  I1  I2

I3:  r4 := r6 + r7	3. Write to reg (2)	      I1

1 instruction issued per cycle

3 instructions operating in parallel

"1 cycle per instruction"

[4]

Pipelining the Transputer Instruction Set

Can the transputer instruction set be pipelined?

{\em ldl} forms address; then accesses memory

{\em stl} forms address; then accesses memory

Sequences like {\em ldl;ldl} are common and could be pipelined

[5]

A Pipelined Transputer

T805		x[i] := y[j]

cycles

ldl	j	2		A M		[A=ALU, M=Memory, .=Skip]

ldl	y	2		  A M

wsub		2	(1)	    . A

ldnl	0	2		        A M

ldl	i	2			  A M

ldl	x	2			    A M

wsub		2 	(1)		      . A

stnl	0	2				  A M

16	(14)    <----------------->

10

This gives (only) 1.6x (1.4x) speed-up over T805.

[6]

Grouping: Pipeline Several Instructions at Once

Consider the previous example:

ldl j; ldl y; wsub 0; ldnl 0

ldl i; ldl x; wsub 0; stnl 0

This is effectively

load  ( Mem[Wptr + 4*y] + 4*Mem[Wptr + 4*j] )

store ( Mem[Wptr + 4*x] + 4*Mem[Wptr + 4*i] )

This is what the T9000 does. (2 cycles for the above)

[7]

Beware Memory Accesses!

Note that executing

ldl j; ldl y; wsub 0; ldnl 0

ldl i; ldl x; wsub 0; stnl 0

in 2 cycles requires access to

4 locals, 2 non-locals and 2 words of instructions

in 2 cycles - 3 words per cycle!

[8]

T9000 - Processor Pipeline

The processor operates by forming "groups" of instructions

eg ldl; ldnl; ldl; ldnl; add

and executing these on a pipeline

Fetch &  Local    Address  Main     ALU/FPU  Write

Group    Cache             Cache

-------  -------  -------  -------  ------   ------

|     |  |     |->|     |->|     |->|     |  |    |

|     |->|-----|  |-----|  |-|---|  |     |->|  | |

|     |  |     |->|     |->| |   |->|     |  |  | |

-------  -------  -------  -----|-  -------  ---|--

|                         |  |               |

^  Groups                 ^  ^               V

|                         |  |               |

Read                    Read  Read          Write

[9]

Use of Pipeline by Instructions (1)

-----------------------------------------------------

local	address	main	alu	write

-----------------------------------------------------

ldc		L

ldlp		L

ldl (hit)	L

ldnl			A	M

ldnlp			A

eqc		L			A

stnl			A			W

-----------------------------------------------------

wsub / bsub		A

sum / add				A

-----------------------------------------------------

[10]

Use of Pipeline by Instructions (2)

Some instructions are implemented in terms of others

stl		ldlp; stnl

adc		ldc; add

ldl (conflict)	ldnlp; ldnl

ldl (miss)	ldnlp; ldnl; copyAtoLocalCache

[11]

Use of Pipeline by Instructions (3)

-----------------------------------------------------

local	address	main	alu	write

-----------------------------------------------------

eqc		L			A

j		X

cj (local)		X		A

cj (other)				X

eqc 0; cj (local)	A		A

eqc 0; cj (other)			A

The instruction targetted by the jump can execute 3 cycles after the

jump (X) has been taken.

[12]

T9000 - Use of Pipeline (1)

[Consider the following example:]

typedef struct {float real, im;} complex;

complex *x, *y, *z;

x->real = (y->real * z->real) - (y->im * z->im);

[13]

T9000 - Use of Pipeline (2)

[This maps to the following code and groups as follows:]

---------------------------------------------------------

ldl		1	-- y	GROUP 1 - 4 instructions

fpldnlsn				  6 bytes

ldl		0	-- z

fpldnlmulsn

---------------------------------------------------------

ldl		1	-- y	GROUP 2 - 6 instructions

ldnlp		1		 	  8 bytes

fpldnlsn

ldl		0	-- z

ldnlp		1

fpldnlmulsn

---------------------------------------------------------

fpsub				GROUP 3 - 3 instructions

ldl		2	-- x		  5 bytes

fpstnlsn

---------------------------------------------------------

...

[14]

T9000 - Use of Pipeline (3)

G1: ldl fpldnlsn ldl fpldnlmulsn

G2: ldl ldnlp fpldnlsn ldl ldnlp fpldnlmulsn

G3: fpsub ldl fpstnlsn

Step 1		Step 2		Step 3

------------------------------------------------------------------

L	G1: ldl,ldl	G2: ldl,ldl	G3: ldl

A			G1:		G2: ldnlp, ldnlp

N-L					G1: fpldnlsn, fpldnlsnmul

FPU

W

------------------------------------------------------------------

1 cycle		1 cycle		1 cycle

[15]

T9000 - Use of Pipeline (4)

Step 4		Step 5		Step 6		Step 7

---------------------------------------------------------------------

L

A	G3:

N-L	G2: fpldnlsn,	G3:

fpldnlsnmul

FPU	G1: fpldnlsnmul	G2: fpldnlsnmul	G3: fpsub

W			G1:		G2:		G3: fpstnlsn

---------------------------------------------------------------------

2 cycles	2 cycles	2 cycles	1 cycle

6 cycles per 3 groups

per 13 instructions gives > 2 instructions/cycle

gives about 108 MIPS

per 3 fp operations gives 25 MFLOPS

[16]

Question:

"Does the hardware grouper imply that good compilers need not be

expensive and that programmers will not have to craft careful

assembler inserts to sustain high efficiency in the pipeline?"

[17]

Answer:

The compiler must be targetted at the T9000:

Issues: grouping and addressing/arithmetic

char *p, c;

c = *(p+1);

Either

ldl p; ldc 1; bsub; lb	1 group - 1 cycle

OR

ldl p; adc 1; bsub; lb	2 groups - 3 cycles [since adc=ldc+add ?]

[Note: discussion with the audience concluded that the main thing to

ensure was that you generate address-manipluating instructions (e.g.

ldc) to compute addresses, NOT arithmetic instructions (e.g. adc) to

compute addresses (even though the arithmetic instructions may use

less bytes of code-space).  This is really all the transputer

compiler writer nees to appreciate about the T9000 pipeline

architecture!]

[18]

T9000 - Memory Hierarchy

-------   -------------   -----------

| VCP |   | SCHEDULER |   |   CPU   |

-------   -------------   -----------

|            |         |   |    |

|            |         ^   v    ^

|            |      ----------- |

|            |      |Workspace| |

|            |      |Cache    | |

|            |      ----------- |

|            |         |   |    |

|            |         ^   v    |

|            |         |   |    |

-------------------------------------

|          MAIN   CACHE             |

-------------------------------------

|

-------------------------------------

|       EXTERNAL  MEMORY            |

-------------------------------------

[19]

T9000 - Memory Access Capabilities of Subsystems

CPU       - up to 4 accesses per cycle

VCP       - up to 3 accesses per cycle

Scheduler - up to 1 access per cycle

[20]

T9000 - Multibanked Memory System

(Physical) address space is divided into 4 banks:

bank 0 is selected by bits 5 and 4 being 00

bank 1 is selected by bits 5 and 4 being 01

bank 2 is selected by bits 5 and 4 being 10

bank 3 is selected by bits 5 and 4 being 11

Each bank contains 4Kbytes of cache memory

All banks may operate concurrently.

[21]

T9000 - Memory Architecture Diagram

|          |          |          |

-----------|------o----------o----------o----------o-

|CPU       |------o----------o----------o----------o-

|          |------o----------o----------o----------o-

-----------|------o----------o----------o----------o-

|          |          |          |

-----------|      |          |          |          |

|Scheduler |------o----------o----------o----------o-

-----------|      |          |          |          |

|          |          |          |

-----------|------o----------o----------o----------o-

|VCP       |------o----------o----------o----------o-

-----------|------o----------o----------o----------o-

|          |          |          |

-----------|      |          |          |          |

|External  |------o----------o----------o----------o-

|Memory    |      |          |          |          |

-----------|      |          |          |          |

---------  ---------  ---------  ---------

|Cache  |  |Cache  |  |Cache  |  |Cache  |

|Memory |  |Memory |  |Memory |  |Memory |

---------  ---------  ---------  ---------

Bank 0     Bank 1     Bank 2     Bank 3

[22]

"Devices" and "Memory"

Devices		affected by accesses, e.g. csr of UART

order of access to different addresses matters

spurious reads matter

cannot be cached

Memory		returns last value written to a given address

order of access to different addresses does not matter

spurious reads do not matter

can be cached

[23]

T9000 [Memory Access] Behaviour

The T9000 accesses memory out of order

e.g. ldl x; ldl y; ldnl z; add performs the accesses as PAR(x,y); z

The T9000 accesses memory speculatively

IF

w = 1

x := y

TRUE

x := z

ldl w; eqc 1; cl L; ldl y; stl z; .L

ldl y will occur even if the conditional jump is taken! [because of

the grouping in the pipeline]

[24]

T9000 Device Accesses

Normal loads and stores are, therefore, unsuitable for accessing

devices.

The T9000 provides special load and store instructions to access

devices

- they execute in order

- there is no speculative execution

[Note: the compiler writer must be aware of when she is dealing with

memory and when she is dealing with a device.  This is easy for occam

compiler writers (since the language distinguishes between these

things) ... but difficult for C!]

The T9000 EMI [External Memory Interface] allows banks to be marked

as "device"

- allows access from links

- allows access from processor "device" instructions only

- allows no speculative access

- is uncached

[25]

T9000 Cache Coherency

The T9000's internal memory is kept coherent.

There is no hardware implementation of cache coherency for external

memory.

Options: uncached memory

software control via {\em flush dirty cache address}

{\em flush dirty cache line}

{\em invalidate cache address}

{\em invalidate cache line}

[26]

T9000 Cache [Summary]

4K bytes per block - 16K total

Each bank - fully associative (256-way)

- 4 words per line (16 bytes)

- write back

- allocate on write miss

The cache may be used as - 100% cache

-  50% cache and 50% RAM

- 100% RAM

When used as RAM, the addresses are programmable.

[27]

Question [to the audience]

"How much arithmetic per external store access needs to be done to

enable the cache mechanisms to keep the processor running at full

speed?  How does the cache hardware affect context switch times and

interrupt response latency?  How do we decide whether to use full,

half or zero on-chip cache?"

[A discussion followed to which the answer was mainly along the lines

of "...it depends!"]