[This is a summary of the answers to questions to Steve Doyle of

INMOS Marketing, INMOS Ltd, Bristol, UK at the T9000 Systems Workshop

given at the University of Kent on the 14th and 15th of September

1992 presented by the World occam and Transputer User Group (WoTUG)

in association with COMETT and the Software Environments for Parallel

Computing (SEPC) Initiative.

The information below is (C) Copyright 1992 INMOS Ltd, UK and is

presented with permission of INMOS.

The University of Kent at Canterbury and INMOS make no claims for the

validity of the information presented below and accepts no liability

for mistakes.

Transcribed by Vedat Demiralp (svd@ukc.ac.uk) on 13th October 1992

from the audio tapes of the answers.

]

T9000 - Questions and Answers

Steve Doyle - INMOS Ltd, Bristol

Steve Doyle (steved@inmos.co.uk), from INMOS Software Marketing was

asked a few questions. The summary of the Q&A's are:

Q: Mixed networks of T2/4/8 with T9000 processors will need special

communication hardware and software. What is your approach to mixed

networks?

A: In the short term, no mixed networks will be supported. In a year's

time we'll have a lot more scope for that. [ie At the start you will

have to use the T2/4/8 toolset for one half and the T9 toolset for

the other]

[From USENET, Steve Doyle reported that he said:

... the Toolset software would not at first release handle

completely arbitrary mixtures of Txxx and T9000 processors as a

single application (that is a single .BTL file), there will, however,

be a level of support for communicating (and booting) from a Txxx

network to a T9000 network (and vice versa). The mechanism for

connecting between OS-link and DS-link networks is the C100 device

and this gives the hardware support necessary.

]

Q: (I couldn't hear the question properly, because it was too weak a

voice,  but it was relevant to configuration and network description

language.).

A: Network Description Language has been/will be upgraded to handle

T9000 networks.

To a question about routing chips, Steve replied that there will be a

family of routing chips available eventually, ranging from probably,

16 links per chip upwards.

And a couple of non-technical questions on how to obtain T9000

documents and reference manuals( from SGS-Thompson offices) and on

the demand for the first engineering samples (demand from developers

and INMOS in-house board designers imply that there wouldn't be any

problem of selling the first few batch of T9000 chips...)