WoTUG T9000 Systems Workshop

University of Kent at Canterbury

14-15th September 1992

This is a quick summary of the proceedings of the T9000 systems

workshop given at the University of Kent on the 14th and 15th of

September 1992 presented by the World occam and Transputer User Group

(WoTUG) in association with COMETT and the Software Environments for

Parallel Computing (SEPC) Initiative.

Roger Shepherd (RMS) and Peter Thompson (PWT) from INMOS Ltd,

Bristol, UK were the invited speakers.

The first day began with an overview of what the T9000 offered,

presented by RMS. Firstly he described virtual channels, routing and

the performance of the channels.  He said that at 100Mbit/s, the

links could do 9.55Mbytes/s unidirectional and 2x 8.74Mbytes/s

bidirectional.  He then went on to discuss the memory system, how the

cache banks were connected up to the CPU, Scheduler and VCP, the

processor pipeline including instruction grouper, the process model

and the configuration which can be done via software.  Finally he

described the control links and their use.

The second talk was given by PWT in which he gave an introduction

to the communications architecture of the T9000.  Comparisons were

made between T805s and T9000s showing how the hardware

implementations of variable-length communications, multiplexing and

virtual channels cause large performance increases.  He also

described how message routing could be used and how the T9000/C104

pair make decoupling software and hardware much easier.

The next talk was given by RMS and covered the pipeline system of

the T9000.  How the instruction grouper worked was covered in detail

with examples.  The result is several instructions can be done per

cycle without the need for very clever optimizing compilers.  However

he did point out that a good T9000 compiler still needs to know about

the grouping to achieve the best results.

In the evening PWT described the memory system, the cache behaviour

and device access.

On the second day, the first talk was given by PWT and described

the communications protocol in detail.  The protocol is layered,

including physical, token, packet and message layers.  The token and

packet types and operation were described in great detail and the

workings of the virtual links presented.  The links have a low level

flow control system using tokens and parity checks.  A presentation

of the calculations for visible bandwidth, taking into account the

protocols was given (the results were given above).  Graphs of the

data throughput versus messages size showed that for reasonable size

messages, the data throughput was good.  The message routing system

(Wormhole Routing) which uses interval labelling, addressing and

header deletion was described.

The final talk, given by PWT, discussed errors in systems:

detection of errors and taking action after them.  He went on to

describe how the T9000 provides a P-process or "protected" process

that runs under the supervision of another process which gets called

when a privileged instruction is called or it tries to change global

state.  As an example he described an ATM switch made from a C104

network with T9000s interfacing to the outside world.

The workshop concluded with questions from the floor about the

T9000.  A T9000 alpha silicon (with the VCP absent) was passed around

the room, courtesy of INMOS Japan.  The VCP is on the beta silicon

and a photograph of the processor die of it was shown.

Several questions were asked at the end to the presenters and also

to Steve Doyle of INMOS marketing about the T9000.  Steve said that

in the short term there will be no support for mixed T2/4/8 and T9000

networks and the network description language used to configure

processor networks will be upgraded to handle virtual links.

Transcribed by David Beckett (djb1@ukc.ac.uk)

and Vedat Demiralp (svd@ukc.ac.uk)

October 14th 1992