Protection and error handling

The IMS T9000 extends the error handling of earlier transputers to allow

error conditions to be easily trapped and handled in software. It can run

code in a protected mode where all memory accesses are checked and certain,

privileged, instructions cannot be executed.

--------------------------------------------------------------------------

Error handling

The first transputers have only a global mechanism for trapping errors;

stopping the entire processor when an error is detected. The IMS T9000

extends this to allow a trap handler to be associated with a process to

provide more localised error handling. When an error occurs, control is

transferred to the trap handler with information about the nature of the

error and where it occurred.

The action of a trap handler will, in general, be dependent on the language

or operating system being used and will be invisible to the applications

programmer. Some languages may include support for user written error

handlers. After taking the appropriate action, for example to report or

correct the error, the trap handler can return control to the process which

caused the error which can then continue execution. Each process can have

its own trap handler, or one trap handler can be shared by several (or all)

of the processes on the transputer.

To maintain compatibility with earlier transputers, the IMS T9000 can also

run processes in a global error mode where the behaviour on error is

identical to the IMS T805. These two types of processes are known as

L-processes and G-processes (for Local and Global error handling)

respectively. Both L- and G-processes can be run in parallel, the processor

dynamically switching modes as it switches between processes. This allows

code compiled for the IMS T805 (which will always be in global error mode)