Introducing the INMOS IMS T9000 family

The INMOS IMS T9000 is the latest member of the transputer family. It is

designed to provide far higher performance and greatly improved

communications facilities.

INMOS has used advanced CMOS technology to integrate a 32-bit integer

processor, a 64-bit floating point processor, 16 Kbytes of cache memory, a

communications processor and four high bandwidth serial communications

links on a single IMS T9000 chip.

The IMS T9000 transputer excels in real-time embedded applications,

delivering exceptional single processor performance and scalable

multiprocessor capability.

The IMS T9000 is binary compatible with previous transputers. It extends

the transputer range, making it easy to upgrade and complement existing

transputer systems. There is extensive, industry standard software support

for all members of the transputer family; this includes high level language

compilers, systems software (such as real time operating systems) as well

as an extensive range of development tools.

-----------------------------------------------------------------

Performance

It is essential that any microprocessor family designed for the embedded

system market provides the required performance at low cost.

The transputer family includes a 16 bit processor, a 32 bit range  of fast

integer and floating point processors and now, the highest performance

member of the family, the IMS T9000. These are all designed to make it easy

to design low cost, high performance systems.

**  the IMS T9000 transputer boasts exceptional single processor

performance; the new superscalar CPU is capable of a peak performance of

200 MIPS and 25 MFLOPS.

**  the IMS T9000 offers sub-microsecond interrupt response and context

switch times, making it ideal for high performance real-time systems.

** the four IMS T9000 communication links provide a total of 80

Mbytes/second bidirectional bandwidth.

**  the interprocessor communications architecture gives scalable

performance - the ability to increase the performance of a system by adding

more processors.

**  the IMS T9000 implementation makes it easy for compilers to fully

exploit the superscalar performance using a range of industry standard

programming languages.

**  the IMS T9000 offers supercomputer performance at an embedded systems

price.

-----------------------------------------------------------------

Multiprocessing

For applications that demand performance that single processors cannot

provide, the IMS T9000 has complete on-chip support for multiprocessing:

** the transputer architecture includes instruction level support for the

creation and scheduling of any number of concurrent processes

** the transputer instruction set includes instructions for communicating

between concurrent processes. The same instructions are used to communicate

between processes running on a single transputer and between processes

running on separate transputers.

**  the presence of a dedicated communications processor which operates

concurrently with the main processor, makes interprocessor communications

flexible and efficient. The integration of the communications system

on-chip makes it easy to write programs for multiprocessor systems

** all the IMS T9000 transputers in a system can be initialised, loaded

with code and monitored for errors through a completely independent

communications system.

-----------------------------------------------------------------

Communications support devices

The IMS T9000 transputer is complemented by a range of communications

peripherals that extend the communications capabilities of the IMS T9000.

The IMS C1XX family ensures that any size of IMS T9000 system can be

constructed, connecting first generation and second generation transputers

and providing an interface to the outside world.

** the IMS C104 is a complete routing switch on a single chip. The IMS C104

connects 32 links to each other via a 32 by 32 way, non-blocking crossbar

switch with sub-microsecond latency. This allows simple, fast

communication between IMS T9000 transputers that are not directly

connected. Multiple IMS C104s can be connected together to make larger and

more complex networks, linking any number of IMS T9000 transputers, or any

other devices that use the link protocol.

** the IMS C100 system protocol converter converts between the first

generation transputer links and control signals and the new IMS T9000

protocol.  The IMS C100 provides an inter-networking solution for transputer

systems, allowing networks to be constructed using the optimum mix of

transputers to satisfy processing power, communication bandwidth and system

cost.

-----------------------------------------------------------------

Software

The success of any microprocessor is determined as much by the quality  of

its software development tools as by any other feature.

INMOS has over a decade of experience in developing software tools  for

transputers and for multiprocessing systems. The range of compilers and

powerful development tools support all the requirements of software

developers.

** instruction set compatibility with the first generation transputer

family means that the IMS T9000 transputer has inherited a significant

range of development and application software.

** the transputer toolset is a set of development tools for programming,

configuring and debugging mixed transputer systems.   The toolset is

available on a variety of host computers including:

IBM PC

NEC PC

VAX

Sun 3

Sun 4

**  INMOS provides a powerful, interactive debugger for debugging programs

running on networks of transputers. This provides full source level debugging

with the ability to set breakpoints in any process and on any processor, and

then to examine the state of the stopped process as well as the low-level

state of the processor.

For fast time to market, and to satisfy the diverse programming

requirements of different application areas, the toolset can be used

with a variety of industry standard compilers, all with major support

for multiprocessing.

The IMS T9000 is supported by a range of compilers including:

ANSI C

C++

Fortran

occam

Ada

**  system software support for the IMS T9000 reflects the requirements of the

embedded systems marketplace. A range of operating systems and real-time

kernels are available for the transputer including:

C Executive

VRTX

CHORUS distributed Unix

This impressive array of development tools, industry standard compilers

and system software satisfies the demands of the embedded systems

market. It also ensures that the user can benefit from a significant

reduction in the critical time to market.

-----------------------------------------------------------------

Applications

The transputer family provides unprecedented price/performance solutions

for a wide range of embedded systems applications.

The IMS T9000 transputer has been specifically developed to satisfy

the requirements of three segments of the embedded systems market:

** the imaging market comprises applications that involve the generation,

manipulation and transmission of image data. Such applications include:

Laser printers

Graphics systems

Image processing systems

Industrial inspection systems

Robotics

** the embedded computing market comprises applications that are run within a

computer environment and add overall performance and functionality to the

computer system. Such applications include:

Application accelerators: (graphics, numerical, scientific, DTP)

Disk arrays and high performance file servers

Databases

X terminals

Supercomputers

Factory automation

**  the embedded communications market can be segmented into two main areas

that require high performance microprocessors. These are:

Networking: low cost LAN interfacing - FDDI, Ethernet;

internetworking systems - bridges, gateways and routers.

Packet switching systems.

The IMS T9000 transputer is highly applicable to the communications

market due to its integrated architecture combining high performance

CPU and communication links with a packet based protocol. The IMS C104

packet routing switch has been designed to support the IMS T9000,

and is useful in a range of telecommunications switching applications.

The transputer family provides a range of price/performance solutions

for all the above applications.