Electronic Engineering Times April 22 1991

Transputer pumped up BY ROGER WOOLNOUGH

Bristol, England--SGS-Thomson took the next step in its global 32-bit

microprocessor strategy last week, unveiling further details and schedules

for the powerful H1 Transputer from its subsidiary Inmos Ltd.

SGS-Thomson--which has remained aloof from other silicon vendors in the

scramble to align with a major RISC architecture--will rely on the power,

flexibility and broad tool support of the new Transputer, dubbed T9000, to

claim a share of the emerging 32-bit embedded computing market.

But the strategy is far from a sure thing. Although the current Transputer

chips have found a following outside the U.S.--Inmos's technical director,

Ian R. Pearson, claims that some 240,000 units of the Transputer family

have been shipped, a figure that would mean the Transputer outsold all

other 32-bit RISC microprocessors last year--the architecture is still not

widely accepted. And the U.S., in particular, has been a disappointing

market for the processors. Further, the sheer scale of the design effort,

and the fact that Inmos is announcing the part with no working silicon in

hand, raise questions about the eventual manufacturability of the T9000.

Sampling of the new Transputer, and of its accompanying C104 packet-routing

switch and C100 system protocol converter, will start in the third quarter

of 1991, with market availability in Ql of 1992. First silicon of the C101

link adapter will be seen in Q4 1991, with market availability in

mid-1992.  But, for now, what Inmos has to show is a series of spectacular

benchmarks based on simulations and a 1.3 million-transistor cache test

chip, built with the three-level metal, 1.0-micron CMOS process that will

be used to fabricate the T9000.  If there are questions about the T9000's

volume production, there can be none about SGS-Thomson's determination to

make the Transputer succeed.  As evidence of the parent company's

ambitions, teams of Inmos executives traveled the globe last week to

deliver presentations in London, New York, Tokyo and San Francisco.  And

this week, SGS-Thomson and Inmos are hosting "Transputing 91," at the

Hilton Hotel, in Sunnyvale, Calif., where hundreds of engineers will

receive in-depth technical briefings on the T9000.

This scenario is very different from when Inmos launched its first

Transputer in 1985. Then, Inmos was a financially strapped small company

that made SRAMs, so a worldwide extravaganza was out of the question. The

Transputer's message spread almost by word of mouth, impaired by the

market's limited understanding of multiprocessing techniques and further

hindered by the Transputer's unique Occam language.

The second-generation T9000 was conceived before SGS-Thomson

Microelectronics acquired Inmos two years ago.  Yet, full resources to go

ahead with development only became available after the merger. But in

August of last year, Ian Pearson pledged that Inmos would remedy the

Transputer's shortcomings in its next generation, then code-named H1 (see

Aug.  13, page 1).

Last week, Inmos added information to the promise, when Pearson and his

team detailed the 50-MHz, 2 million-transistor device, boasting 10-times

performance improvement while maintaining binary compatibility with its

predecessors, the T400 and T800. At last week's disclosures Inmos was

showing only subsystem-level silicon.

The sheer performance of the part--which peaks at 200 native Mips,

according to Inmos simulations--could be enough to attract attention from

the 32-bit market. But to achieve the performance, Inmos is resorting to a

three-level-metal, l-micron CMOS process using Tungsten plugs, yielding a

transistor density of 10,000 transistors/mm2 on a 180-mm2 die--a challenge

to even the most seasoned CMOS vendor.

(The only other triple-metal 32-bit microprocessor that is currently known

to be on the drawing board is Intel's 100MHz 486, which was presented in a

technical paper at this year's International Solid-State Circuits

Conference.)

Besides the global marketing push and the aggressive performance targets,

Inmos will be stepping up a program designed to remove earlier objections

to the Transputer, by supporting industry-standard software, including

mainstream programming languages such as C, C++ and Fortran/77, as well as

Inmos's own Occam and operating systems for both distributed computing and

real-time embedded processing. Inmos is cooperating with Chorus Systems,

on its microkernel-based version of Unix for distributed systems and Ready

Systems, on its VRTX real-time kernel for embedded applications.

The software vendors will be bringing their wares to a unique

architecture. Like its predecessors, the T9000 family is based on Inmos's

concept of cellular computing. Each powerful processor has its own small

on-chip memory and is connected to the outside world by four fast serial

links. Such processing elements are commonly used in massively parallel

research computers, but Inmos claims the architecture's main strength will

be in conventional embedded computing.

"With its combination of powerful computing and communications, the T9000

is opening up new fields for the microprocessor," said Philippe Geyres,

general manager of SGS-Thomson's Programmable Products Group.  "It is

going to be valuable for such applications as real-time processing and

switching of high-definition video and of broadband ISDN. Other new

applications will be in such areas as multimedia workstations, where there

is a need for both a high degree of communication capability and huge

computing power."

It is the Transputer's combination of computing and communications that

Inmos believes gives it an edge over competitors in these applications.

The company argues that traditional CISC and RISC microprocessors, by

emphasizing increased computing performance, do not offer the balanced

solution needed for many embedded applications. Inmos believes that by

placing equal emphasis on computing and system communications,

Transputer-based designs can span the whole range from single-processor

applications through various kinds of multiprocessing, to massively

parallel architectures.

Three main elements

To achieve this range, the T9000 family combines three main elements: the

T9000 Transputer itself; a line of communications chips that allows T9000s

to link with each other, with other types of Transputers, and with

conventional bus systems; and software support for development, systems

and applications, available both from Inmos and from third parties

throughout the world.

The T9000 Transputer chip integrates on a single chip the CPU; a 16-kbyte

local memory- the communications system, which includes its own

communications controller (called the virtual channel processor) and four

serial links; and several other support functions.

The most aggressive element of the T9000 design is the execution unit,

which uses a pipelined superscalar-like architecture to process as many as

40 instructions simultaneously, reaching a peak execution rate of 200 Mips

at 50 MHz. At the head of the pipeline is a 32-instruction fetch-ahead

buffer. From this buffer, a hardware scheduler, called an instruction

grouper, picks instructions and groups them together for dispatch. The

pipeline can accept one group-up to eight instructions--on each cycle.

The actual contents of an instruction group are constrained by the

pipeline design. The pipeline can execute two local cache references, two

address computations, two main cache references, an ALU operation, and a

write or jump operation on each cycle. So a group can contain, for

example, no more than two local cache references.

In real code, instructions will seldom come in just the right order to

provide for full groups.  What happens in practice, according to Inmos, is

that the fetch logic picks up four instructions on each cycle. The grouper

will combine these as best it can, actually creating groups of from one to

three instructions most of the time. When a multiple-cycle instruction --

say, a multiply-comes along, that stalls the pipeline and gives the

grouper a chance to work ahead a little bit.

So while the theoretical upper limit for pipeline activity is 40

instructions - one eight-instruction group in each of the pipeline stages

at once a more typical scenario would have about eight instructions in the

pipeline at any one time, according to Inmos.  That would give sustained

performance somewhere between 50 and 80 Mips, assuming not too many

multiple-cycle instructions come by.

The processor contains both an integer unit, which does primarily

single-cycle operations, and a multicycle FPU, which operates on 32-bit

and 64-bit floating-point numbers as specified by the IEEE 754 standard.

While the T9000 is upward binary compatible with previous Transputers, it

in particular implements the same instruction set as the T805, but with

many additions.

Inmos estimates that a T9000 running at 50 MHz can typically execute code

compiled for the -T805 10 times faster than a 20MHz T805. But this higher

performance can be achieved without having to replace existing development

tools and software. Inmos claims that only a modest amount of work is

needed to modify compilers to produce code optimised for the T9000.

Inmos also said that it has overcome the disadvantages of some recent

implementations of pipelined and superscalar microprocessors, which

require careful programming to obtain the target performance. With the

T9000, details of the pipeline are transparent to the programmer, and the

processor appears to be the normal Transputer architecture.  In order to

support the processor's high execution rate, Inmos has devised an

extraordinary memory architecture for the T9000. The part has two separate

caches--a local one attached to the pipeline, which functions almost as an

extended register file, and a main cache.  The latter is actually the

Transputer's main memory, which is organised as 16 kbytes of fully

associative cache. The full associativity, aside from being unique in the

industry, permits the small memory to achieve upward of 98 percent hit

rates, according to the company. There are two buses between the pipeline

and local cache, and four between the pipeline and main memory, providing

enormous bandwidth to the processor.

In line with the original Transputer concept, the T9000 also has a

complete communications subsystem on chip to support interprocessor

communications.  This subsystem includes four 100-Mbit/s, full-duplex,

serial communication links, each with its own pair of DMA channels.  The

links can be directly connected between Transputers, with no external

buffering or other glue logic. Each serial link has a packet-based link

protocol, supporting a data rate of 10 Mbytes/s. This gives the T9000 a

total bidirectional communications bandwidth of 80 Mbytes/s.  Though the

T9000 is primarily dependent on its on-chip memory for performance, the

part also has provision for substantial off-chip storage. It includes an

on-chip programmable-memory interface, which can be linked to memory

without glue logic. Providing an external memory bandwidth of 200

Mbytes/s, the memory interface can address up to 4 Gbytes of memory in

four independent banks. This allows EPROM and dynamic, static, and video

RAM to be addressed directly via 64-, 32-,16- or 8-bit data buses.

The T9000 communicates to external components under the control of a 5-MHz

clock, simplifying system design and retaming compatibility with previous

Transputers.

Supporting the CPU

The T9000 processor by itself is a formidable device, requiring 2 million

transistors. But to apply the part to the most demanding communications

and computing applications, Inmos realised the need for supporting chips.

The transputer alone, with its four links would quickly starve for data in

many multiprocessing topologies. And the T9000 by itself provides no

friendly way to connect to any of the buses or devices used by the rest of

the world.

So Inmos created a line of communications peripherals called the C1XX

family. These parts provide off-chip support for multiprocessing, and

allow any size of T9000 system to be built. They also permit connections

between first-generation and second-generation Transputers, and provide an

interface standard bus systems. Prelaunch details of the ClXX family were

reported last year (see Aug 13, 1990, page 1).

The C104 is a complete packet routing switch that acts like a single-chip

PBX. This connects 32 links to each other through a nonblocking crossbar

switch with submicrosecond latency. In this way, it emulates a direct

connection between each of the devices in a T9000 network, permitting any

multiprocessing topology the application wants.

If required, multiple C104s can be connected together to make larger

networks. Any number of T9000 Transputers can be linked in this manner.

In devising its technique for routing messages across a network, Inmos has

borrowed from packet-switched data communications, exploiting such

concepts as virtual channels, header addressing and wormhole routing. The

C104 chip was developed as part of a project called Puma, within the

Common Market's Esprit research program, and it is expected to find

applications in non-Transputer system communications as well.

The second C100-series chip addresses the fact that it would be handy to

use older T800 Transputer parts--many of which were specialised for

particular I/O tasks and all of which were relatively economical with the

new T9000. The T9000 has a new communications protocol, but mixed

Transputer networks can be built using the C100 system protocol converter.

This allows the optimum combination of Transputers to be used, thereby

meeting the needs of processing power, communication bandwidth and system

cost.

The third communications chip is the C101 link adaptor, which provides a

parallel interface between a T9000 link and external systems, such as

buses, peripheral devices and other microprocessors.

The Software side

In the software support area, Inmos made it clear that it does not intend

to repeat the errors of the past. In the original Transputer introduction,

the only programming language supported was Occam, a parallel programming

language unique to the Transputer. But in recent years, Inmos has gathered

a large suite of more conventional tools around its architecture, and this

effort will directly benefit the T9000.  Since the T9000 has instruction

set compatibility with the first-generation Transputer family, use can be

made of existing development and application software. In addition, Inmos

is supporting the T9000 with a selection of specially developed compilers

for industry-standard languages and through several tool-set enhancements.

Third-party software includes compilers for C++ (Glockenspiel) and Ada

(Alsys), and real-time kernels and operating systems for distributed Unix

(Chorus), VRTX32/T (Ready Systems) and C-Executive (JMI Software

Consultants).

This week, JMI (Springhouse, Pa.) will announce its port of the

C-Executive real-time kernel to the T9000. C-Executive is a preemptive,

real-time kernel that is modeled after Unix and is specifically designed

for C programming. The kernel is coded primarily in C, making it portable.

It currently runs on 20 microprocessors, including most of the major RISC

chips, as well as the PC. Many Unix programmers like it because it

provides an easy transition from Unix to embedded systems. The Transputer

version of C-Executive comes with an optional file system and a system

debugger.

Altogether, the combination of enormous processing power and familiar

tools could be sufficiently attractive to overcome old prejudices about

the Transputer. SGS-Thomson and Inmos certainly hope so. The companies see

major opportunities for the T9000 in many areas of the 32-bit

microprocessor market, where worldwide deliveries are expected to grow

>from 20 million units this year to 67 million units by 1995.

Estimates suggest that, by then, embedded applications will account for 61

percent of the market, compared with 44 percent today. Office automation

applications will represent 47 percent of the total, with communications,

mu1timedia and military making up most of the balance in embedded uses.

The T9000 could also give a new dimension to application accelerators for

PCs and workstations. As an example of the possibilities, Inmos marketing

director Paul Strzelecki described an accelerator board with 16

interconnected T9000s, which would deliver 400 Mflops and 3,200 Mips.

Simon Loe, UK -correspondent for Electronic World News, Ron Wilson,

Nicolas Mokhoff and Ray Weiss contributed to this story.