Network communications

The use of INMOS links for directly connecting transputers has already been

described. The new link protocol not only simplifies the use of links

between processors but also provides hardware support for routing messages

across a network.

-----------------------------------------------------------------

Message routing

The VCP (virtual channel processor) on the sending IMS T9000 packetises

messages to be sent over a link and adds a header to each packet to

identify the destination process. At the receiving end, the VCP uses the

header to send the data in each packet to the intended process. These

headers can also be used for routing packets through a communication system

connecting a number of IMS T9000s together. This extends the idea of

multiple channels on a single hardware link to multiple channels through a

communications system; a communications channel can be established between

any two processes even if they are running on transputers that are not

directly connected. The header still just specifies the destination of the

packet; the programmer does not need to know how to route that message to

its destination.

Advantages for the programmer

-----------------------------

The ability to have channels between any two processes in a network  has a

number of significant advantages for the programmer. It simplifies the

description of multiprocessor systems by separating the hardware

architecture from the software configuration. The programmer doesn't need

to be concerned with the details of placing channels on links or routing

messages through the network. This removes a lot of the problems with

placing of processes on processors - the decision now can be made just on

the basis of the resources (memory size, etc.) available on each processor

without worrying about the available connectivity.

The programming model for networks of IMS T9000 transputers is unchanged

>from that for the first generation of transputers. There is, however,

greater flexibility in configuring software. An important feature is that

the hardware and software configurations, and therefore their descriptions,

can be kept completely independent. The same hardware, and the same

description of that hardware, can be used for many different programs.

Routers

-------

The routing components in a network can be separated from the processing

elements. Messages can be passed from one processor, through any number of

routing devices, to the destination processor. This creates a temporary

path through the routing system for that message so, from the programmers

point of view, there still appears to be a single channel directly

connecting a process on one transputer with a process on another.

<picture>

Figure 7.1  -- A routing system

As a packet arrives on a link, the destination address must be inspected

before the outgoing link can be determined. The time before the output link

can be determined is therefore proportional to the address length. Further,

the address itself must be transmitted through the network and consumes

network bandwidth. It is therefore important that this address be as short

as possible, both to minimise latency and maximise bandwidth.

The router needs to arbitrate between packets which arrive at the  same

time and have to be sent out of the same link. Ideally, it should start to

output the packet as soon as possible; i.e. immediately after the output

link is determined, provided that the link is not already in use by another

packet. This keeps the latency through the network small, in contrast to a

typical packet switching network which uses a `store and forward' algorithm

in which each packet is read into a buffer, the address information is

decoded and then the packet is sent out. The delay that would be introduced

by this is unacceptable in a transputer network. Also the amount of

buffering needed would make a VLSI implementation of a large routing switch

impractical.

Separating routers and processors

---------------------------------

There are a number of advantages to keeping the communications devices  and

processing elements separate in a system. Processors can be directly

connected where appropriate, which avoids the silicon costs and extra

routing delays in a small system that doesn't need to use the routers.

Also, the design of the routing devices and processing elements can be

optimised for their different roles. For example, the routing component can

have a larger number of links than would be possible if the two devices

were integrated, because the processor already needs a large number of pins

for the memory interface and other functions. Having a routing device with

many links means that large network with a small number of routers can be

built, hence minimizing cost and latency and maximizing bandwidth. If

messages had to flow through the processor, it would increase the pin

count, power consumption and packaging costs. This approach also allows the

construction of scaleable architectures where the communications throughput

and processing power can be balanced.

Parallel networks

-----------------

Because the new link architecture allows all the virtual channels  of a

transputer to use a single link, complete, system-wide connectivity can be

provided by connecting just one link from each transputer to the routing

network. This means that the IMS T9000, with its four links, can be

connected to several different networks. This can be exploited in a number

of ways. For example, two or more networks can be used in parallel to

increase bandwidth, to provide a general purpose communications network and

an independent monitoring/debugging network, or as a `user' network

running in parallel with a physically separate `system' network.

-----------------------------------------------------------------

The IMS C104

An important benefit of the IMS T9000's serial links is that it is easy to

implement a full crossbar in VLSI, even with a large number of links. The

use of a crossbar allows packets to be passing through all links at the

same time, making the best possible use of the available bandwidth.

If the routing logic can be kept simple it can be provided for all  the

input links in the router. This avoids the need to share the hardware,

which would cause extra delays when several packets arrive at the same

time. It is also desirable to avoid the need for the large number of packet

buffers commonly used in routing systems. The use of small buffers and

simple routing hardware allows a single VLSI chip to provide efficient

routing between a large number of links.

Wormhole routing

----------------

The IMS C104 (figure 7.2) is one of a family of compatible communications

support devices for the IMS T9000. It includes a full 32 x 32 non-blocking

crossbar switch, enabling messages to be routed from any of its links to

any other link. In order to minimise latency, the switch uses `wormhole

routing' - the connection through the crossbar is set up as soon as the

header has been read. The header and the rest of the packet can start being