The IMS T9000 transputer

The IMS T9000 is the latest member in the transputer family of high

performance microprocessors. It is part of a broad range of 16 and 32 bit

microprocessors with compatible instructions sets and interfaces. As well

as providing high performance processing, they are designed to be simple to

use and enable the construction of low cost systems. Transputers include

functions to enable multitasking on a single processor and the building of

multiprocessor systems.

-----------------------------------------------------------------

Overview

<picture>

Figure 2.1  -- Block diagram of IMS T9000

The IMS T9000 integrates a high performance central processing unit (CPU),

a 16 Kbyte cache, communications system and other support functions on a

single chip. The main functional blocks of the IMS T9000 are shown in

figure 2.1. The function of each of these is outlined below, more details

will be found in the following sections.

Processor

---------

The IMS T9000 CPU contains a 32 bit arithmetic and logic unit (ALU) and a

64 bit floating point unit ( FPU). The FPU operates on 32 and 64 bit

floating point numbers as specified by the IEEE 754 standard. The CPU also

includes instructions for byte and half word operations. The CPU uses 32

bit linear addressing and can address up to 4 Gbytes of memory.

The IMS T9000 is binary compatible with previous transputers. In particular

it implements the same instruction set as the IMS T805 [1] with many

additions.

The instruction set is designed for efficient execution of compiled  code

and there is a wide range of language compilers available for the

transputer including a Plum-Hall validated ANSI C compiler, a validated Ada

compiler and Fortran, occam and C++ compilers. These are complemented by a

full set of software tools for developing and debugging programs for single

transputers and networks of transputers. In addition there are a number of

system level software products, such as real time kernels and distributed

operating systems.

The transputer includes a hardware kernel for scheduling processes  and

performing communications. These operations are directly supported in the

instruction set.

The IMS T9000 can run code in protected mode. In this mode all memory

accesses are made through a memory management unit which checks and

translates addresses before using them to address the memory system.

Further, only a subset of the full instruction set may be executed,

preventing protected code from executing privileged instructions.

There is improved support for error handling over earlier transputers;

errors can be trapped and handled independently for each process in

addition to the global error handling provided previously.

Hierarchical memory system

--------------------------

The IMS T9000 includes a 16 Kbyte unified cache to provide single cycle

access to instructions and data. The cache provides a peak bandwidth of 200

Mwords/s. The CPU also includes another small cache for the most frequently

used local variables of a program which provides another 150 Mwords/s of

memory bandwidth.

The external memory interface is highly programmable, allowing large

memory systems, containing different types of devices, to be built with

little or no external logic. There are four independent sets of memory

control signals simplifying the use of different device types in the same

system. The memory can interface to 8, 16, 32 or 64 bit wide devices. The

maximum data transfer rate across the memory interface is 50 Mwords/s.

Communications system

---------------------

An important issue in multiprocessor system design is the communications

architecture. To achieve efficiency and ease of use, communications must be

properly integrated into the entire processor architecture.

The transputer hardware and instruction set provides simple and efficient

communications between processes and between processors. Both internal and

external communications are handled identically, using the same source code

and machine instructions.

To support interprocessor communications, there is a complete

communications  subsystem on chip. This includes four 100 Mbits/s

full-duplex, serial communication links each with its own pair of direct

memory access (DMA) channels. The links can be directly connected between

transputers with no external buffering or other glue logic. The use of

serial links simplifies routing of links on a circuit boards and the

interconnection of boards in a system.  A communications processor, which

manages all link communications, operates concurrently with the main CPU so

that data transfers do not adversely affect CPU operation.

Two additional links are provided for system control and monitoring.

Initialisation and booting of the processor can optionally be done through

these links.

The communications subsystem also includes four `Event' channels.  As well

as acting as interrupt inputs, these can be used, as inputs or outputs, for

more general synchronization and signalling.

Multiple internal buses

-----------------------

To support the high degree of concurrent operation on the IMS T9000, and to

maintain the high internal data rates required, there are four sets of 32

bit address and data buses internally. These provide multi-por t access to

the on-chip cache from the various functional units of the IMS T9000.

System services

---------------

The system services section provides all the general facilities necessary

for the operation of the transputer. This includes the power and ground

connections, and the clock input (5 MHz). Other important connections are a

capacitor, which is required for the on-chip phase locked loops which

generate all the internal high frequency clocks, and the processor speed

select pins which can be used to select the frequency of the internal

clocks (up to the maximum speed for a particular device). There is also a

reset input - however, as the IMS T9000 includes on-chip power-on reset

circuitry, external reset logic may not be required in an embedded control

application.

-----------------------------------------------------------------

The transputer architecture

An important design decision was that transputers should be programmed  in

a high level language. The instruction set has, therefore, been designed

for simple and efficient compilation.  The instructions are all of the same

format and chosen to give a compact representation of the operations most

frequently occurring in programs.

The CPU of the IMS T9000 contains three registers (Areg, Breg and Creg)

used for expression evaluation, which form a hardware stack. Loading a

value into the stack pushes Breg into Creg, and Areg into Breg, before

loading Areg. Storing a value from Areg pops Br eg into Areg, and Creg into

Breg. Similarly, the FPU includes a three register floating-point

evaluation stack. When values are loaded onto, or stored from, the stack

the floating-point registers push and pop in the same way as the Areg ,

Breg and Creg registers. Analysis of a large number of programs, shows that

3 registers provides an effective balance between code compactness and

implementation complexity.

<picture>

Figure 2.2  -- Processor registers and memory

The transputer has three other registers used when executing code.  These

are:

** The instruction pointer which points to the next instruction to be

executed.

** The workspace pointer which points to an area of store where local

variables are kept. This area is also used as a stack for procedure calls,

etc.

** The operand register which is used in the formation of instruction

operands.

The addresses of floating-point values are formed on the CPU stack, and

values are transferred between the addressed memory locations and the FPU

stack under the control of the CPU.

Most transputer functions use the contents of these stacks, and most

instructions reference the stacks implicitly. For example the add

instruction adds the top two values in the CPU stack, leaving the result on

the top of the stack. The use of a stack reduces the need for instructions

to specify the location of their operands which reduces the size of

instructions and hence of compiled code.

-----------------------------------------------------------------

Support for concurrent processes

Most computers have the ability to effectively run several user tasks  or

processes concurrently. These processes are created and scheduled by the

host operating system. The operating system kernel provides the ability for

processes to communicate with the operating system and with each other.

Every transputer includes a hardware kernel with the ability to execute

many software processes at the same time, to create new processes rapidly,

and to perform communication between processes within a transputer and

between processes on different transputers. All of these operations are

integrated into the hardware and instruction set of the transputer and are

very efficient. Further details of the transputers scheduling mechanism

will be found in section 5.

-----------------------------------------------------------------

Pipelined, superscalar implementation

To increase the execution rate of the transputer instruction set,  the IMS

T9000 is able to issue several instructions per cycle. A superscalar

micro-architecture was designed which implements the same high level

architecture and instruction set as the IMS T805 but with much higher

performance.

Some recent implementations of pipelined and superscalar microprocessors

have required very careful programming to obtain the claimed performance.

They require that instructions are presented to the pipeline in a sequence

that will keep the processor busy. This makes developing effective

compilers very difficult, often forcing programmers to resort to assembly

code to achieve the required performance. This puts the burden of arranging

the correct sequencing of instructions on the programmer, adding to the

development time and hence costs of a product.

<picture>

Figure 2.3  -- Block diagram of grouper and pipeline

The details of the IMS T9000 pipeline are transparent to the programmer.

The processor appears to be the simple transputer architecture described

above and straightforward code written for that programming model will get

nearly the best performance out of the processor. An optimising compiler

for the IMS T9000 can, of course, generate more efficient code if the

details of the internal architecture are taken into account.

The pipeline

------------

Instructions are executed in a five stage pipeline: the first can  fetch

two local variables; the second can perform two address calculations, for

accessing non-local or subscripted variables; the third stage can load two

non-local variables; the next can perform an ALU or FPU operation; and the

final stage can do a conditional jump or write.

A conventional pipeline is designed to allow several instructions  to be

executed simultaneously; different parts of each instruction being handled

in different stages of the pipeline. In order to allow multiple

instructions to be issued per cycle (as well as multiple instructions being

executed in each cycle) the IMS T9000 does not simply send a sequence of

instructions through the pipeline but has hardware which assembles groups

of instructions from the instruction stream. These groups are chosen to

make the best use of the available hardware and one group can be sent

through the pipeline every cycle. Instructions are put into groups in the

order that they arrive at the CPU; dependencies within the group are

handled automatically by the pipeline.

The grouper can be thought of as a hardware optimiser; it recognises

commonly occurring code sequences that the processor can execute

effectively. The design of the grouping mechanism and the pipeline is based

on analysis of the code typically generated by high level language

compilers.

An IMS T9000 running at 50 MHz can execute code compiled for the IMS T805

typically 10 times faster than a 20 MHz IMS T805. This means that existing

development tools and software can be used to generate code for the IMS

T9000. It also means that only a modest amount of work is required to

modify compilers to produce code optimised for the IMS T9000.

Grouping of instructions

------------------------

The grouping of instructions takes advantage of the high degree of

concurrency and multiple buses in the processor. For example, both caches

are multi-ported and can each support two reads by the CPU simultaneously.

This allows two <F86>load local instructions to go into one group, and the

group could also contain two sets of instructions to calculate addresses

and fetch non-local variables. These could all be combined with an

arithmetic operation such as add. More details of the transputer

instruction set can be found in [3].

As an example of how the grouper works, consider the assignment and

expression evaluation shown below. The code produced is shown along with

the number of the pipeline stages in which it is executed.

ldl   j         load local variable j

ldl   b         load base address of array b

wsub            calculate address of b[j]

ldnl 15         load value of element b[j+15]

ldl   k

ldl   c

wsub

ldnl  7         load value of c[k+7]

add             add two values on top of stack

ldl   i

ldl   a

wsub

stnl  1         store into a[i+1]

This code sequence will be executed as three groups (i.e. in 3 cycles)  as

shown below. The exact contents of each group will depend on the code which

precedes and follows this. The first group might contain other instructions

>from earlier in the instruction stream.

First group                     ldl, ldl, wsub, ldnl

Second group                    ldl, ldl, wsub, ldnl, add

third group                     ldl, ldl, wsub, stnl

Since the processor can fetch one word, containing four bytes of

instructions  and data, in each cycle it is possible to achieve a

continuous execution rate of four instructions per cycle (200 MIPS).

However, if any of the instructions require more than one cycle to execute,

then the instruction fetch mechanism can continue to fetch instructions so

that larger groups can be built up. Up to 8 instructions can be put into

one group and there may be five groups in the pipeline at any time.

Improvements over IMS T805

--------------------------

In addition to executing several instructions each cycle, the number  of

cycles required to perform many arithmetic and logical operations has been

reduced from previous transputers by adding extra hardware. This, combined

with the faster clock speed and new micro-architecture, means a ten-fold

increase in speed over the IMS T805.

In addition there is improved support for error handling, and protecting

code and data from the errant behavior of a program. The IMS T9000 provides

better access to the transputers scheduling mechanism, making it easier to

implement software kernels for a particular processing model.

-----------------------------------------------------------------

Hierarchical memory system

<picture>

Figure 2.4  -- IMS T9000 hierarchical memory system

The IMS T9000 has a complete, hierarchical memory system providing fast and

efficient access to data and instructions. There are two separate caches on

chip, a general purpose unified (code and data) cache and a small cache for

local variables.

These caches can provide fast, multi-ported access to data because they are

on chip. They also reduce the number and frequency of accesses to external

memory, allowing lower cost, slower devices to be used without degrading

performance. Finally, because the majority of external memory accesses will

be cache refills (and therefore multiple word reads and writes) fast memory

access methods, such as page mode, can be used.

Subsection 2.5.1 -- Main cache

The main cache consists of four independent banks, each containing  256

lines. Each line holds data from four consecutive words (16 bytes) in

memory. An access can be made to every bank on every cycle which, with the

multiple internal buses, means there is a very high bandwidth between the

cache and different functional units within the IMS T9000.

<picture>