Communication links

Transputer links provide a simple and regular way of interfacing to

peripherals and host systems as well as communicating between transputers.

On a single transputer, processes can communicate via channels; the

provision of links allows processes on different transputers to communicate

in the same way. The IMS C104 routing device enables this communication to

take place across a network, even between transputers that are not directly

connected.

The same communication model can be used to communicate with peripheral

devices or a host system using a link adaptor which converts from the

bit-serial protocol of the links to a parallel port.

-----------------------------------------------------------------

Using links between transputers

Transputer links can be used to implement point to point communication

between transputers. This allows transputer networks of arbitrary size and

topology to be constructed. Point to point links have many advantages over

bus based communications in a multiprocessor system:

** There is no contention for the communication mechanism, regardless of

the number of processors in the system.

** There is no capacitive load penalty as more processors are added to the

system.

** The communications bandwidth does not saturate as more communicating

devices are added to the system. Rather, the larger the number of

transputers, the greater the total communications bandwidth of the system.

** Because each transputer in a system uses its own local memory, overall

memory bandwidth is proportional to the number of transputers in the

system. This is in contrast to a large, global memory where the processors

must share the available memory bandwidth.

For small systems, the four transputer links on the IMS T9000 can provide

complete connection between up to five devices. By using additional message

routing devices such as the IMS C104, networks of any size can be built

with complete connection between all IMS T9000s. If a system does not need

complete connection or the flexibility of routing that the IMS C104

provides, then networks can be built just from directly connected

transputers.

-----------------------------------------------------------------

Advantages of using links

The advantages of using links for communication are efficiency, simplicity

and hardware independence.

Efficiency

----------

There is a separate DMA controller for every input and every output

channel which allows data to be transferred without processor involvement.

To exploit this, the transputer deschedules a process which is waiting for

a communication to complete, freeing the processor to execute another

process. When the communication is complete, the process is rescheduled,

providing automatic synchronization with the data transfer.

Simplicity

----------

The communication links are, however, very simple to use. The transputer

has simple instructions for performing input and output and these are

available to the programmer either as function/procedure calls in a high

level language or, in the case of occam, as an integral part of the

language. For example, in a C program, to transfer an array of 256 bytes

from the array data to a channel c, the following call could be used:

ChanOut (c, data, 256);

In occam, the same operation could be written as:

c ! 256::data

This output operation requires four instructions: three to load the address

of the channel, the address of the data and the number of bytes, followed

by the output instruction itself. It is worthwhile comparing this with the

complex code required to do the equivalent transfer on a traditional

microprocessor. For example, it would require a DMA controller to be

programmed and, in order to allow some degree of multitasking, it would be

necessary to set up the interrupt hardware and write an interrupt handler

to control the data transfer. All of this is done automatically by the

input and output instructions on the transputer.

As a more concrete example, consider the case of a file server running  on

a host system talking to a program running on the transputer. This would

provide the transputer program with all the host operating system

facilities such as filing system, terminal i/o etc. At the transputer end,

the communication is very simple: a single line of code, as outlined above.

At the host end, a lot of complex code (probably written in assembler) is

required to handle the data transfer, either programming a DMA controller

or polling the status registers of the memory mapped port. In the case of a

Unix system, it will also be necessary to write a device driver to

interface to the hardware.

Of course, when the communication is between two transputers, then  both

ends of the communication are equally simple.

Hardware independence

---------------------

As well as being fast and easy to use, channel communications provide  a

degree of hardware independence.

The same communication mechanism can be used to communicate between

concurrent processes, with peripherals or a host system, and even to handle

interrupts. This simplifies the development and testing of code as each

process can be functionally tested before being used in the complete

system. A good description of program development for transputers can be

found in [4].

Furthermore, exactly the same code can be used to communicate between

processes on the same transputer (using so called `soft channels') and to

communicate between transputers (using links, or `hard channels'). Not only

is the source code the same, but the same transputer instructions are used

- the transputer determines at run time whether it is using a hard or a

soft channel. This saves the programmer from having to make decisions about

the final hardware implementation while developing and testing code. The

IMS T9000 takes this separation of software from hardware one step further

than previous transputers.

-----------------------------------------------------------------

IMS T9000 links

On previous transputers the programmer was limited to assigning two

channels, one in each direction, to each link. To map a particular piece of

software onto a given hardware configuration the programmer has to map

processes to processors within the constraints of available connectivity.

The problem is illustrated in figure 6.1 where 3 channels are required

between two processors, but only a single link connection is available.

One possible solution, and one that is frequently suggested by transputer

users, is the addition of more links. However this does not really solve

the problems; there is still limited connectivity available. The number of

extra links that can be added is limited by VLSI technology. This

`solution' does not address the more general communication problems in

networks, such as communication between non-adjacent processors, or

combining networks in a simple and regular way.

<picture>

Figure 6.1  -- Multiple communication channels required between processors

Subsection 6.3.1 -- Virtual channels

The solution chosen in the IMS T9000 was to add multiplexing hardware to

allow any number of processes to use each link, so physical links can be

shared transparently. These channels which share a link are known as

`virtual channels'; they have the same behavior as software channels.

The IMS T9000 has four data communication links, each with a DMA controller

and the ability to synchronize with the scheduling of processes. The links

and DMA engines are controlled by a separate communications processor, the

virtual channel processor (VCP), which works concurrently with the CPU.

This supports practically a large number of virtual channels on each link.

<picture>

Figure 6.2  -- Shared links between IMS T9000s

Virtual links

-------------

Each message sent across a link is divided into packets. Every packet

requires a header to identify its destination process. Packets from

different messages are interleaved on the link. There are a number of

advantages to this:

** It makes the transputer simpler to use as it separates the software

configuration from the hardware. The programmer does not need to limit the

number of channels between processors or explicitly allocate channels to

links.

** Channels are, generally, not busy all the time therefore the VCP can

make better use of hardware resource by keeping the links busy with

messages from different channels.

** Messages from different channels can effectively be sent concurrently -

the processor does not have to wait for a long message to complete before

sending another.

Virtual channels are always created in pairs to form a `virtual link';

this means there is no need for a return address in packets, the

acknowledgemen ts are simply sent back along the other channel of the

virtual link.

Sending packets

---------------

The IMS T9000 sends the first packet of a message and then waits for an

acknowledgement from the receiving processor before sending the next. The

process which sent the message cannot proceed until the last packet of the

message has been acknowledged. Messages and acknowledgements from other

virtual links can be sent while waiting for an acknowledgement on a virtual

link. This ensures that a single virtual link cannot monopolize a physical

link.

<picture>

Figure 6.3  -- Multiple channels sharing a link

Receiving packets

-----------------

The initial packet of a message is acknowledged if a process has requested

a message on that virtual link. The acknowledgement can be sent as  soon as

the inputting process is identified, as long as the inputter is able to

accept another packet. This means that the entire packet does not have to

be received before the acknowledgement is sent. In this way the

acknowledgement can be received by the transmitter before all of the data

packet has been sent and the transmitter can send the next message packet

immediately.

The IMS T9000 provides one packet buffer for each virtual link so that each

input can be ready to accept an unsolicited packet. This means that other

virtual channels sharing a physical link are not delayed if one virtual

channel is not ready to input. This buffering of the first packet only

takes place if the receiving process is not ready to input, otherwise the

data is written directly to the inputting process's workspace. This buffer

is not visible to the programmer; all communications are still synchronized

at the message level.

The virtual channel processor

-----------------------------

The VCP routes messages to and from processes on IMS T9000s. It shares each

physical link between any number of processes. It also supports non-local

communications by using the IMS C104 to route messages in a network of

transputers. This can provide multiple virtual channels between any two

transputers in a network. Requests to send messages are queued by the VCP

so that the main CPU is not delayed waiting for packets to be sent.

Implementation

--------------

To achieve the speed required to match a faster processor, and to  support

the virtual channel protocol, a new, simple link standard has been

implemented. The original transputer links are referred to as oversampled

(OS) links and use a pair of wires. The IMS T9000 links have four wires for

each link (a data and strobe line in each direction) and are known as DS

links. All signals are TTL compatible.

The links are asynchronous; the receiving device synchronizes to the

incoming data. This simplifies clock distribution within a system, the

exact phase or frequency of the clock on a pair of communicating IMS T9000s

is not critical. It also means that devices with different processor speeds

can communicate.

Subsection 6.3.2 -- Levels of link protocol.

As with any communications system, the links can be be described at  a

number of levels with a hierarchy of protocols. At the highest level a

message consists of the data that the user sends down a channel from one

process to another. Any type of data or message can be sent in this way.

This communication is synchronized; it will not take place until both

processes are ready and the two processes will not continue until the

message transfer is complete.

<picture>

Figure 6.4  -- High Level protocol

Packet level protocol

---------------------

In order to transfer a message from one IMS T9000 to another, the virtual

channel processor sends it as one or more packets. This allows packets from

a number of different channels to be interleaved on the same link. Each

packet is acknowledged by the receiving IMS T9000, to maintain synchronized

communication and to limit the amount of buffering required.

Every packet has a header defining the destination address followed  by the

data bytes and, finally, an `end of packet' or `end of message' token. See

figure 6.4.This simple protocol supports messages of any length; the

receiving device knows when each packet and message ends without needing to

keep track of the number of bytes received. It also maintains

synchronization at the message level.

A packet can contain up to 32 data bytes. If a message is longer than  32

bytes then it is split up into a number of packets all, except the last,

terminated by an `end of packet' token. The last packet of the message,

which may contain less than a full 32 bytes, is terminated by an `end of

message' token.

Shorter messages can be sent in a single packet, containing 0 to 32  bytes

of data, terminated by the `end of message' token. With this protocol zero

length messages can be sent, allowing efficient synchronization between

processors.

Packet acknowledgements are sent as zero length packets terminated  with an

`end of packet' token. This type of packet can never occur as part of a

message because a zero length data packet must always be the last, and

only, packet of a message, and will therefore be terminated by an `end of

message' token.

Token level protocol

--------------------

In order to support the packet level protocol described above, a lower

level protocol is needed for encoding tokens which may contain a data byte

or control information. Each token has a parity bit plus a control bit

which is used to distinguish between data and control tokens. In addition

to the parity and control bits, data tokens contain 8 bits of data and

control tokens have two bits to indicate the token type (e.g. `end of

message').

<picture>

Figure 6.5  -- Low level protocol

Bit level protocol

------------------

At the lowest, hardware, level the signals on the data and strobe  lines of

a link encode a sequence of bit values. The protocol guarantees that only

one of the two wires will have an edge in each bit time. The levels on the

data wire give the values of the transmitted bits. The strobe signal

changes state whenever the data wire does not. These two signals encode a

clock along with the data which makes it easy to asynchronously detect the

data at the receiving end.

<picture>

Figure 6.6  -- Hardware level

The first generation of transputers use a phase locked loop to synthesize

a high frequency clock signal which is then used to sample the link data.

This is adequate for the data rates involved, but would not easily support

the bit rates of 100 Mbits/s and greater used by the IMS T9000.