Protection and error handling

The IMS T9000 extends the error handling of earlier transputers to allow

error conditions to be easily trapped and handled in software. It can run

code in a protected mode where all memory accesses are checked and certain,

privileged, instructions cannot be executed.

--------------------------------------------------------------------------

Error handling

The first transputers have only a global mechanism for trapping errors;

stopping the entire processor when an error is detected. The IMS T9000

extends this to allow a trap handler to be associated with a process to

provide more localized error handling. When an error occurs, control is

transferred to the trap handler with information about the nature of the

error and where it occurred.

The action of a trap handler will, in general, be dependent on the language

or operating system being used and will be invisible to the applications

programmer. Some languages may include support for user written error

handlers. After taking the appropriate action, for example to report or

correct the error, the trap handler can return control to the process which

caused the error which can then continue execution. Each process can have

its own trap handler, or one trap handler can be shared by several (or all)

of the processes on the transputer.

To maintain compatibility with earlier transputers, the IMS T9000 can also

run processes in a global error mode where the behavior on error is

identical to the IMS T805. These two types of processes are known as

L-processes and G-processes (for Local and Global error handling)

respectively. Both L- and G-processes can be run in parallel, the processor

dynamically switching modes as it switches between processes. This allows

code compiled for the IMS T805 (which will always be in global error mode)

to be run in parallel with code specifically compiled for the IMS T9000.

-----------------------------------------------------------------

Protected mode

The IMS T9000 can also run code in protected mode. This is designed to

allow run-time checking of programs written in `unsafe' languages such as C

and also to provide memory management. For example, C allows pointers to

data or functions. Without checks for valid pointers, these could contain

an illegal memory address such as: another process's data or code; a non

word aligned address; or a function pointer which does not point to valid

code. As no checks are defined in the language it is important to be able

to check such accesses at run time, if needed.

The protection mechanism is intended to support software development  and

debugging, and programming secure systems. It protects the user's processes

or tasks from each other and also protects an operating system kernel, or

other run time support, from user code. Although code run in this mode is

frequently referred to as a `protected process', it is not the process

which is protected but the rest of the world that is protected from errors

in the process.

Protected mode processes

------------------------

Any L-process can run a piece of code as a protected mode process (or

P-process); the processor saves the state of the L-process and starts

executing the P-process. The P-process is executed until control is

returned to the L-process because of an error, protection violation or some

other reason. It is important to realize that P-processes are not scheduled

by the transputer's own scheduler - they only run under the control of a

supervisor L-process. Any of the instructions or other events that might

cause a P-process to be descheduled, will cause control to be returned to

the supervisor. The relationship between a P-process and its supervisor is

analogous to that between an L-process and its trap-handler. In both cases

the processor can be thought of as swapping between the two pieces of code.

Executing illegal instructions

------------------------------

Because control is returned to the supervisor when a P-process attempts to

execute a privileged or illegal instruction, it is possible to provide

communication and other facilities to a P-process in a controlled way, but

one which is invisible to the programmer. For example, the input and output

instructions are privileged so, if a P-process attempts a communication

then it will trap to the supervisor L-process. This L-process can examine

the state of the P-process and, if the attempted communication is `legal',

perform the communication and return control to the P-process. The

P-process will continue as if a normal communication had occurred.

There is also a `syscall' instruction which can be used by a P-process to

explicitly request some action by the supervisor.

Memory management

-----------------

When running in protected mode, all memory accesses are checked and

translated. Each P-process can access four regions of memory. The size and

base address of each region can be set, and each can have different

protections. Each area can be given permission for code to be executed from

it and for data to be written. For example an area of memory containing

code would normally be marked with execute permission but write protected.

All addresses generated when the processor is running in protected  mode

are logical addresses. These are translated to physical addresses by

combining the low order bits of the logical address with the high order

bits from the control register for that region. The translation and

checking is done in parallel with other address generation operations and

so imposes no overhead on memory access time.

The IMS T9000's memory management can be used to implement swapping of

memory to and from disk and relocation (although it does not support

page-based virtual memory). This can be used to implement most operating

system kernels. It can also be used for `stack extension'. All the

instructions which move the workspace pointer are checked for a valid

address after the operation. If it is found that the workspace address is

no longer valid then a trap occurs, the supervisor process can then

allocate more memory for the processes stack and restart it.