Support for multiprocessing

The requirement for processing performance in embedded systems is

continuously increasing as control algorithms become more sophisticated and

as systems become more complex. In the long term, the only solution to

these ever increasing demands for performance is the use of multiple

processors to perform parallel processing.

Transputers are the only microprocessors specifically designed to  tackle

the problems of building multiprocessor systems.There are advantages other

than just performance to using multiple transputers in a system; it allows

scaleable systems to be built, where more processors can be added as demand

increases, or to provide the optimum balance of price versus performance.

The communications facilities of the transputer family can also be used to

build distributed systems where, for example, the processors are located

near the equipment or components they control and use links to communicate

with other processors in the system. In addition, transputers can be used

to build high reliability, fault tolerant systems.

Fast interrupt response and process switch

------------------------------------------

In most embedded applications, there is a need for fast real time  response

(both to external interrupts and for context switching in multitasking

systems). The design of the IMS T9000 processor exploits the presence of

the two on-chip caches by having only a small number of registers in the

CPU. This means that there is little state to be saved when an interrupt or

task switch occurs, so these operations are extremely fast. These types of

operations are very efficient on the transputer because of the hardware

scheduler.

The register stacks are duplicated so that, when a process running  on the

IMS T9000 is interrupted, the contents of the stacks do not need to be

written to memory. This results in a sub-microsecond interrupt response.

Furthermore, the duplication of the register stacks enables floating-point

arithmetic to be used in an interrupt routine without any performance

penalty.

-----------------------------------------------------------------

The transputer model of concurrency

The model of concurrency and communication implemented by the transputer

hardware is based on the ideas of communicating sequential processes. All

the features for creating processes and communicating between them are

accessible from any high level language for the transputer and are

implemented directly by the occam programming language [2].

Processes and channels

----------------------

Each process can be regarded as a black box with internal state, which  can

communicate with other processes using communication channels. Each channel

is a point to point connection between two processes. One process always

inputs from the channel and the other always outputs to it. Communication

is synchronized: the first process ready to communicate waits until the

second is also ready, then the data is copied from the outputting process

to the inputting process and both processes continue.

Each process starts, performs a number of actions and then terminates.  An

action may be a set of sequential processes performed one after another, as

in a conventional programming language, or a set of parallel processes to

be performed at the same time as one another. Since a process is itself

composed of processes, some of which may be executed in parallel, a process

may contain any amount of internal concurrency, and this may change with

time as processes start and terminate. Ultimately, all processes are

constructed from three primitive processes: assignment; input and output.

Program structure

-----------------

Figure 5.1 shows an example of a system constructed from three

communicating processes. In this case there are separate processes to

handle the external hardware (the screen and keyboard) and to execute the

main, application, process. This is a modular design - only the hardware

handling processes have to be changed if the software is moved to a new

environment, the same interface (the data sent and received on channels or

`protocol') can be presented to the application process. The keyboard

handler can be interrupt driven, only being scheduled when a character is

typed, the interrupts appearing as communications. The input and output

processes can provide buffering and other filtering of the data, all of

which is invisible to the main application process, which could even be

placed on a separate processor. This use of separate processors need not

just be for performance reasons but might be done, for instance, if there

are a large number of peripheral devices which could be better handled by a

low cost 16 bit transputer. One or more high performance transputers could

then be used for the main computing processes.

<picture>

Figure 5.1  -- Processes and channels

Example

-------

The code for creating parallel processes in C is very simple. For  example,

if the three processes in the example above are external functions, then

the following code is all that is needed to run them in parallel:

#include <<stdlib.h>

#include <<channel.h>

#include <<process.h>

/*

declare externally defined functions

*/

extern keyboard_handler (Process *p, Channel *to_app, Channel *echo);

extern screen_handler (Process *p, Channel *echo, Channel *from_app);

extern application (Process *p, Channel *to_app, Channel *from_app);

/*

declare pointers to process and channel data structures

*/

Process *kbd_p, *scrn_p, *appn_p;

Channel *to_app, *from_app, *echo;

/*

allocate and initialize channel data structures

*/

to_app   = ChanAlloc();

from_app = ChanAlloc();

echo     = ChanAlloc();

/*

allocate and initialize the process data structures

*/

kbd_p  = ProcAlloc (keyboard_handler, 0, 2, to_app, echo);

scrn_p = ProcAlloc (screen_handler, 0, 2, echo, from_app);

appn_p = ProcAlloc (application, 0, 2, to_app, from_app);

/*

now run the three processes in parallel, this call

will return when all three processes have terminated

*/

ProcPar (kbd_p, scrn_p, appn_p, NULL);<F84>

A more complete explanation of how parallel programs can be written  for

the transputer can be found in INMOS Technical Note 68, <F86><#7f>Develo

ping parallel C programs for transputers" [5].

The equivalent program in occam would be:

CHAN OF BYTE to.app, from.app, echo :

PAR

keyboard.handler (to.app, echo)

screen.handler (echo, from.app)

application (to.app, from.app)

Multiprocessor programs

-----------------------

<picture>

Figure 5.2  -- Transputers and links

Every transputer implements these concepts of concurrency and

communication.  As a result, the same model can be used to program an

individual transputer or to program a network of transputers. Figure 5.2

shows a typical network of transputers connected by serial links. When a

number of processes run on an individual transputer, the processor shares

its time between the concurrent processes, and channel communication is

implemented by moving data within memory. When this programming model is

used to program a network of transputers, each transputer executes the

process, or processes, allocated to it.

Communication between processes on different transputers is implemented

directly by transputer links. Thus the same program can be implemented on a

variety of transputer configurations, with one configuration optimized for

cost, another for performance, or another for an appropriate balance of

cost and performance as illustrated in figure 5.3.

<picture>

Figure 5.3  -- Mapping processes onto one or several transputers

-----------------------------------------------------------------

Other models of concurrency

Although the transputer has direct support for concurrent process  which

communicate via channels, it is possible to use the same features of the

transputer to build other types of multiprocessor system or to support

different scheduling models. The IMS T9000 includes a number of

instructions for manipulating the transputer process queues; these make it

simple to write real-time kernels, exploiting the efficient task switching

of the transputer architecture. There are also instructions for ensuring

that the data in the cache and in memory are consistent. These can be very

useful when implementing a shared memory system.

Shared memory

-------------

In a shared memory system, a number of processors have some sort of  common

area of memory which they can all access. This has some advantages over the

channel communication model, especially where very large amounts of data

need to be shared or moved between processors. The transputer has hardware

and software support for shared memory systems.

The PMI has a set of signals for controlling access to the external  memory

interface by an external device. This is primarily intended for use with a

DMA based co-processor. It can also be used, with external arbitration

logic, to allow all of the processors in a system to access the shared

memory.

Alternatively, there may be a number of blocks of memory that can  be

switched into the memory map of different processors under software

control. These blocks can be used for exchanging data and passing messages

between processors. To synchronize the switching of these blocks of memory

between processors, the ideal method is to pass messages over the

transputer links; as the memory is switched to a processor's address space,

it is sent a message from the previous user of the memory to inform it that

it is now the new `owner' of the memory. This allows large amounts of data

to be moved from one processor to another but without the overhead of

copying all of it over a link.

In any shared memory system, the use of a cache can be a problem.  In the

IMS T9000 there are instructions for forcing changed data in the cache to

be written out to main memory and for marking data in the cache as invalid

so that it will be read from main memory. As the exchange of data is

synchronized between processors, these instructions can be used to make

sure that the correct data is in both the main memory and the cache of the

processors involved.

It is also possible to mark banks of external memory to be `uncacheable';

data from that area of memory will never be put in the cache. This  ensures

that a number of processors or other devices which make random reads and

writes of that memory will always get the most up to date data. In this

case there must still be some synchronization of the memory accesses to

make sure that information is not read by a processor until it has been

written; again, this synchronization can be done over the transputer links.

-----------------------------------------------------------------

Hardware scheduler

The IMS T9000 processor includes a hardware scheduler which implements the

transputer model of concurrency. In many applications this will remove the

need for a software kernel. However, the transputers own scheduling

mechanisms can be accessed from software to provide efficient support for

the implementation of standard real-time kernels.

At any time, a transputer process may be:

active       -      being executed

-      on a list waiting to be executed

inactive       -      ready to input

-      ready to output

-      waiting until a specified time

The scheduler operates in such a way that inactive processes do not

consume any processor time. The active processes waiting to be executed are

held on a list of process workspaces. This is implemented using two

registers, one of which points to the first process on the list, the other

to the last. In figure 5.4, P is executing, and Q, R and S are active,

awaiting execution.

A process runs until it is unable to proceed because it is waiting  to

input or output, or waiting for the timer. Whenever a process is unable to

proceed, its instruction pointer is saved in its workspace and the next

process is taken from the list. Actual process switch times are very small

as little state needs to be saved; it is not necessary for the processor to

save the evaluation stack on descheduling.

<picture>

Figure 5.4  -- Transputer process queue

-----------------------------------------------------------------

Interrupts, events and timers

As well as process scheduling and communications, the scheduling hardware

also supports simple handling of interrupts and timers. Any event that a

process might need to wait for (whether it be a communication, an interrupt

or a timeout) can be treated in the same way as a communication. For

example, an interrupt handler simply has to wait for an input from a

special channel which is mapped onto an interrupt (`Event') input. Because

inputs are synchronized, that process will not proceed until the `input'

becomes ready, i.e. until there is an interrupt.

This makes interrupts on the transputer very easy to use. An interrupt

handler is simply a process like any other waiting on an input from the

interrupt `channel'. This contrasts greatly with the traditional idea of an

interrupt handler as something difficult which needs to use special

instructions and be written in a very different way from other program code

(usually in assembler).

The IMS T9000 has four sets of pins, known as `Event' channels, which can

be used for control and synchronization purposes. Each Event channel can be

configured either as an input or an output. As inputs they can be used as

interrupts, to cause a fast processor response to a external signals. When

an Event channel is configured as an output, the process outputting to it

will be descheduled until the external device provides the necessary

handshake signal.

The transputer has two timers; one of which `ticks' every microsecond,  the

other ticks every 64 microseconds. The current value of the processor timer

can be read, or a process can perform a<F86> timer input in which case it

will become ready to execute when a specified time has been reached. Both

these uses of the timer are treated as inputs similar to channel

communication. If the timer is simply being read then the current timer

value is provided immediately; if the process is waiting for a particular

time, then it is descheduled until that time.

-----------------------------------------------------------------

Shared resources

The IMS T9000 also provides efficient hardware support for controlling

access to a shared resource. This could be a hardware resource (e.g. a

printer) or a piece of software running on a particular processor in a

network. Each process which wants to use the resource (a `client') can make

a request to the controlling process (the `server'). This request is done

in the form of a channel communication and can, therefore, be done across a

network by using transputer links. If the resource is available then the

requesting client is given access to it, otherwise it is put on a queue

until the resource becomes free. If multiple clients request a resource

then they are all automatically queued until it is available.

<picture>

Figure 5.5  -- Client/server model of resources

The resource mechanism can provide pairs of channels between the server

and the processes accessing it. This can be used, for example, to implement

remote procedure calls across a transputer system.