Network communications

The use of INMOS links for directly connecting transputers has already been

described. The new link protocol not only simplifies the use of links

between processors but also provides hardware support for routing messages

across a network.

-----------------------------------------------------------------

Message routing

The VCP (virtual channel processor) on the sending IMS T9000 packetizes

messages to be sent over a link and adds a header to each packet to

identify the destination process. At the receiving end, the VCP uses the

header to send the data in each packet to the intended process. These

headers can also be used for routing packets through a communication system

connecting a number of IMS T9000s together. This extends the idea of

multiple channels on a single hardware link to multiple channels through a

communications system; a communications channel can be established between

any two processes even if they are running on transputers that are not

directly connected. The header still just specifies the destination of the

packet; the programmer does not need to know how to route that message to

its destination.

Advantages for the programmer

-----------------------------

The ability to have channels between any two processes in a network  has a

number of significant advantages for the programmer. It simplifies the

description of multiprocessor systems by separating the hardware

architecture from the software configuration. The programmer doesn't need

to be concerned with the details of placing channels on links or routing

messages through the network. This removes a lot of the problems with

placing of processes on processors - the decision now can be made just on

the basis of the resources (memory size, etc.) available on each processor

without worrying about the available connectivity.

The programming model for networks of IMS T9000 transputers is unchanged

from that for the first generation of transputers. There is, however,

greater flexibility in configuring software. An important feature is that

the hardware and software configurations, and therefore their descriptions,

can be kept completely independent. The same hardware, and the same

description of that hardware, can be used for many different programs.

Routers

-------

The routing components in a network can be separated from the processing

elements. Messages can be passed from one processor, through any number of

routing devices, to the destination processor. This creates a temporary

path through the routing system for that message so, from the programmers

point of view, there still appears to be a single channel directly

connecting a process on one transputer with a process on another.

<picture>

Figure 7.1  -- A routing system

As a packet arrives on a link, the destination address must be inspected

before the outgoing link can be determined. The time before the output link

can be determined is therefore proportional to the address length. Further,

the address itself must be transmitted through the network and consumes

network bandwidth. It is therefore important that this address be as short

as possible, both to minimize latency and maximize bandwidth.

The router needs to arbitrate between packets which arrive at the  same

time and have to be sent out of the same link. Ideally, it should start to

output the packet as soon as possible; i.e. immediately after the output

link is determined, provided that the link is not already in use by another

packet. This keeps the latency through the network small, in contrast to a

typical packet switching network which uses a `store and forward' algorithm

in which each packet is read into a buffer, the address information is

decoded and then the packet is sent out. The delay that would be introduced

by this is unacceptable in a transputer network. Also the amount of

buffering needed would make a VLSI implementation of a large routing switch

impractical.

Separating routers and processors

---------------------------------

There are a number of advantages to keeping the communications devices  and

processing elements separate in a system. Processors can be directly

connected where appropriate, which avoids the silicon costs and extra

routing delays in a small system that doesn't need to use the routers.

Also, the design of the routing devices and processing elements can be

optimized for their different roles. For example, the routing component can

have a larger number of links than would be possible if the two devices

were integrated, because the processor already needs a large number of pins

for the memory interface and other functions. Having a routing device with

many links means that large network with a small number of routers can be

built, hence minimizing cost and latency and maximizing bandwidth. If

messages had to flow through the processor, it would increase the pin

count, power consumption and packaging costs. This approach also allows the

construction of scaleable architectures where the communications throughput

and processing power can be balanced.

Parallel networks

-----------------

Because the new link architecture allows all the virtual channels  of a

transputer to use a single link, complete, system-wide connectivity can be

provided by connecting just one link from each transputer to the routing

network. This means that the IMS T9000, with its four links, can be

connected to several different networks. This can be exploited in a number

of ways. For example, two or more networks can be used in parallel to

increase bandwidth, to provide a general purpose communications network and

an independent monitoring/debuggin g network, or as a `user' network

running in parallel with a physically separate `system' network.

-----------------------------------------------------------------

The IMS C104

An important benefit of the IMS T9000's serial links is that it is easy to

implement a full crossbar in VLSI, even with a large number of links. The

use of a crossbar allows packets to be passing through all links at the

same time, making the best possible use of the available bandwidth.

If the routing logic can be kept simple it can be provided for all  the

input links in the router. This avoids the need to share the hardware,

which would cause extra delays when several packets arrive at the same

time. It is also desirable to avoid the need for the large number of packet

buffers commonly used in routing systems. The use of small buffers and

simple routing hardware allows a single VLSI chip to provide efficient

routing between a large number of links.

Wormhole routing

----------------

The IMS C104 (figure 7.2) is one of a family of compatible communications

support devices for the IMS T9000. It includes a full 32 x 32 non-blocking

crossbar switch, enabling messages to be routed from any of its links to

any other link. In order to minimize latency, the switch uses `wormhole

routing' - the connection through the crossbar is set up as soon as the

header has been read. The header and the rest of the packet can start being

transmitted from the output link immediately. The path through the switch

disappears after the `end of packet/message' token has passed through. This

is illustrated in figure 7.3 . This method is simple to implement and

provides very low latency as the entire packet doesn't have to be read in

before the connection is made.

<picture>

Figure 7.2  -- Block diagram of IMS C104

Minimizing routing delays

-------------------------

The ability to start outputting a packet while it is still being input  can

significantly reduce delay, especially in lightly loaded networks. The

delay can be further minimized by keeping the headers short and by using

fast, simple hardware to determine the link to be used for output. The IMS

C104 uses a simple routing algorithm based on interval routing (described

in section 7.3.1).

<picture>

Figure 7.3  -- Packet passing through IMS C104

Because the route through each IMS C104 disappears as soon as the packet

has passed through and the packets from all the channels that pass through

a particular link are interleaved, a single virtual channel cannot `hog' a

route through a network. Messages will not be blocked waiting for another

message to pass through the system, they will only have to wait for one

packet.

Control links

-------------

Like the IMS T9000, the IMS C104 has two control links. One link receives

control and programming information, the other enables all the devices in a

system to be daisy-chained. The routing information for each link of each

IMS C104 is programmed, via the control link, from the controlling

processor.

Subsection 7.2.1 -- Using IMS T9000s with IMS C104s

A single IMS C104 can be used to provide full connectivity between 32 IMS

T9000s. It can also be used to connect other compatible communications

devices, for example to provide an interface to first generation

transputers via a protocol converter, or to peripheral devices via a link

adaptor. IMS C104s can also be connected together to build larger switches

connecting bigger networks of IMS T9000s.

The IMS C104s that the packets pass through do not need to have information

about the complete route to the destination, only which link each packet

should be sent out of at each point. Each of the IMS C104s in the network

programmed with information that determines which output link should be

used for each header value. In this way, each IMS C104 can route packets

out of whichever link will send it towards its destination.

Header deletion

---------------

An approach that simplifies the construction of networks is to provide  two

levels of header on each packet. The first header specifies the destination

transputer (actually, the output link from the routing network), this

header is removed as the packet leaves the routing system. This exposes the

second header which tells the VCP in the destination transputer which

process (actually, which virtual channel) this packet is for. To support

this, the IMS C104 can route packets of any length. Any information after

the initial header bytes used by the IMS C104 is just treated as part of

the packet, even if it is going to be interpreted as a header elsewhere in

the system.The IMS C104 can set any output link to do header deletion, i.e.

to remove the routing header from the front of a packet after it been used

to make the routing decision. The first part of the remaining data is then

treated as a header by the next device that receives the packet.

<picture>

Figure 7.4  -- Header deletion

As can be seen from figure 7.5, by using separate headers to identify the

destination processor and a process within that processor, the labelling of

links in a routing network is separated from the labelling of virtual

channels within each processor. For instance, if the same 2 byte header

were used to do all the routing in a network, then the virtual channels in

all the transputers would have to be uniquely labelled with a value in the

range 0 to 64K. However, by using two 1 byte headers, all the IMS T9000s

can use virtual channel numbers in the range 0 to 255. The first byte of

the header will be used by the routing system to ensure that the packets

reach the appropriate IMS T9000 before the virtual channel number is

decoded.

<picture>

Figure 7.5  -- Using header deletion to label a network

<picture>

Figure 7.6  -- Using header deletion to route through sub-networks

The advantages of using header deletion in a network are:

** It separates the headers, and therefore the routing information, for

virtual channels from those for the routing network.

** The labelling of the network can be done independently of the

application software running on the network.

** There is no limit to the number of virtual channels that can be handled

by a system.

Any number of headers can be added to the beginning of a packet so  that

header deletion can also be used to combine hierarchies of networks as

shown in figure 7.6. An extra header is added to route the message through

each network. The header at the front of each packet is deleted as it

leaves each network to enter a sub-network.

Routing control channels

------------------------

For very large networks, the usual method of connecting control links,  in

a chain, might introduce an undesirable delay. In this case, because of the

common virtual link protocol, an IMS C104 can be used to route the control

links to all the devices in a system more directly, as shown in figure 7.7.

<picture>

Figure 7.7  -- Routing control links through an IMS C104

-----------------------------------------------------------------

Routing algorithms

In order to route a message through a network, an algorithm is required

which is: complete (ensures that all messages arrive); deadlock free;

optimal (packets take the shortest route); scaleable (networks of any size

can be built) and simple to implement.

Subsection 7.3.1 -- Labelling networks

<picture>

Figure 7.8  -- Labelling a network

For each routing component there will be a number of destinations  which

can be reached via each of its output links. Therefore, there needs to be a

method of deciding which output link to use for each packet that arrives.

The addresses that can be reached through any link will depend on the way

the network is labelled. An obvious way of determining which destinations

are accessible from each link, is to have a lookup table associated with

all the outputs (see figure 7.8 ). In practice, this is difficult to

implement. There must be an upper bound on the lookup table size and it may

require a large number of comparisons between the header value and the

contents of the table. This is inefficient in silicon area and also

potentially slow.

<picture>

Figure 7.9  -- Interval labelling

However, a labelling scheme can be chosen for the network such that  each

output link has a <F86>range of node addresses that can be reached

through it. If it is then ensured that the ranges for each link are

non-overlapping, a very simple test is possible. The header just has to be

tested to see into which range, or interval, it falls and, hence, which

output link to use. For example, in figure 7.9, a header with address

<F86>n would be tested against each of the four intervals shown below:

The advantages of interval labelling are that:

** It is `complete' - any network can be labelled.

** It is simple to implement in hardware - it requires little silicon area

which means it can be provided for a large number of links as well as

keeping costs and power dissipation down.

** Because it is simple, it is also very fast, keeping routing delays to a

minimum.

Subsection 7.3.2 -- Avoiding deadlock

Deadlock can occur in a network unless the routing algorithm is designed

to avoid it. Any program with communicating processes can also deadlock if

not designed carefully. It is important here, to distinguish between

deadlock as a property of the network and as a property of a program

running on the network. A deadlock free network cannot cause a program

to deadlock (but, of course, neither can it prevent a badly designed

program from deadlocking). An essential property of a router in a deadlock

free network is that, like a transputer or an IMS C104, it can communicate

on all of its links concurrently.

As a simple example consider a network of four nodes (see figure 7.10) with

one link in each direction between each node. If the routing algorithm

sends all messages clockwise and all nodes start sending to the opposite

corner at the same time, every link will become busy and the network will

deadlock. It is possible to add buffers to the network, but this will only

delay the point at which deadlock occurs. The amount of buffering needed to

avoid deadlock is dependent on the network size and the application program

running on the network.

<picture>

Figure 7.10  -- Deadlock in a network

In this example, deadlock can easily be avoided by modifying the routing

algorithm to send messages in opposite directions from alternate nodes. In

this case, each node will only need to send one message in each direction

at any time. In this network, buffering can be added just to smooth the

flow of data (i.e. to prevent a process having to wait to send a message

when the network is busy) but it is not needed to prevent deadlock.

It is possible to use interval labelling to label any network in a

deadlock free way. Many regular networks have optimal, deadlock free

routing algorithms. Examples are trees, hypercubes and grids. These

networks can then be combined, so that any network can be optimally

labelled as if constructed from these sub-networks.