Other communications devices

To complete the IMS T9000 family, a full range of communications products

are planned. These will provide the ability to interface transputers to a

range of devices and technologies.

-----------------------------------------------------------------

Mixing transputer types: the IMS C100

The first of these devices is the IMS C100. This allows an IMS T9000 to

communicate with a first generation transputer. The two transputer families

have different electrical characteristics and data protocol. The IMS C100

converts between the four wire DS links of the IMS T9000 and the two wire

OS links of the earlier transputers.

The other conversion done by the IMS C100 is between the IMS T9000 control

links and the Reset, Error and Analyse signals used to control the IMS T805

and similar device.

The IMS C100 provides an inter-networking solution for transputers,

allowing transputer systems to be constructed using the optimum mix of

devices. The IMS C100 has four modes of operation to enable:

** A single IMS T9000 to work in a network of first generation transputers.

** An existing transputer system to control a sub-system of IMS T9000s.

** An IMS T9000 network to interface to a network of first generation

transputers.

** A first generation transputer to emulate an IMS T9000.

<picture>

Figure 8.1  -- IMS C100 used with an IMS T9000

The IMS C100 converts both data and control protocols between the two

transputer types and is intended to be used in conjunction with software

running on the attached transputers.

<picture>

Figure 8.2  -- IMS C100 used with a first generation transputer

-----------------------------------------------------------------

Interfacing to peripherals and host systems

To complete the family of communications devices, a range of interface

devices are being designed. These will convert between the serial link

format and a parallel interface, for example. The first of these devices

will interface to a microprocessor bus. This will allow the IMS T9000 to

communicate with non-transputer systems.