Figure 3.1  -- Network of control links

The control links provide an independent communication network which  can

be used to load code, do hardware debugging, monitor a running system for

errors and perform diagnostic functions, both for a single IMS T9000 and a

network.

Because of the great flexibility of the memory interface and the

communications  system of the IMS T9000 there are a number of configuration

registers that need to be programmed. For all of these, the development

tools will program the registers using high level descriptions of the

system. For example, as noted above, there is an interactive tool for

developing configuration data for the PMI. Similarly, the communication

system is set up using high level language descriptions of the software and

hardware networks.

There are two ways of programming the configuration registers: by  writing

to them from a program running on the IMS T9000 itself; or via a control

link from the host system. The first method is used when the system is

booted from ROM, for example in an embedded system. The second method can

be used in a development environment or, in a multi-transputer system,

where only one processor is initialized (or `configured') from ROM and all

the others are configured via their control links from that root processor.

In both cases the IMS T9000 development tools will generate the data to be

programmed into ROM or sent to the control link of a processor.

There are a number of stages of initializing and loading code onto  the IMS

T9000 after it has been reset. These are known as `reset levels' and during

the initialization process, every IMS T9000 must go through each level from

complete reset, to having application code running. Each of these levels

can be done from ROM or through a control link.

-----------------------------------------------------------------

Loading and bootstrapping

The transputer can also be bootstrapped in two ways: from code received

down a link or from ROM. All INMOS development tools generate programs to

be loaded by either method as required during development or in a

production system.

There are a number of advantages to the ability to load code from  a link.

It greatly simplifies the development cycle - there is no need to keep

programming new EPROMs with new versions of code (or use an EPROM

emulator); it can simply be loaded down a link. It simplifies testing of

hardware - a transputer provided with the minimal essential external

signals (5 volts, clock, etc) will be guaranteed to work; there is then 16

Kbytes of on-chip RAM in which to load test code. In a multiprocessor

system, only the root processor needs to be booted from ROM - the others

can be booted down a link with code contained in that system ROM. It is

even possible to switch between ROM and link booting, in order to do field

testing and diagnose faults in an installed system.

-----------------------------------------------------------------

Examples

To show how simple it is to build systems using the IMS T9000, a few

example block diagrams are given here. In the simplest cases these are

almost complete circuit diagrams.

The first example (figure 3.2) is a complete working system using the IMS

T9000s internal RAM as the system memory. The processor boots from ROM

which contains the application software. This processor can communicate

with other transputers or peripherals through its data links. It can be set

to boot from ROM or from link for development and test purposes. The full16

Kbytes of on-chip RAM is available for program workspace.

<picture>

Figure 3.2  -- Complete IMS T9000 system with EPROM

Figure 3.3 shows how a low cost system can be built using a small amount of

SRAM. This could be combined with ROM and peripherals for a low cost

embedded application.

<picture>

Figure 3.3  -- Low cost system with 64 Kbyte of SRAM

The third example in figure 3.4 shows how a large amount of DRAM can be

connected to the IMS T9000 with no external logic for decoding, control

signal generation or buffering.

<picture>

Figure 3.4  -- High performance system with 8 Mbytes of DRAM.

The memory in each bank is enabled by separate strobe signals so all  of

the above memory types could be combined on a single IMS T9000.