Software and systems

INMOS provides a wide range of standard software and hardware products  to

support development for the transputer. These have been designed to enable

users to evaluate transputers and to develop systems easily and within the

shortest possible timescales.

Development tools include compilers for languages such as C, Fortran  and

occam as well as the software needed to test, program and debug systems

built from one or many transputers. All the special features of the

transputer are available from high level languages (either as part of the

language or as library calls).A wide range of software is also available

from third-party suppliers including compilers, such as Ada, and real-time

kernels, e.g. VRTX and C Executive.

INMOS also supplies a range of modular hardware products. These exploit

the ability to build very compact transputer systems (such as an IMS T805

with 4 Mbytes of memory on a board measuring approximately 2.5 cms by 9

cms) to provide a range of small, cost effective `TRansputer Modules'

(TRAMs). These modules can be mounted on a variety of motherboards, which

are available for a range of host systems. The motherboards provide an

interface to the host development system and can be connected to build

larger systems. The standard sizes and interfaces of the modules and

motherboards have been adopted by a number of third party developers to

extend the range of compatible systems products available to transputer

users.

INMOS will continue to support all these standards for the IMS T9000

product family, extending them where necessary to exploit the new features

of these products.

More details of the systems and software products currently available  for

the transputer family can be found in [5].

-----------------------------------------------------------------

Development software

INMOS has a range of development software, running on different hosts,  for

the transputer family. These tools are aimed mainly at developing code for

embedded systems, i.e. not necessarily running under the control of an

operating system. It is expected that the end products will either be

connected to a host system or will be completely self-contained units.

Software can be developed in standard high level languages using

cross-compilers running on a range of host machines. Programs for single

transputers can be developed using just conventional programming tools,

such as compilers and linkers. All the languages include extensive support,

in the form of run-time libraries, for concurrency and communication. It is

possible to write a program consisting of many concurrent processes

entirely in C (or any other language available for the transputer).

Programs written for multi-transputer systems, or programs written as many

sub-programs running in parallel on a single processor, will require the

use of extra programming tools. The transputer development system includes

tools for preparing a program for execution on a parallel processing system

and for debugging such systems. These tools include `configuration' tools

which are used for describing the hardware, mapping processes to

transputers and setting up the communications channels. It is possible to

boot and load a network with code from the host development system, or from

a ROM connected to one of the transputers in the network. Programs can

communicate with a `server' on the host system to get access to host

facilities such as i/o. In addition there are tools for debugging a program

running on a network of transputers. An outline of some of these transputer

specific tools is given below.

All of the programming tools are available for all members of the

transputer family and, where appropriate, are used in the same way and

provide the same functions for all processor types.

Subsection 9.1.1 -- Configuration tools

In discussing IMS T9000 transputer systems, the word `configuration' is

used in two senses. The first is when an IMS T9000 transputer, or an IMS

C104, is initialized - at this time a number of internal `configuration'

registers have to be written to program the PMI, the VCP and other

subsystems. The process of preparing a program for loading onto a

transputer network is also referred to as `configuration' (and the software

tools used are known as `configurers'). In this description of the

development process, the word `configuration' is reserved for the latter

meaning of software configuration; the setting up of the hardware will be

called `initialization'.

The configuration tools are used to build programs consisting of a  number

of processes or sub-programs running in parallel on one or more

transputers. Input files are used to describe the hardware, the software

and a mapping of the software onto the hardware. From these, the

configuration tools produce the files which are used to initialize and load

the transputer network.

Hardware description

--------------------

The hardware is described using a Network Description Language (NDL).  For

each transputer in the system, this specifies the processor type, the

amount and types of memory and peripheral devices. It also describes the

routing network used, if any, and how the data and control links of all the

devices in the system are connected.

The configuration tools use this description to program the PMI and  VCP

registers of the IMS T9000 and to label the links of any IMS C104s used.

The information in this file is also used to create the bootable version of

a program to run on the network.

If certain simple rules are followed in the construction and labelling  of

networks, then the tools can check the descriptions for errors and deadlock

freedom. The NDL description can also be checked against the actual

hardware.

Software description

--------------------

The NDL file for a particular system will normally be provided by  the

hardware vendor or designer. The programmers using the system only need to

include a reference to the NDL file in the software configuration file. The

NDL description exports the names of the processors and routes in the

network for use in the software and mapping description.

The software description has to specify the object code files for  each

process in the system and the procedure interface (parameters and their

types). Optionally, other language dependent attributes can be defined. For

example, the size of stack and heap areas for a C program can be specified.

The software description must also specify the way that any communication

channels are used between processes.

Mapping software to hardware

----------------------------

A mapping of software (processes) onto hardware (transputers) must  also be

given. The mapping can be as simple as a series of statements of the form:

`place process on processor' for each process in the program. Any number of

processes can be placed on each processor, allowing a program to be

initially tested on a single processor before the multi-processor version

is tried. The configuration tools automatically work out the mapping of

channels onto virtual links. If necessary, for example to access the host

system or a particular piece of hardware, the programmer can explicitly map

channels onto links or routes through the network.

Configuration languages

-----------------------

To provide a degree of flexibility for the user, there are two `dialects'

of configuration language: a C-like one and an occam-style one. These

perform identical functions but each has a different syntax, loosely based

on these languages. These configuration languages are used for describing

the structure of the software and how it is mapped onto the hardware.

Types of networks

-----------------

The INMOS development tools support development of programs for:

Networks consisting of IMS T9000 transputers only (`non-routed' networks).

Networks consisting of IMS T9000 transputers and IMS C104 routers (`routed'

networks).

Networks consisting of any other transputer types.

The tools do not directly support arbitrary, mixed networks of IMS T9000

transputers and first generation devices. However, it is possible to

connect the two types of networks, via an IMS C100, although the code for

the two sub-networks has to be developed separately. The two networks can

then be loaded from the host, via a common route.

In the case of non-routed networks (of any transputer type) the

configuration tools automatically add routing software to the program to

provide any communications required between processors which are not

directly connected.

A network of IMS T9000 transputers can be loaded with code compiled for an

IMS T805 or an IMS T9000. This allows users to write programs for the IMS

T9000 even if the compiler used is only available for the IMS T805. It also

means that existing, compiled code can be run on an IMS T9000 system.

Subsection 9.1.2 -- Initializing and loading a network

Transputer systems can be bootstrapped in two ways; either from ROM  or

from link. The initialization and initial code loading are done via the

control link. This initial boot code then loads the main application code

from the data links of the processor.

Levels of initialization

------------------------

The initialization and loading of code for the IMS T9000 are done in a

number of stages. The various levels of initialization can be done either

by code running on an IMS T9000 booted from ROM, or from the host system

via the control link. In a network, different processors may be initialized

to different levels from ROM with the later stages being done via the

control link.

Booting a system from link

--------------------------

The `boot from link' option is normally used during program development  or

whenever a system needs to be able to run different programs at different

times.

In order to load a network from a host system, connections to a single

control link and a single data link are required. This data link normally

goes directly to an IMS T9000, the rest of the network being loaded via

this processor. The development tools generate data files which are used to

do all the initialization and loading of code onto the network.

Booting a system from ROM

-------------------------

The development tools can produce a number of different types of ROM.

These range in function from performing the (partial) initialization of a

single IMS T9000, to booting an entire system.

When booting a system completely from ROM, it is possible to have  a single

ROM on one processor. This root processor boots from the ROM and then

initializes and loads the rest of the network via links; all other

transputers in the network being set to boot from link.

Subsection 9.1.3 -- Host servers

A server is a program that runs on the host machine to give software,

running on an attached transputer system, access to various host facilities

such as i/o and disk storage. The server typically loads the executable

code onto the transputer network via a link interface. It then waits for

requests and data to be sent by the transputer program. These requests

generally come from the run-time library, when the program makes calls to

standard input and output functions (e.g. printf() in C).

The server allows the development tools running on the host to control  the

target transputer system in order to reset the system, do any

initialization needed and then load a bootable program file. Software

running on the host can also use the server to access the transputer system

for testing and debugging.

The nature of the connection from the host to the transputer system

depends on the type of the host system, but generally provides access to

transputer links either directly, via a link adaptor on the host bus, or

through some other standard communications system such as Ethernet. In many

cases the server software includes a device driver, which handles the low

level details of the hardware interface, plus a set of functions to access

the link through the device driver.

Subsection 9.1.4 -- Debugging

INMOS provides an interactive symbolic debugger for debugging programs

running on networks of transputers. This supports source level debugging of

programs which consist of a number of parallel processes running on any

number of processors. The user can set breakpoints, inspect the state of

processes (including expression evaluation, modification of variables,

backtracing procedure calls, etc) as well as examining the low level state

of each transputer in the system. A very useful feature is the ability to

`jump' down a communication channel between two processes - this allows the

state of two communicating processes to be examined.

The debugger is currently being developed further to make it more  powerful

and easier to use. Some of the features that will be added are:

** Window based user interface.

** List all processes running in the network.

** Stop processes to examine their state.

** Source level single stepping.

** History tracing (e.g. keeping track of communications events).

** Variable watchpointing.

Subsection 9.1.5 -- IMS T805 emulation

An IMS T9000 can be booted from a ROM which performs all initialization and

then executes a loader program. The loader then waits for code to arrive on

any of the data links. This emulates the behavior of the IMS T805 which,

after reset, waits for bootstrap code to arrive on a link. With the

addition of an IMS C100 to do protocol conversion, this provides the

ability to plug an IMS T9000 directly into an existing transputer network

and program it as if it were an IMS T805.

Because the IMS T9000 is binary compatible with, and has the same

programming model as, previous transputers the programmer can use existing

development tools, source code, libraries and programming techniques.

This compatibility also makes it easy for systems companies to port

existing software, such as real-time kernels, compilers and so on, that

have already been developed for the current transputer range. This enables

an IMS T9000 specific version of these products to be developed very

quickly. Additional work can then be done, if necessary, to extend the

product to make use of new features of the IMS T9000.

-----------------------------------------------------------------

Iq Systems products

There is already a wide range of TRAMs and motherboards designed for  the

first generation of transputers. This includes modules with transputers

plus various amounts and types of memory, through to various industry

standard interfaces such as SCSI and GPIB. There is a complementary range

of motherboards interfacing to hosts such as PC and Sun.

These TRAMs and industry standard motherboards make it easy to develop,

prototype and build multiprocessor systems, based on the transputer family.

Subsection 9.2.1 -- IMS T9000 products

To support the IMS T9000, a number of products compatible with the existing

TRAM definition are being designed. In the longer term a new module

standard is being defined to exploit the faster links and other features of

the IMS T9000. In addition the approach to systems design using the IMS

T9000 will be somewhat different because of the facilities provided by the

virtual links.

The objectives of the new range of systems products are:

** No jumpers or switches on boards - the user can simply plug together

modules to build a system and start using it.

** No cables for interconnection within a system - all link

interconnections via a backplane and IMS C104s.

** Flexible but fixed network topology - this will be chosen to provide

complete connection between all transputers in a system with minimal

latency.

** Provision of 3.3V for new high speed, low power components.

** Ability to use existing TRAMs where appropriate.

** Ability to build fault tolerant and `live insertion' systems.

** Standard interfaces inside and outside the box.

The opportunity will be taken to increase the modularity of the systems

components, and to improve the design mechanically and provide better

support for peripheral interface connections. To allow flexible

interconnection of boards and modules, a backplane architecture is being

defined to enable the construction of routed and non-routed systems. These

and other changes are being made based on experience and customer feedback.

The new standards will embrace modules, boards and system interconnections.

This includes the construction and interconnection of scaleable systems

with small (1-10) through medium (up to 64) to large (>256) numbers of

processors.

The new module standard will include the provision of a ROM which  may be

used to provide configuration data for the memory interface, etc. In some

cases the processor might boot from this ROM as well. The ROM will also be

used to store useful information about the board, such as the module type,

serial number, vendor, memory size and speed, and information about other

peripheral devices.

The new backplanes will be based on metric standards and will provide  a

standard backplane interface. This will probably follow the board and

connector formats defined by the Futurebus Plus standard.

The transputer links are, naturally, used for connecting between

transputers  on a single board or within a system. There is also a need for

longer connections between systems, for example to support interfacing

between a target system and the development host. Two standards are being

defined; an electrical connection for distances up to about 10 meters, and

a low cost optical fiber interconnect for longer distances.

Compatible development products

-------------------------------

Initially, INMOS will provide a TRAM compatible with current standards  but

containing an IMS T9000. To provide compatibility this will have an IMS

C100 to convert the links and control signals. The IMS T9000 will boot from

ROM so it can be initialized and then be ready to load code down a data

link; it will then appear just like a very fast IMS T805.

This TRAM can be used in an existing development environment to do  initial

evaluation of code running on the IMS T9000. The program running on this

TRAM can communicate but will obviously not have the IMS T9000 advantages

of very high speed links and the virtual channel mechanism.

A similar product will be developed, allowing the IMS T9000 to be used in

an existing development system, but providing direct connection of the IMS

T9000 links. This will enable more realistic multiple IMS T9000 development

to be undertaken.

IMS T9000 specific products

---------------------------

INMOS is developing a range of IMS T9000 modules based on the new standard.

These will range from simple `compute only' modules, with a transputer and

memory, through to interface modules. These will provide access to

standards such as SCSI, FDDI, etc. Motherboards for these modules will also

be supplied, both to the new standard and for popular host computers, such

as the IBM PC.

Host interfaces

---------------

There is a need to provide a new type of interface to efficiently  support

communication between a host system and a program running on an IMS T9000

system.

The existence of multiple virtual links into a network can be exploited  to

simplify the way that software on the transputer accesses host services.

This can also be used to provide all transputers with access to the host.

The handling of virtual channels on the host could be implemented in

hardware for highest performance or software for greatest flexibility and

lowest cost. The choice depends on the capabilities of the particular host

hardware and operating system, as well as user requirements. For example,

the data transfer speed required will be different in a development

situation and an accelerator.

The requirements for connecting into the data link network and the  control

network are quite different. The data links will typically have a

relatively small number of virtual channels connecting to the host, but

will require very high data rates (especially if the IMS T9000 system is

being used an an accelerator or co-processor). There are potentially a very

large number of virtual control links but these can run at a lower data

rate.