THE CHECKOUT PROGRAMS

---------------------

Checkout is a suite of programs for finding and testing

transputer networks.  It consists of a network mapper (CHECK), a

memory tester (MTEST), a functional test (FTEST) and a general

purpose loader (LOAD).  As of release 1.1, two more tools have

been added - a network congruency checker (KONG), and a c004

reading/setting tool (C004).  As of release 1.5, a hex monitor

has been added, (CKMON).  As of release 2.0, KONG and C004 have

been integrated into CHECK.

The output of CHECK is designed to be fed into the other

programs via a pipe or redirection.  The programs can be chained

together to do a complete test of your hardware. A file piped

into CHECK will be used to impose that numbering scheme on the

Transputers, and carries C004 programming information.

The programs are all written in ANSI C and will run on any

host that has an ANSI C Compiler and an INMOS standard link

module to drive the transputer hardware (see the file link.c).

THE PLAYERS

-----------

CHECK - a transputer network mapper.

Determines Transputer types, versions, and topology of a

network. Also determines the speed of the boot link, and does a

basic 5s and As test of the first 2K of internal RAM of each

Transputer it comes across. It does this test down all links it

finds, so each link has about 4K bytes of traffic in each

direction at minimum. CHECK runs completely in internal RAM.

CHECK leaves an ISERVER compatible vestigial router at each

node, that is capable of:

1) setting a default ISERVER-transparent path from the host to

any Transputer in the network,

2) Resetting a Transputer to a state similar to that just after

reset, so it may be subsequently booted by any bootable code file.

This means, for instance, that the Occam compiler may be loaded

and run on any Transputer in the network (see LOAD).

Refer to the file protocol.doc for more information.

CHECK will number processors as a width-first search from the

root processor, starting at 0. This may not be the most

convenient numbering. For instance, with a mesh connection you

may wish to number them by rows and columns. CHECK will do this

by piping in a file.

CHECK takes a template input in one of two forms,

a) A previous CHECK run

b) An INMOS toolset map file.

CHECK will report missing link connections, (useful for

debugging hardware) and additional connections, and will

indicate extra transputers found. It returns an error code of 1

if any transputers or link connections are missing.

C004 connections are reported in one of two ways.

<linkx> - <linky>	says that linkx input connects to linky

output, and linky input connects to

linkx output.

<linkx> > <linky>	says that linkx input connects to linky

output.

An example C004 report line follows. All whitespace is

an arbitrary number of spaces, tabs, carriage returns.

2 C004b (1-5 7>8 23>30 26-28 )

2 C004b   [ -5---1-8 -------- -------U --S-Q--- ]

Both lines report the same configuration.  However, line 1

can be very verbose with big configurations, but is easy to

program. It is reported by CHECK /CL.

Line 2 is of fixed length, and references links as 0-9, A-V.

It is position-dependent - the position determines the output

link, the letter the input link. It is reported as CHECK /C4.

Thus you can write your own.

If you are using the INMOS MMS software, you can configure it

once with the MMS, take a snapshot using CHECK, pipe the output

to a file, and use that file subsequently.

CHECK /h      gives information on the program.

MTEST  -  transputer memory test program

MTEST tests the memory of each transputer in your network.  It

reports sizes and cycle times, and tells you why it stopped

looking for more memory using the X switch. T2s with C004s off

links are not tested unless asked for with the C switch.

In a network, it tests all Transputers concurrently.

Usage:      CHECK | MTEST

Help:       MTEST /H

FTEST  -  transputer functional test

FTEST does a functional test of each transputer in the network.

The tests are taken from the full functional tests done at

probe.  FTEST currently tests T2, T414, T425 and T800.  T801

support will follow soon.

Usage:      CHECK | FTEST

Help:       FTEST /H

LOAD  -  general loader

LOAD utilises the router left in the network by CHECK to allow

arbitrary transputer programs to be placed on any node.

Iserver messages on the boot link are routed to the host by the

remaining CHECK nodes.  This means that you can run the compiler

or any other iserver program on any processor in your network.

if a processor number is specified without a filename, it is

reset, and not loaded. Useful if the bootable file is to load

more than one Transputer. If this is the case, the transputers

to be reset only must be 'downstream' of the one to be loaded.

Usage:      CHECK | LOAD /n program.b8h

where n is the target processor number

ISERVER /ss

Help:       LOAD /H

CKMON - a simple hex monitor for Transputers

This enables you to look at the state of a Transputer after a

program has run or failed, look at the state of the Links, Wptr,

Iptr, and inspect memory, either as words or bytes, or

dis-assemble it as code.

Commands are DW <addr>, DB <addr>, DC <addr> respectively.

This debugger assumes the user has good familiarity with the

operation of Transputers.

Usage:  CKMON /F <filename> /n

where <filename> is the result of a previous CHECK run, to give

the monitor an idea of the topology of the network, and n is the

processor to be inspected according to the map.

--------------------------------------

Sources :- In Check2V5.lzh are the sources required to rebuild

all this on a PC, Sun3, Sun4 (I hope). You do not need an Occam

Compiler to do the port. Source is ANSI and K&R style, #ifdefed

appropriately using __STDC__.

BBS Phone # (719)630-4121.

Cheers,   Andy.

-------------------------------------------------------------------------------

ADDENDUM

-------------------------------------------------------------------------------

The Checkout Tools

The checkout suite of tools enables users to examine and configure networks

of transputers. The toolset consists of:

check  Network tester.

mtest  Memory tester.

ftest  Function test suite.

load  Network loader.

We shall now examine the use of each of these tools.

check

-----

The check tool provides a means of examining a network of transputers. To use

it simply type:

check | mtest -c

NOTE  The c option is used to give us results for the T2 processor.

This will produce the following results:

check 2.2 | mtest 2.01

Part rate Mb Bt [  Link0  Link1  Link2  Link3 ] RAM,cycle

0 T800d-25 0.13 0 [   HOST    1:1    ...    ... ] 4K,1+28K,3+2016K,5+4K,3;

1 T2   -17 0.56 1 [   C004    0:1    ...   C004 ] 2K,1.

In detail,

Processor 0  has the following memory

4K,1  4K bytes of 1 cycle memory (internal RAM) plus,

28K,3  28K bytes of three cycle RAM plus,

2016K,5  2016K bytes of five cycle RAM plus,

4K,3  4K bytes of three cycle RAM.

Processor 1  has the following memory

2K,1  2K bytes of one cycle memory ( internal RAM ).

ftest

-----

The ftest tool performs a functional test on all of the transputers in

the network. The answer should always be OK! To use ftest we type:

check | load -2 myprogram.b8h

This will run a program called myprogram.b8h, on processor two.

Configuring An IMS B008

Introduction

This section will describe how to configure the IMS B008

development motherboard using the checkout tools. For

further details of the checkout tools the reader is referred to

"Using The Checkout Tools".

The IMS C004

The IMS C004 is a 32 way link switch. It can be programmed

to enable any combination of links to be connected in any

manner. All of the spare links on the IMS B008 are brought to

the IMS C004. It is the job of the IMS T222 processor, on the

IMS B008 board,to configure the IMS C004 to set up the links.

The user can either write an explicit program for the IMS T222,

use the SGS-Thomson MMS tools, or can use the checkout

tools.  We shall concentrate on the latter approach.

Configuring Using check

Our aim is to connect the IMS B008 so that the processor in slot

2 of the board is connected to the host processor. We can do

this in one of two ways

Connect a physical link from PipeTail to a spare link.

Connect a link from the last processor to the first via the IMS C004.

To configure the IMS B008 we use the following steps:

check /CL > config.b8

check /CS < config.b8

In detail,

check /CR  to clear the IMS C004.

check /CL > config.b8  Store the current configuration.

edit config.b8  Edit the configuration file.

check /CS < config.b8  Reconfigure the IMS C004.

The file produced from Step 2 will look as follows:

Part rate Mb Bt [  Link0  Link1  Link2  Link3 ]

0 T800d-25 0.13 0 [   HOST    1:1    3:1   ...  ]

1 T2   -17 0.55 1 [    ...    0:1    ...   2:C  ]

2 C004b ()

3 T800c-17 0.13 1 [    ...    0:2    4:1   ...  ]

4 T800c-17 0.13 1 [    ...    3:2    ...   ...  ]

This reads the current transputer configuration, including the

contents of any C004s it finds (the /CL  option).

Next, we must edit the config.b8 file to put in the desired

C004 connections as follows:

Part rate Mb Bt [  Link0  Link1  Link2  Link3 ]

0 T800d-25 0.13 0 [   HOST    1:1    3:1   ...  ]

1 T2   -17 0.55 1 [    ...    0:1    ...   2:C  ]

2 C004b (2-10)

3 T800c-17 0.13 1 [    ...    0:2    4:1   ...  ]

4 T800c-17 0.13 1 [    ...    3:2    ...   ...  ]

This tells the IMS C004 to internally connect its Link2

to its Link10.  On the IMS B008 board this

connects TRAM slot 2 Link0 to slot 0 Link3 .

The final stage is to pump this configuration file down to the

IMS C004. The output from this stage is as follows:-

Part rate Mb Bt [  Link0  Link1  Link2  Link3 ]  CHANGES

0 T800d-25 0.13 0 [   HOST    1:1    3:1   4:0  ]       4+

1 T2   -17 0.55 1 [    ...    0:1    ...   2:C  ]

2 C004b (2-10 )

3 T800c-17 0.13 1 [    ...    0:2    4:1   ...  ]

4 T800c-17 0.13 1 [    0:3    3:2    ...   ...  ] 0+

This has an extra field that describes the changes from the

configuration pumped in to that actually found. On more complex

networks it would mark extra transputers as such, with all the

extra connections. If you wish, you may edit the file in step 2

above to include these proposed extra connections - then check

would automatically match these to the connections found after

configuring the B008. If you wish to use a different numbering

scheme, check will follow that; e.g. processors in slot 1 and

above are numbered from 100.

It is possible on some SGS-Thomson/INMOS boards that check will

not find the C004s in the network. This is because a latch on

the configuration T2 has powered up such that the C004 is held

in a permanently reset state. The solution to this is to do the

following :-

check | mtest /c /t2

Run the Memory test program

mtest on all T2x2s in the system, regargless of whether a C004

is found on the links. mtest ensures that the last thing it

writes to external memory of T2s is 0.

check

This should now find all C004s.

Configuring An IMS B014 Using check

To configure a fully connected IMS B014 VME card we must use the following

configuration script:

check 2.2

Part rate Mb Bt [  Link0  Link1  Link2  Link3 ]

0 T800d-25 0.13 0 [   HOST    1:1    4:1    4:0 ]

1 T2   -17 0.55 1 [    2:C    0:1    ...    3:C ]

2 C004b (1-2 3-6 7-8 9-10 11-12 )

3 C004b (1-2 3-6 7-8 9-10 11-12 )

4 T800d-20 0.76 1 [    0:3    0:2    5:1    5:0 ]

5 T800d-20 0.77 1 [    4:3    4:2    6:1    6:0 ]

6 T800d-20 0.77 1 [    5:3    5:2    7:1    7:0 ]

7 T425a-25 0.77 1 [    6:3    6:2    8:1    8:0 ]

8 T2   -20 0.80 1 [    7:3    7:2    ...    ... ]