LOAD  -  general loader

LOAD utilises the router left in the network by CHECK to allow

arbitrary transputer programs to be placed on any node.

Iserver messages on the boot link are routed to the host by the

remaining CHECK nodes.  This means that you can run the compiler

or any other iserver program on any processor in your network.

if a processor number is specified without a filename, it is

reset, and not loaded. Useful if the bootable file is to load

more than one Transputer. If this is the case, the transputers

to be reset only must be 'downstream' of the one to be loaded.

Usage:      CHECK | LOAD /n program.b8h

where n is the target processor number

ISERVER /ss

Help:       LOAD /H