PROTOCOL LEFT BY CHECK ON NETWORK

---------------------------------

The term ISERVER packets will be used here. This means an occam

protocol of INT16::[]BYTE.

1) From network to HOST.

Each node knows the link down which it was booted. Messages

of ISERVER protocol arriving on any link are passed back to

the boot link.  The router handles packets of any size,

however the special length of 45332 is used internally.

2) From HOST to network.

The system works with the concept of a default path. This

sets a path to a particular node in the network. This node

is referred to as the Target. ISERVER packets are routed

directly to the target. Either they arrive at the

Target as ISERVER packets, or they are stripped of the

length, and only the []BYTE gets to the Target. This mode is

used to boot transputers.

The modes and default path are set by an 'escape' from the

ISERVER protocol. When the packet length is 65535, it marks

an 'escape'.  The normal []BYTE does not follow an INT16

length of 65535. Instead, a control sequence follows:

SETTING THE DEFAULT PATH

The following sequence sets the default path:-

$FF(BYTE), $FF(BYTE), 0(BYTE), INT16::[]BYTE

<-      escape    ->  < set >  < path >

[]BYTE at the end is a succession of link numbers (from 0

through 3) which describe the path. The first byte is the

link out of the root transputer, byte 1 is the link out of

that transputer, and so on. THIS MESSAGE DOES NOT REACH THE

TARGET NODE, It stops one before.

At this point, any ISERVER packet gets to the target node as

an ordinary ISERVER packet, that is INT16::[]BYTE.

SETTING RAW MODE

The following sequence will tell the node just before the

Target node to strip the INT16 length from the packet before

passing it to the Target :-

$FF(BYTE), $FF(BYTE), 2(BYTE)

<-      escape    ->  < boot >

HALTING A NODE

The following sequence will tell the Taget node to prepare

to boot. The Target node (running CHECK code) will reset all

Links except the boot link, and then execute OPR $1FF, the

start instruction. The Transputer is now in a state similar

the that just after reset, and will accept a bootstrap FROM

ANY LINK.

$FF(BYTE), $FF(BYTE), $FF(BYTE)

<-      escape    ->  < halt >