CKMON - a simple hex monitor for Transputers

This enables you to look at the state of a Transputer after a

program has run or failed, look at the state of the Links, Wptr,

Iptr, and inspect memory, either as words or bytes, or

dis-assemble it as code.

Commands are DW <addr>, DB <addr>, DC <addr> respectively.

This debugger assumes the user has good familiarity with the

operation of Transputers.

Usage:  CKMON /F <filename> /n

where <filename> is the result of a previous CHECK run, to give

the monitor an idea of the topology of the network, and n is the

processor to be inspected according to the map.