A Processor to Generate

C code to Communicate Using

occam Protocols

In the course of writing various special servers to support

communications between the host machine and a transputer board it is common to

use some sort of user defined protocol between the host CPU and the transputer.

The occam compiler will provide all of the error and consistency checking

between the SC's but there has been no automated way of ensuring that when the

host CPU communicates with the transputer that it will be using the same tag

constants or the same number and type of variables in the I/O list.  This

document describes the capabilities and usage of a utility which will take

occam source code and generate from it C source code which will aid the

programmer in writing C code for the host CPU to use when communicating with

the transputer.

The name of the utility is cprot.  It is used from the host environment

by typing

cprot [[-m2|-t[2|4|8]]] [-h2|-h4] [-l] protocol.occ

Execution parameters are as follows:

-m2	This parameter tells the processor to generate C code which is

for communicating with an M212 transputer

-t2	This parameter specifies that the host CPU is communicating

with a T212 transputer.

-t4	This parameter causes the processor to generate C code to

communicate with a T414 transputer.

-t8	The processor will generate code for communicating with a T800

transputer when this parameter is used.  Please note that the

parameters -m2, -t2, -t4 and -t8 are mutually exclusive.  If

none of these parameters are specified -t8 will be assumed.

-h2	This parameter causes the generation of code for a host CPU

which has two bytes per word.

-h4	This parameter causes the generation of code for a host CPU

which has four bytes per word.  Notice that the parameters -h2

and -h4 are mutually exclusive.  If both parameters are omitted

then four bytes per word are assumed.

-l	This parameter controls whether the code generated is for

little endian or big endian hosts.  Intel processors are little

endian architectures.  That is, the address of a word is the

address of the least significant byte in the word (this is the

convention the entire transputer family uses).  Motorola

processors are big endian architectures.  If this parameter is

omitted "big endian" is assumed.

When using the processor only protocol declarations and VAL statements

should be in the file being processed.  Also the occam code which will be

communicating with the host should have been fully compiled and linked before

trying to use the host processor to generate C code for the host.  The

processor assumes that the occam code being processed is valid!

The processor will generate two files.  The first is a C header file

containing the #define statements to declare the constants and tags found in

the occam source code file for the C code that will be running on the host.

The second file contains C source code for function routines that can be used

to read and write data via the link adaptor.  The names of the two output files

will be generated from the name of the input file.  If the input file has the

name protocol.occ the header file will have the name protocol.h and the source

code file will have the name protocol.c.  If the occam file contains the

following code

VAL patch.size IS 16:

PROTOCOL draw

CASE

clear.screen

draw.patch; INT; INT; INT; [patch.size][patch.size]BYTE

:

The processor would generate a header file with the following definitions:

#define patch_size      ((tint)16)

#define clear_screen    ((unsigned char)0)

#define draw_patch      ((unsigned char)1)

Notice that all of the occam names which have dots ('.') in them have been

changed.  C uses dots as operators to access subfields in programmer defined

data types.  So the processor replaces dots with the underscore character

('_').  The C source code that the processor would generate would look

something like the following:

#include "protocol.h"   /* where protocol.occ was the name

* of the occam file with the

* declarations in it.

*/

void    read_clear_screen(LinkId, Timeout)

int	LinkId, Timeout;

{

/* This routine needs no code in it since this part of the

* protocol only has a tag associated with it!

*/

}

void    read_draw_patch(LinkId, Timeout, a, b, c, d)

int	LinkId, Timeout;

tint    *a;

tint    *b;

tint    *c;

tbyte   d[patch_size][patch_size];

{

/* Some C code which magically reads the correct things from

* the link and stores it in the correct places!

*/

.

.

.

}

void    write_clear_screen(LinkId, Timeout)

int	LinkId, Timeout;

{

byte_to_link(LinkId, Timeout, clear_screen);

}

void    write_draw_patch(LinkId, Timeout, a, b, c, d)

int	LinkId, Timeout;

tint    a;

tint    b;

tint    c;

tbyte   d[patch_size][patch_size];

{

byte_to_link(LinkId, Timeout, draw_patch);

/* Some C code which magically writes the appropriate

* data up the link to the transputer.

*/

.

.

.

}

The meaning of the data type tint will be dependent upon the execution

parameters which specify the transputer type and the host word size.  It will

always be the C data type with the same size as INT for the transputer being

communicated with.  Also available are the types tint16, tint32, tint64, tbool,

treal32, treal64 and tbyte.

For the user code to use the C code for a read the user should emulate

the occam input code

CHAN OF draw:

INT x.position, y.position, size:

[patch.size][patch.size]BYTE patch:

input ? CASE

clear.screen

...  Clear the screen

draw.patch; x.position; y.position; size; patch

...  Draw patch

with C code like the following:

tbyte   tag;

tint    xpos, ypos, size;

tbyte   patch[patch_size][patch_size];

.

.

.

switch (tag=tbyte_from_link(LinkId, Timeout)) {

case clear_screen:

read_clear_screen(LinkId, Timeout);

/* Clear the screen */

.

.

.

break;

case draw_patch:

read_draw_patch(LinkId, Timeout, &xpos, &ypos, &size,

patch);

/* Draw the patch */

.

.

.

break;

default:

/* Some sort of error handling code */

.

.

.

}

When writing your C server code you will need two special variables.

They are LinkId and Timeout.  These variables are required by the LINK.C

interface supplied by Inmos with the D705B version of the Transputer Tool Set.

They must be passed to the I/O routines which implement the protocols.  The

link you wish to use for I/O must be opened and closed via the LINK.C interface

routines.  The source code for the LINK.C interface is included with the D705B

software and its object code must be linked in for the protocol functions to

work.  For more information about the LINK.C interface refer to your D705B

documentation.