Promload documentation

----------------------

Promload is a Transputer bootable that enables you to test a boot-from-ROM

program on a boot-from-link Transputer.

It opens a hex file given as a parameter and treats it as an EPROM hex dump.

It loads it to just below IBOARDSIZE (an environment variable that gives

the size of your boot-from-link Transputer), moves the workspace pointer to

80000070, and jumps to two below IBOARDSIZE (the ROM entry point).

Usage: iserver -sb promload.btl filename.hex

where filename.hex looks like this :-

0003feec f2 fa 30 f0 e0 7c 73 42 24 f1 25 f2 d4 74 76 41

0003fefc 25 f2 42 24 f1 25 f2 d4 17 74 24 f2 23 f3 42 24

0003ff0c f0 24 f2 fa 30 f0 e0 77 d8 74 24 f2 23 f3 42 24

0003ff1c f0 41 25 f2 24 f2 fa d5 77 73 f4 a6 21 f0 21 f5

0003ff2c 23 07 40 d0 73 70 f9 23 a0 70 75 fa 30 d1 43 71

0003ff3c f4 a9 71 c4 a2 21 00 21 f0 21 f5 70 72 fa 30 7e

0003ff4c 25 f2 70 72 fa e0 0b 70 72 fa 30 7d 25 f2 70 72

0003ff5c fa e0 70 41 25 f2 d0 63 0b ba 22 f0 61 23 2a 04

0003ff6c 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

0003ff7c 00 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00

0003ff8c 00 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00

0003ff9c 00 00 00 01 00 00 00 00 00 00 00 01 00 00 00 00

0003ffac 00 00 00 00 00 00 00 01 00 00 00 00 00 00 00 00

0003ffbc 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01

0003ffcc 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01

0003ffdc 00 00 00 01 00 00 00 00 00 00 00 00 00 00 00 00

0003ffec 00 00 00 01 00 00 00 01 00 00 00 01 00 00 00 00

0003fffc 00 00 69 08

The first (address) field is ignored.

Cheers,  Andy Rabagliati       EMAIL: rabagliatia@isnet.inmos.COM