INTRODUCTION

------------

This document describes two tools which are provided to enable EPROM

programming with the toolset language systems (e.g.  the occam toolset).

Equivalent facilities are provided with the Transputer Development

System.

The tools provided are an interactive means to generate parameters for

the memory interface configuration, and a tool to transform code files

into EPROM images.

The memory interface configuration tool produces timing data from memory

interface parameters.  The parameters you supply can be in a file or in

response to the tool's prompt.  The tool produces a graph of the strobe

timings and a file containing the parameters which can be used to

program a ROM.  There is also an interactive interface to this tool

which allows the user to experiment with configurations.  Graphical

output is in ASCII characters or in PostScript; the tool is provided in

source so that it can be modified for other graphic standards if

necessary.

The EPROM tool reformats the output from a language system into Intel

Hex format ready for loading into EPROM programmers.  This tool is

provided in source so that it can be adapted for use with other EPROM

programmer formats.

The later chapters of the manual contain notes written by the author of

the software which will be of interest to anyone wishing to modify the

source versions of the tools which are provided.

Distribution

The root directory contains the following subdirectories:

src: The C source of the main tools

bin:    Executable binaries of the tools

lib:    Sources and binaries of loaders

These directories should be copied onto your machine, and the bin

directory added to the search path.

EXTERNAL MEMORY INTERFACE PROGRAM

---------------------------------

Introduction

The IMS T414 and IMS T800 transputers have a configurable memory

interface which allows a variety of types of memory device to be

connected using few extra components.  The interfaces on the two

processors are identical so `T414' will be used to mean `T414 or T800'

in this chapter.

The external memory interface configuration program produces timing

diagrams for potential configurations of the memory interface and warns

of possible errors in the design.  It indicates whether one of the

T414's preset configurations would be suitable, or whether it would be

necessary to use an externally programmed configuration.  Should an

external configuration be necessary, the program can produce a table of

data describing it.  This table is read by another program in the

transputer toolset, the EPROM hex program (q.v.), which includes them in

a ROM for the target system.

This chapter describes how to use the program and outlines its

capabilities.  Familiarity with the memory interface of the T414 and the

properties of memory devices will be assumed: relevant information will

be found in the device data sheets and in Inmos Technical Note 9,

Designing with the IMS T414 and IMS T800 Memory Interface .  Section

1 of this note is particularly useful, although the prospective designer

should read the entire document.

Output from the program is either plain ASCII files containing timing

data or PostScript format files containing waveform diagrams.  These

formats were chosen so that the results of the program could be easily

included in reports or other documentation.  The program is capable of

saving and reloading configurations to allow for design over an extended

period and for comparison of different configurations.

The current version of the program does not include the full

characterisation data for the processors.  When these are added, the

timing information generated by the program may change slightly.

Operation

The program is called iemi .  It has no command line options, control

being entirely interactive.

Output from the program is organised into several page s.  As each page

is shown, the user has the option of selecting another page for display

by keying in its number.  The information displayed and options

available on each page will now be described.

Page 0

This page acts as an index to the others.  It shows the title of each

page and permits the selection of one of them.  Other options are also

available, only from this page:

S: save pages to a file

The program prompts for the name of a file to which the data will be

written.  If the file already exists, the opportunity to abandon the

save is given.  Output is in plain ASCII format with a form feed (FF)

character after each page.  It includes full timing information and a

representation of the timing diagrams for read and write cycles.  An

error is reported if the data could not be saved.

L: load previously saved configuration

A filename is prompted for, and the configuration saved in that file is

read in.  The program expects a file in the same format as that in which

it saves configurations.  This means that if the file has been modified

in any way since saving, the operation in not guaranteed to work.

Checks are made on the file, but these are not exhaustive.  In

particular, if any part of page 1 is changed in the saved file, the load

may well fail.

If loading does not succeed because the file has a bad format, the

current configuration is reset to standard configuration 31.  If loading

fails because the file could not be found or could not be opened for

reading, the load is abandoned without losing the current configuration.

P: generate PostScript file

The program prompts for a filename.  If the file already exists, the

user may abandon the operation.  The program writes to the file a

program in the PostScript page description language to draw the timing

diagrams for the chosen memory interface configuration.  The waveforms

shown are the same as those which can be seen by selecting pages 4 and

5.

The file produced fully conforms to the PostScript structuring

conventions, allowing it to be processed by other programs.  The diagram

is designed to fit lengthways on an A4 page, and is suitable for

inclusion in technical notes and reports.  The file can be sent directly

to an Apple LaserWriter or other PostScript output device.

Q: quit

Selection of this option exits the program.

Page 1

This page shows the input parameters to the program.  It is from these

parameters that the program computes the timing information and the

waveforms.  The parameters can be adjusted and the effects of these

changes seen.

The first parameter is the device type.  Knowledge of this enables the

program to deduce the time taken for a half cycle of the signal

ProcClockOut : this is Tm , the basic unit of time of the memory

interface.  The value of Tm for a ClockIn frequency of 5MHz is shown

next.  The number of wait states in the current configuration is

computed from the other parameters and displayed.  The number of periods

Tm which each Tstate T1-T6 lasts is then given, followed by the names

and, where applicable, programmed durations of the strobes

notMemS0 - notMemS4 , notMemRd and notMemWrB .  The strobes each have

two names: one which can be up to 9 characters in length, and one

consisting of just one character.  The short names are used in the

timing information on pages 2 and 3, while the long names are used to

label the waveforms on pages 4 and 5, and in the PostScript output.  The

signal names are initialised to sensible defaults.

The refresh period is given as a number of ClockIn periods (18, 36, 54,

or 72) or as Refresh Off if zero is selected.  The chosen write mode is

shown as Early or Late as appropriate.  The MemWait input may be

connected to one of the strobes S2-S4; alternatively it may be connected

to a simulated external wait state generator that causes

MemWait to become inactive (low) a set number of Tstates after the

start of T2.  Note that this mode is not supported directly by the T414;

in a final design, a circuit would have to be built to perform this

function.  The former case is shown by the strobe name, while the latter

is shown just as the chosen delay.

If the current connection of MemWait causes the signal to become

inactive just as ProcClockOut is falling during T4, a warning that there

is a hazard of a wait race condition is given.  This is because

MemWait is sampled on the falling edge of ProcClockOut -- and if the

signal is changing while being sampled, the result is undefined.

If the currently set configuration happens to correspond exactly to one

of the preset configurations of the T414, the program reports the fact.

When the page is displayed, the user has the option to select a new page

by entering its number, or entering C to change one of the parameters.

In the latter case, a prompt appears requesting selection of the

parameter to be changed.  The possibilities are as follows:

D: device type

A menu of the available devices appears, and the user is invited to

select one.  As elsewhere, just pressing RETURN at this point leaves the

current selection unchanged.

T1...T6: Tstate period

The length of the chosen Tstate can be changed.  Legal values are in the

range 1...4 Tm periods.

S0...S4: strobe names and durations

The durations and names of the strobes can be altered.  Note that S0 is

a fixed strobe, so its duration cannot be changed.  The duration of a

strobe can be 0...31 Tm periods.  If the value for S1 is set to zero,

then notMemS1 stays high throughout the cycle; if the value for S2, S3

or S4 is set to zero, then the strobe is low for the duration of the

cycle.

RS: read strobe names

The names for the read strobe can be altered.

WS: write strobe names

The names for the write strobe can be altered.  Note that because the

four byte write strobes have the same timing, only one is considered.

R: refresh period

The refresh period can be set to one cycle every 18, 36, 54 or 72

ClockIn periods, or to 0 to switch refresh off.

W: wait connection

The wait connection can be set to S2, S3, S4, or to a number in the

range 0...60.  The latter choice corresponds to the connection of

MemWait to a wait state generator that holds the signal high for the

given number of periods Tm after the start of T2.

WM: write mode

The write mode can be set to early or late to suit the type of memory

being used.

C: reset to standard configuration

The parameters can all be reset to those for one of the built in

configurations of the T414.  Valid configuration numbers are 0...11 and

31.

Page 2

This page shows general timing information for the interface, such as

delays between various strobes and required access times of the memory

devices to be used.  The total cycle time is given in nanoseconds and in

processor clock cycles.  No special options are available from this

page.

Page 3

This page gives timing information of special interest to designers

working with dynamic memory, including various access times and the time

for 256 refresh cycles.  Thus can the designer ensure that the

requirements of the memory devices to be used are met.  Again, no

special options are available.

Page 4

Page 4 shows graphically the timing for a memory read cycle.  The

Tstates and strobes are labelled, and bus activity is shown.  The

point where data are latched into the processor is also indicated.  If

the cycle is too long to fit horizontally on the screen, it may be

scrolled left and right using the L and R options.  The displayed area

moves by about 15 characters each time these options are used.

Page 5

Page 5 shows the waveforms for a memory write cycle.  Scrolling to the

left or right is permitted in the same way as on page 4.  The read and

write cycle diagrams are combined when the PostScript output is

produced.

Page 6

This page gives a configuration table for the current configuration.

This is a listing of the data which have to be placed in a ROM situated

at the top of the transputer's memory map in order to achieve the

desired configuration.  The table consists of 36 words of data, but only

the least significant bit in each is used.  The address and contents are

given for each location.  Note that if page 1 indicates that the

configuration is one of the transputer's preset ones, there will be no

need for a ROM; configuration can be achieved by connecting the

MemConfig pin of the device to one of the address/data lines.

EPROM HEX PROGRAM

-----------------

Introduction

It is easiest to boot a transputer network from a host transputer using

one of its links.  But if there is no host computer, you can boot a

transputer from EPROM.

Only one transputer needs to be booted this way because, once booted, a

transputer can boot its neighbours with a program which is stored in the

same EPROM.

This chapter describes several different ways an application can be

booted and explains how they work.

Your application in EPROM

Unless your application has extremely tight demands for space

efficiency, the simple case described first will be sufficient.  If you

use this simple case, most of the design has already been done by INMOS.

The sections following describe more detail and how the system can be

adapted to specific needs.

Network program (simple case)

In this case the program is loaded by a root transputer which has a

EPROM.  The network can consist of one or more processors.

You need only supply file containing a bootable program as produced by

INMOS tools.  The structure of this file is discussed later as is the

function of the precompiled multi-board loader procedure which comes

with the toolset.  The operation of the multi-board loader is similar to

the one described in Loading transputer networks which is Technical

note 34, available from INMOS.

Make sure that the program will run on the network you are using before

you blow the program into EPROM.  This can normally be verified by

booting the program file through the link on the root transputer from

the development host.

To invoke the EPROM software type, for instance:

ieprom /s /p netprog.btr /r 16 /o image

The program will report the storage requirements; the RAM size 0 bytes

indicates that the save on analyse option is disabled.  The data for the

EPROM programmer appears in a file called image .

An example of the simple case

My network consists of only one processor with four 32Kbyte EPROMS and a

T414 and does not need memory configuration.  I want to make the error

light flash.  My EPROM programmer accepts Intel Hex format but cannot

separate the bytes.

The program is written in occam, developed on an IBM pc:

PROC wiggle.error ()

VAL delay IS 5000000:

TIMER time:

SEQ

GUY

CLRHALTERR

PRI PAR

WHILE TRUE

INT now:

SEQ

GUY

TESTERR   --tests and clears error

time ? now

time ? AFTER (now PLUS delay)

GUY

SETERR

time ? now

time ? AFTER (now PLUS delay)

SKIP

:

It is compiled with the e, and w options.

The configuration program is as follows:

#USE "wiggler.c4u"

PROCESSOR 0 T4

wiggle.error ()

And the EPROM image file was created with this command:

ieprom /r128 /i /s   /p wiggle.btr /b

Notice that the input program file has the .btr extension.  The tool

prompts for start addresses of the blocks which is always zero, and the

start addresses of the blocks in EPROM which is also always zero.  The

byte selectors are done one at a time (0, 1, 2, 3).

Main procedure only

To program the machine in this way you must use a language system which

can generate a file which is equivalent to occam toolset files with

.rxx extensions.  This file is specified to the /s option of the EPROM

image creation program.  No /p option is specified -- the transputer can

only execute what has been written into this procedure.

The advantage of programming a special main process over a single

processor network is that it makes more optimal use of both EPROM and

RAM.  No copy of code is made, compared with the network load method

which copies the program into RAM before execution.

The disadvantage is that the EPROM is often slower access memory than

RAM which may have an impact on the speed of execution.

Transputer monitor

Conventional microprocessors' development boards have monitors which are

used to bootstrap the processor.  This is often used with RS232 for

programs to be downloaded from a host or a more sophisticated monitor

may enable programs to boot from a secondary storage device under the

control of the microprocessor.  You can use the transputer in this way

as well, and an example of a monitor for some INMOS boards is provided

with the release.  In practice, it is generally more convenient to use a

link adaptor interface with a host computer.

The transputer monitor is a case of single processor applications which

were discussed in the previous section.

Network program (general case)

The way that the EPROM software is provided means that you have complete

flexibility, even if you are specifying both a loader procedure and a

network program

The occasions when a new loader procedure is needed are not likely to be

met very often.  Either the network program would be in a different

format to the one currently used by INMOS, or the loader needed to

continue execution after all packets had been sent to the network.

EPROM tool input files

The program can take three files as input.  They are:

1) a main procedure

2) a PROGRAM

3) the output from the external memory interface configuration program

Of these only the main procedure must be specified, the others are

optional.

The main procedure

This will either be the final application or a loader program.  In

either case it will be of the xx format as described in the occam2

toolset manual .  The program will read this file and extract from it

the target transputer type and the version number of the file.  An error

is flagged if an the version number is incompatible with the processor

type.

The PROGRAM file

The PROGRAM file is only required when a loader program is being used,

and must be of the .btr type.  Its contents are copied without

modification to the final EPROM for use by the loader.  The file should

therefore be of a format compatible with the chosen loader program.  No

checks are made on the validity of this file.

The memory configuration file

This file is produced using the save command of the external memory

interface configuration program .  The chapter describing that program

should be consulted for further information.

The configuration file is scanned to find the configuration table, which

is then inserted into the EPROM at the correct point for the processor

to read it during initialisation.  If no configuration file is

specified, a blank configuration is stored in the EPROM.  If a

configuration file is specified and the processor type given in the main

procedure file is one without a configurable memory interface, an

appropriate warning message will be printed and the file ignored; no

configuration data will be inserted in the EPROM.

It is important that the file produced by the external memory interface

program not be edited before it is used.  Some checks are made on the

file but these are not exhaustive -- the use of an edited file can

produce unexpected results.

EPROM tool output files

The program can produce output in a form readable by humans or in a form

readable by EPROM programmers.  The supported formats are described

below.  Whichever form is used, it is sometimes necessary to output the

data in blocks.  This block mode operation is also discussed.

Note that there is no output for unused areas of the EPROM -- if the

buffer in the EPROM programmer is not initialised before loading the

files produced by this program into it, unused areas of the EPROM will

be filled with random data.  Although the operation of the bootstrap

code and loader programs will not be affected by the presence of random

data, these areas of the EPROM cannot subsequently be programmed without

erasing the whole device.

Hex dump

This simple format is intended to be used to check the output from the

program.  The dump consists of rows of 16 bytes each, prefixed by the

address of the initial byte of each row.  The format contains no

characters other than the hexadecimal digits, the space character and

newlines.

Intel hex format

This is a commonly used protocol for EPROM programming equipment.  A

sequence of data records is sent.  Each record contains a few bytes of

information, a start address and a checksum.  In addition, a special

record marks the end of a transmission.  Since the format only supports

16-bit addresses, longer addresses are truncated and a warning message

printed.  Records produced by this program contain at most 16 bytes

each.

Intel extended hex format

This format, also known as Intel 86 format, is similar to Intel hex, but

adds another type of record.  The new type 02 record is used to specify

addresses of more than 16 bits.  The type 02 record contains a 16-bit

field giving a segment base offset .  This value is shifted left four

places and added to subsequent addresses.  This mimics the operation of

the segment registers on the Intel 8086 range of microprocessors.  The

segment base offset value persists until the next type 02 record occurs.

This format therefore allows addresses up to 20 bits in length.  Again,

longer addresses will be truncated by the program with a printed warning

to that effect.  The program minimises the number of type 02 records

inserted in its output.

Block mode operation

Block mode must be used if either the programmer cannot separate

out the data bytes in the way described above, or the programmer's

buffer in not large enough to hold all the data for the EPROMs together.

Block mode disabled

If the /b option is not specified on the command line, the program will

write its output to the file specified by the command line /o option.

Once the data file is written, the program exits.  In this simple mode,

addresses in the output file range from zero up to the specified total

EPROM size.  Bytes are output in the order in which they should appear

in the transputer's address space.  If separation of the bytes for

several EPROMs is required, it must be done by the EPROM programmer.

Block mode enabled

If the /b option was given on the command line, then a wider range of

possibilities is available.  Data blocks are output each into a separate

file.  For each block, the start and end addresses in the EPROM space

can be given, together with the corresponding output address.  Further,

if a set of byte-wide EPROMs is being produced to go side-by-side in a

system, it is possible to generate the data required for each byte in

the word separately.

First of all, the program asks for the offset into the EPROMs of the

start of the block to be written.  This offset can be any value greater

than or equal to zero and less than the total EPROM size entered

earlier.  Then the offset into the EPROMs of the end of the block is

prompted for: this can be any value less than or equal to the total

EPROM size and greater than or equal to the start offset.  If the end

offset is equal to the start offset, no data will be output.  Between

them, these two values specify the section of code that is to be

included in this block.

The option is then given to output just the data for one particular byte

in each word, or for the entire word.  If a byte number is entered, then

only bytes whose address has that number as its byte selector will be

output.  This function is useful when preparing side-by-side EPROMs.

The name of the file to which this block of data is to be written is

then prompted for; any filename specified with the command line /o

option is ignored.  The block of data is the written out.

The diagram below illustrates a rather atypical case where a total of

64K of EPROM is being attached to a T414 32-bit processor using sixteen

4Kx8 EPROMs.  Sixteen separate files are to be produced using block

mode.  It is assumed that the programmer has only four kilobytes of

buffer space, starting at address 8K.  This means that the output

address must be 8K for each file, so addresses in the file will range

from 8K to 12K.  The parameters for the sixteen blocks are thus as

follows:

block   start offset  end offset   output   byte

number  into EPROMs   into EPROMs  address  number

0      0K             16K         8K       0

1      0K             16K         8K       1

2      0K             16K         8K       2

3      0K             16K         8K       3

4      16K            32K         8K       0

5      16K            32K         8K       1

6      16K            32K         8K       2

7      16K            32K         8K       3

8      32K            48K         8K       0

9      32K            48K         8K       1

10      32K            48K         8K       2

11      32K            48K         8K       3

12      48K            64K         8K       0

13      48K            64K         8K       1

14      48K            64K         8K       2

15      48K            64K         8K       3

Calling the EPROM tool

The program is controlled both by command line options and

interactively.  The program is called ieprom and its command line

options are as follows:

tabular ll

-s<filename> specify main procedure file name

-p<filename> specify PROGRAM file name

-c<filename> specify configuration file name

-o<filename> specify output filename

-r<decimal number> specify EPROM size in kilobytes

-m<decimal number> specify target machine RAM size in kilobytes

and enable ``copy for analyse''

-h produce simple hex dump

-i use Intel hex output format

-e use Intel extended hex output format

-b enable block mode (see below)

-l load program only and remain silent

tabular

The program then proceeds to construct the EPROM contents.  If one of

the files which is read is found to have a bad format, an error message

is produced and the program terminates.  Otherwise, the program reports

the required amounts of EPROM and RAM.

At this stage the program has, in effect, created an image of the data

that are to appear in the target transputer's memory space in an

internal buffer.  The task now is to transfer the contents of this

buffer into EPROMs for the target system.  Since most EPROM devices are

only eight bits wide, and the transputer data bus is 16 or 32 bits wide,

several EPROMs will be required ``side-by-side''.  In a 16-bit system,

one of the two EPROMs will contain the data bytes at even addresses,

while the other contains those at odd addresses.  In a 32-bit system,

each of the four EPROMs will contain every fourth byte.  This means that

at some stage the data will have to be separated out into the two or

four blocks required, one for each byte in the word size of the

destination system.  Many EPROM programmers can perform this task, but

it may be found more convenient to use separation features, especially

if the programmer's buffer is not large enough to hold the contents of

all the EPROMs simultaneously.

Note that the T212 16-bit transputer has a memory interface with a

``byte access'' mode that allows the use of byte-wide memory devices.

This mode is discussed in detail in the data sheet for the device and

allows the use of a single EPROM for booting the transputer.  If this

mode is being used, separating out the bytes for the EPROM will not be

necessary.

Typical T414 system example

Suppose that it is required to produce four 32Kx8 EPROMs in a T414

transputer system.  The T414 will require the four 8-bit EPROMs to

provide the data on its 32-bit bus.  Since a separate EPROM is needed

for each byte, block mode will have to be used.  The total EPROM size

would be entered as 128K (32K multiplied by 4), and four blocks would be

generated.  Each would have a start offset of zero and an end offset of

128K, and the byte numbers would be specified successively as 0, 1, 2

and 3.  This would produce four files, one for each EPROM.  The EPROM

programmed from the file generated with byte number 0 would ultimately

be connected to data lines 0 to 7, while that with byte number 3 would

be connected to data lines 24 to 31.  The output address that should be

used will depend on the EPROM programmer being employed.

Typical T212 system example

Suppose a single 16Kx8 EPROM is to be programmed for a T212 system using

its ``byte access'' mode mentioned above.  This can be done directly

without the -b flag, entering a total EPROM size of 16K.  The output

file generated will have addresses in the range 0 to 16K.

Alternatively, if a different range of output addresses is required, or

if the programmer cannot handle 64K of data in one pass, block mode must

be used.  When prompted for the byte number, the user should just press

RETURN to allow all bytes to be included in the EPROM.

Repeating options

If a large number of EPROMs is to be programmed on a regular basis, and

block mode is being used, it will be found convenient to automate the

production of the data files.  The simplest way to achieve this is,

during one run, to make a note of the entries made to the program and

subsequently construct a file containing them, one entry per line.  When

the program is next run, the interactive input stream can be redirected

to come from this file.

Initialising the transputer state

The action of the processor on reset, when booting from EPROM, is to

start executing instructions from the location (MOSTPOS INT) - 1: this

is

7FFFFFFE for 32-bit machines and 7FFE for 16-bit machines.  A

backwards jump to the bootstrap code is inserted by the program at this

address.  The bootstrap code first preserves then initialises the

processor state (including the links, the event channel, the timers and

the process queues).

If the copy for analyse option is requested (see below), it also makes a

copy of the bottom 600 bytes of RAM at the top of the RAM area.  This is

done to preserve information which might be useful in determining the

state of the processor before it was analysed, but likely to be

overwritten by a loader procedure.

Calling the main procedure

After the processor is initialised, the main procedure is called.  It

should be declared as follows:

PROC INMOS.ENTRY.POINT (INT entry.point,

[60]BYTE buffer,

VAL[600]BYTE memory.copy,

VAL[]BYTE program.buffer)

SEQ

... the code

:

The parameters are:

entry.point The main procedure should store the offset from MostNeg

INT of successor process (see below) here.  This entry address will also

be the initial value of the workspace pointer for the successor process.

buffer This array contains the state of the transputer before it was

reset; this information will be useful in debugging programs.  When

retyped into an array of INT s, the contents of buffer are as follows:

buffer[ 0]: value of Iptr

buffer[ 1]: value of Wptr

buffer[ 2]: low priority process queue front pointer

buffer[ 3]: low priority process queue back pointer

buffer[ 4]: high priority process queue front pointer

buffer[ 5]: high priority process queue back pointer

buffer[ 6]: error state

buffer[ 7]: value of halt-on-error flag

buffer[ 8]: floating point unit error state (T800 only)

buffer[ 9]: low priority timer value

buffer[10]: high priority timer value

buffer[11]: undefined

buffer[12]: address of the array memory.copy

buffer[13]: undefined

buffer[14]: undefined

memory.copy The 600 bytes of information copied from low memory.

program.buffer The area in the EPROM where the data bytes from the

PROGRAM file are stored.  If (SIZE program.buffer) is zero, then there

is no PROGRAM file in the EPROM.  This happens when no program file is

specified to the EPROM program on its command line.

If the procedure returns, it should leave the entry address and initial

workspace pointer for the successor process in entry.point .  If no

successor process is to be run, the procedure should not return.

Program

There are several ways in which the EPROM hex program can be used,

depending on whether a PROGRAM file is supplied or not.

If no PROGRAM file is given, then it is assumed that the main procedure

file contains the final application program that the system is to run.

This mode is intended for those single processor systems where

performance will not be adversely affected by the fact that the

transputer is running its program from EPROM.  In this case, the

procedure should be written so that it either loops indefinitely or all

processes STOP ; if it ever returns, subsequent operation of the system

is undefined.

In some systems, accesses to EPROM are significantly slower than those

to RAM; in such cases, it can be worthwhile copying the application code

from EPROM into RAM before executing it.  This can be achieved by

supplying the application code as a PROGRAM file to the EPROM hex

program.  The procedure should then be a suitable loader program which

performs the copy from EPROM to RAM and sets up the entry address for

the loaded code as described above.

The EPROM that the program helps to produce may be connected to the

master transputer of a network and be required to boot that network

along its links.  In this case, the procedure should be the multiboard

loader and the PROGRAM file should include the code to run on each

processor in the system.

If present, the network program will be distributed by the main

procedure described above.  Once the loader has distributed all of the

network it exits leaving the processor with the EPROMs as the root

processor of the configuration as described in user manual of the

language system being used.

Memory configuration

The memory configuration file is optional.  Transputers have a number of

selectable standard external memory interfaces.  For transputers with an

external memory interface 32 bits wide, this interface can also be

programmed to a non-standard set-up.  In these cases the memory

interface is read by the transputer after reset from specified addresses

in its address range.  These addresses are within the area covered by an

EPROM which can be used to boot a transputer.

If a memory configuration is required in the EPROM, the configuration

must be provided to the EPROM hex program as a series of address/value

pairs.  In the case of T4 transputers there must be 36 pairs.

EXTERNAL MEMORY INTERFACE PROGRAM NOTES

---------------------------------------

Introduction

It will be found useful when reading the following to refer to a listing

of the program, to copies of the IMS T414 or T800 data sheets, to Inmos

Technical Note 9, ``Designing with the IMS T414 and IMS T800 Memory

Interface'' by Tony Gore and David Cormie, and possibly to a treatise on

the PostScript page description language.

Defined constants

The value SX is the horizontal size of the output device, less 1.  Thus

it will take the value 79 for an 80 column screen.  It should be changed

if the program is to be used with other output devices.

The value noval is a flag value used in the tables of timing information

where a value is not meaningful: the value will be printed as a single

dash character by the function vtos() .

tscale is the scaling used in time calculations.  Since these

calculations are performed in integers, inaccuracies may result if the

nanosecond were used as the basic unit of time.  tscale is normally set

to 10, so that all timings are held internally in units of 0.1

nanoseconds.  This affects the tolerance table and cycle time variables.

The remaining definitions are for indices into the arrays outmin[] ,

outmax[] , start[] and stop[] .

Variables

outmax[] stores the values shown in the `max' column of the timing

tables.

outmin[] stores the values shown in the `min' column of the timing

tables.

start[] holds the number of the Tstate (counting from 0) where each

of the strobes goes low.

stop[] holds the number of the Tstate (counting from 0) where each of

the strobes goes high.  If both start[] and stop[] are equal to the

variable endstate , then that strobe never becomes active.

nst is the number Tstates shown in a display on page 4 or 5.

chst is the number of screen characters occupied horizontally by one

Tstate .

leftst if the number of the leftmost Tstate shown.  This is changed

by scrolling operations.

rtst if the number of the rightmost Tstate shown.  This is also

changed by scrolling operations.

pst is the number of points (1 point = 1/72 inch) occupied

horizontally by one Tstate in the PostScript output.

The tolerance tables, which have the same names as in the occam version,

give characterisation data for the devices, such as the possible skew of

various signals.  The units of time used are dependent on the value of

tscale (see section 1).

dtm[] is half the device cycle time, indexed by device number, again

measured in the time units given by tscale .

waitrace is a flag which is true when there is a wait race hazard.

waitperiods is the total number of wait states added to the memory

cycle.

extrastate is true if the entire cycle lasts an odd number of periods

Tm , and an extra state needs to be added at the end.

endstate is the total memory cycle time in periods Tm .

a is an instance of a struct config and holds the current

configuration.  The structure has the following members:

type , the number for the device type;

t[] , the duration of the Tstate s;

s[] , the duration of each strobe;

rfsh , the interval between refresh cycles in periods of ClockIn , or

zero if refresh is switched off;

wait , the connection of the wait strobe: -1 corresponds to

notMemS2 , -2 to notMemS3 and -3 to notMemS4 .  Nonnegative values

correspond to connection to a wait state generator extending MemWait by

the given number of Tm periods after the start of T2;

wrm , the write mode, which is 0 for early write and 1 for late;

snam[][] , the long names for the strobes;

sch[] , the short names for the strobes.  Strobes are stored in the

order S0...S4, Read, Write.

snam[][] holds the default long names for the strobes.

sch[] holds the default short names for the strobes.

conf holds the number of the current configuration, if it is one of

the preset configurations of the memory interface, and -1 otherwise.

prefig...[] hold the parameters for the preset interface

configurations, and are indexed by the configuration number.

dnam[] holds the names of the devices known to the program, indexed

by the device number 0...8.

fp points to the FILE structure for the stream to which output is

currently directed.  This is normally stdout , but is changed when a

save operation is taking place.

page holds the number of the currently displayed page.

qf is a flag set to true when the user requests that the program

exit.

s[] and t[] are string areas used for many purposes.

pagedraw[] is an array of pointers to functions which handle the

drawing of each page, indexed by page number.

pagekey[] is an array of pointers to functions to handle user input

after each page.  Note that there are only four distinct keyboard

handlers, as some are shared between several pages.

Functions

main() initialises the current configuration's strobe names to their

defaults, and all the elements of arrays outmin[] and outmax[] to

noval .  It resets the horizontal scroll position for pages 4 and 5,

leftst , to zero, the device type to `T414-17' and the wait

connection to 0, so that no wait states will be generated.  By calling

conftoa() it resets all the parameters to the defaults for configuration

31 (the declaration of conf initialises it to 31).  The call to

compute() then works out all the timing information and other

variables so that they are in a consistent state.

fp is set to stdout so that output is to the user's console, and,

while the user has not chosen to quit the program,

pagedraw[page] and pagekey[page] are called alternately.

page0() simply prints the index page, calling centre() where

appropriate.  Note that all printing is done with fprintf(fp,...) so

that when fp points to a real file, the output is saved.

page1() draws the parameter page.  It includes a call to race() to

check the wait race condition and print a warning message if necessary.

Note that the EMI clock period is scaled by tscale so that the value

appears as a time in nanoseconds.  Data for the page are extracted from

the structure a and other variables.  It is important that these be made

consistent by calling compute() and atoconf() before this function.

page2() draws the general timing page.  It calls vtos() to render a

time value either as a number, or as a `-' character if the value is

equal to noval .

page3() draws the dynamic memory timing page.  Again vtos() is called

to print the time values.

page4() calls waves() with parameter 0, so drawing the read cycle

timing.

page5() also calls waves() but with parameter 1, so it draws the

write cycle timing.  Pages 4 and 5 have been integrated in this way

because of the large amount of information which they have in common.

page6() prints the configuration table by calling ctlin() for each

line.  The data to go into the table are taken entirely from the

structure a .

key0() prompts the user for a page number, or to select one of the

other options available from page 0.  If `S' is entered, save() is

called; if `L' is entered, load() is called, and if `P' is entered,

dops() is called to generate a PostScript file.

key1() is called after page 1 is printed, and allows selection of a

new page, or `C' to change one of the displayed parameters.  In the

latter case, changes() is called.

key2() allows only selection of a new page number.

key4() allows selection of a new page, and left and right scrolling

by entering `L' or `R'.  scrolll() and scrollr() are called as

appropriate.

centre() prints the string passed to it in the middle of the line.

It uses the defined constant SX to determine the required indentation.

save() writes out all the pages to a file.  The name is prompted for,

and an attempt is made to open the file for reading.  If this succeeds,

the file is closed and the user is asked if it is to be overwritten.

The user can then abandon the save operation.  The file is then opened

for writing, and fp set to its file handle.  Each of the pagedraw[]

routines is then called in turn, and a form feed character printed after

each.  Finally, the file is closed, and fp restored to stdout .

conftoa() takes a configuration number in conf and loads the

structure

a with the values appropriate to it, by looking up the values in the

prefig...[] arrays.  No change is made if conf is set to -1.

atoconf() takes a configuration in the structure a and checks to see

if it corresponds exactly to any of the preset ones.  If it does, conf

is set to the relevant value, and otherwise to -1.

changes() allows the user to alter any of the available parameters.

It prints a list of the parameters that can be adjusted, and prompts for

one of them.  In each case the old value is printed, and the user

invited to enter a new value.  If no entry is made, the current value is

left unchanged.  All inputs are checked for validity.  After the changes

are made to the values in a , atoconf() is called to check whether the

new configuration is actually the same as one of the preset ones, and

set conf appropriately.  Finally compute() is called to make all the

variables consistent.

vtos() converts a number into a string, returning `-' if the number

was equal to noval .  The resulting string is padded to 5 characters so

that the tables of data in pages 2 and 3 are aligned in columns.

ctlin() is used to print one line of the configuration table for page

6.  It is called with five arguments, v , w , x , y and z .  On each

line two table entries are printed; each consists of an address and a

digit that is either 1 or 0.  v is the line number, from which

ctlin() computes the addresses to be printed.  The two digits printed

are bit x of w and bit z of y .

compute() works out both the activity on all the strobes and the

timing parameters.  It stores the Tstate where each strobe goes low in

the array start[] , and the Tstate where it returns high in the array

stop[] .  It simulates the wait state generator if necessary, and

computes the values of waitperiods , waitrace , extrastate and

endstate .  It checks whether any strobes extend past the end of the

cycle and changes the values accordingly.  Then the entries in

outmin[] and outmax[] are calculated; this part of the program is a

direct translation of the occam original.

dstr0() is a function to draw the top half of the waveform for a

strobe signal.  This consists of just underline characters and spaces.

The function is called by waves() , which draws the strobe signals.

dstr1() is a companion function to dstr0() .  It draws the bottom

half of the waveform for a strobe signal, using underline characters,

strokes in both directions and spaces.  Both dstr0() and dstr1() are

passed a strobe number and extract their information from the arrays

start[] and stop[] .

waves() is responsible for rendering the waveforms on pages 4 and 5.

It takes an argument, p , which determines whether it draws the read

cycle timing or the write cycle timing.  First of all, the width

available to show each Tstate is calculated, as is the total number of

states that can be shown.  These are stored in the variables chst and

nst .  Then the states are labelled across the top of the page, and the

ProcClock waveform drawn underneath.  dstr0() and dstr1() are used to

render the strobes S0-S4 and MemWait .  The activity on the

MemAD lines is rather more difficult to represent, as the bus can be

tristated.  Special inline code is used to draw these signals.  Either

the read or the write strobe is then drawn using dstr0() and

dstr1() , according to the value of p .  A mark is added to indicate

the time (at the end of T5) when data are latched into the transputer.

race() simply prints a warning message if the flag waitrace is true.

It is called by page1() .

scrolll() adjusts leftst so as to scroll the waveform display to the

left.  The move is by about 15 characters.

scrollr() adjusts leftst so as to scroll the waveform display to the

right.  Again, the move is by about 15 characters.

load() is the routine which extracts a configuration from a saved

file.  The user is prompted for a filename, and given the chance to

abandon the load operation.  The file is then searched for a line

containing `Page 1'.  Subsequent lines are read in, and rudimentary

checks are made for validity.  Values are written into the structure a

as they are found.  This means that if the file is found to have bad

format partway through a load operation, the data in a will only be

partially valid.  In this case, the configuration is reset to default

configuration 31, and a message to that effect printed.  The program is

thus sensitive to the saved files being edited, although certain

operations are safe.  For example, all the pages in the saved file can

be deleted as long as page 1 remains intact and retains its header.

Modifying the contents of page 1 is, however, dangerous.

All the remaining functions deal with generating PostScript output.

dot() converts a text string into a PostScript compatible format.

This principally involves inseting ` ' characters before open and close

parentheses.  The text is then printed out right justified to a margin

134 points in from the left hand edge of the page.  This routine is used

in printing the names of signals on the timing diagram.

txt() prints a string at chosen x and y coordinates and is used for

all the other text to appear on the diagram.

hup() prints a small upwards sloping diagonal line suitable for the

rising edges of waveforms.

hdown() prints a downwards sloping line for the falling edges of

waveforms.

alongl() prints a horizontal line one Tstate in length.

alongs() prints a slightly shorter horizontal line.

along() prints a still shorter horizontal line.  The waveforms are

all drawn by these last five functions, called in various sequences.

dwave() draws a strobe signal using the information in start[] and

stop[] , calling the above routines in an appropriate order.

dops() is responsible for the creation of the entire PostScript file.

It obtains a filename, and in the same way as save() , checks whether or

not the file already exists before overwriting it.  It calculates the

number of points horizontally that each Tstate can occupy, and stores it

in pst .  It then generates the prologue for the PostScript file.  The

creation date is not set as there is no standard C library call to

obtain the current date.  When compiled for a particular host, however,

it would be trivial to add the small piece of machine dependent code

necessary.  The fonts used are `Helvetica' and `Helvetica Bold', which

should be supported by most output devices.  The page is turned through

ninety degrees so that the printing is done in landscape mode.  First

the Tstates are labelled, and then the waveforms drawn one at a time.

Unlike the output from pages 4 and 5, the read and write cycle timings

are presented here in one diagram.  dwave() is called to draw the simple

strobes, and the more complicated signals like ProcClock and

MemAD are drawn separately.  A mark is added to show where the read

data are latched into the transputer.  The (empty) trailer is finally

written to the file and the file closed.

Compilation

Since the program uses only standard library functions and header files,

it can be compiled directly on most machines.  The program has been

tested on compilers with 32 bit int s and with 16 bit int s.

EPROM HEX PROGRAM NOTES

-----------------------

Introduction

Since the EPROM hex program needs to read in files at various stages

during its execution, it is impossible to make it completely

machine-independent.  Those parts of the program that deal with files

are therefore isolated in a separate module.  There is one portability

issue that has not been addressed: the program will produce invalid

output files if compiled for a machine which does not use the ASCII code

for its printable characters.

It will be assumed that the reader of this chapter has a listing of the

program to hand.

Defined constants

Constants are defined for the four files that the program has to deal

with and the three output data formats.  CRLF is a string containing an

ASCII carriage return and line feed sequence; escape codes such as n do

not always translate to the same characters across all machines.

MAXCONF is the maximum size in words of a ``configuration table''.

Configuration tables are discussed in the data sheets for the IMS T414

and IMS T800 transputers and in the documentation for the external

memory interface program.  packet and packet give the buffer size used

by the loader programs that distribute code across transputer networks.

The remaining definitions are offsets into the workspace used by the

initialisation code which is ultimately written into the EPROM.

Variables

(*outbyte)() is a pointer to a function which is called with each

byte of data that is to be written.  By changing the value of this

pointer, the data can be directed to the desired sink.

ofp is the handle of the file to which output is being written.

omode indicates the data format selected for output.  It takes on one

of the values IHEX , IEHEX , or PHEX .

obuf is a buffer for 16 bytes of output data.  The first free byte in

the buffer is the one indexed by obptr .

oadd is the address to be sent to the EPROM programmer of the next

byte to be processed.  It is incremented by each call to a routine

pointed to by outbyte() .

sadd is the start address to be sent to the EPROM programmer of the

block that is being constructed in obuf .  Thus oadd is the sum of sadd

and obptr .

startw is the start address of the block that is to be output to the

programmer.

endw is one more than the last address in the block that is being

output to the programmer.

blocks is a flag which is set to 1 when block mode is enabled.

type is the processor type code, extracted from the main procedure

file given to the program.  The types which the program currently

supports are `2', for the T212 16-bit processor, `4' for the T414 32-bit

processor and `8', for the T800 32-bit floating-point processor.  The

value of type is used as an index into the following arrays, which

should be extended if additional processors are to be supported:

bpw[] is the wordlength of the processor in bytes (``bytes per

word'').

bsm[] is the byte select mask for the processor, i.e.  a mask which

extracts those bits in an address which select the byte within the word.

valid[] is an array of flags which indicate when a given processor

number is valid.

mint[] is the value of MostNeg INT for the processor, i.e.

.

confb[] is the length of a configuration table for the processor in

bytes.

confw[] is the length of a configuration table for the processor in

words.

max is the limit on the size of a loader program for the processor.

memstart[] is the number of words reserved by the processor at the

bottom of memory.

modes[] contains strings describing the available output formats.

space holds the total computed RAM requirement of the code being

loaded into EPROM.

workspace is the total workspace memory requirement of the code.

vec is the amount of separate vector space required by the main

procedure.

SC is the offset into the main procedure where it should be entered.

end is the EPROM address where the PROGRAM data finish (and hence

where the main procedure starts).

start is the EPROM offset of the start of the initialisation code.

jmpdest is the destination of the jump instruction which is placed at

the top of the EPROM.  This will either be the same as start or will be

the address of a further jump instruction.  The latter case occurs when

a two-byte instruction is insufficient to reach

start from the end of the EPROM.

files[][] holds the names of the three files which the program may

need to open.

cfa is a flag which is set to 1 if the user requests that code to

copy away the bottom 600 bytes of memory on initialisation be inserted.

This is necessary if it is intended to analyse the target processor with

a debugging tool.

ramsize holds the amount of RAM attached to the target processor,

measured in bytes.

tsize holds the size of the final EPROM.

counter counts the bytes that are output.  It is incremented on each

call to a function pointed to by outbyte() .

hicount holds the highest value that count reaches.  It is thus the

total number of bytes that are to be programmed into the EPROM.

bytesel holds the selection of bytes that are to be output.  Bits in

its value are set according to whether the corresponding byte is to be

sent.

Functions

main() begins by setting omode to its default value of IHEX .  It

then parses its command line arguments by concatenating them into a

single string which is then stepped through one character at a time.

Filenames are copied into the relevant element of files[][] and omode ,

cfa and blocks are set when their controlling switches are

encountered.  tsize and ramsize are also set.  If, when the entire

command line has been processed, no EPROM size has been set or no main

procedure filename has been found, an error is reported.  The other two

input files are optional.

If cfa is set, the program will need to know the total amount of RAM in

the target system --- this is so that it can compute the destination

address for the copy operation.  The value entered is tested to

determine whether it is a power of two; if it is not, the user is

warned.  The twin motivations behind this are that most systems have

just one or two banks of memory devices, so the total memory size is a

power of two and that the user may enter, for example, ``1000K'',

thinking that this is the same as one megabyte.  The memory size in

bytes is stored in ramsize and echoed to the screen for verification.

The function it() is then called to construct the EPROM contents and

produce the output files.

prog() reads in a PROGRAM file, passing its contents to the function

pointed to by outbyte() .  The PROGRAM file has no header that must be

processed, and so this function simply checks that the file could be

opened and then passes on to rcode() .

sc() reads a main procedure file, extracting from the header

information which it requires.  The format is that produced by the IBOOT

tool with the -R (omit bootstrap) option.  The format is as follows:

INT32     descriptor string length

[]BYTE    descriptor string

INT32     id string length

[]BYTE    id string

INT32     target processor

INT32     version number

INT32     scalar workspace requirement in words

INT32     vector workspace requirement in words (version 10 files only)

INT32     program entry point offset in bytes

INT32     program code size in bytes

[]BYTE    program code

The descriptor string length is read in; if the value is unreasonably

large, it is likely that the file is of the wrong type and so an error

message is printed.  If value is reasonable, the string itself is read.

The procedure is repeated for the id string.  The information in the

strings is not used.

The processor type is then read in and checked for validity using the

array valid[] .  Then the version number and workspace requirement are

read in.  If the version number is 10 then the separate vector space

requirement is read; otherwise, the version number must match the

processor type.  Finally the entry point and code length are read, and

rcode() is called to load the code block, sending the data to the

function pointed to by outbyte() .

readint32() is used by sc() to assemble four consecutive bytes from

the input file into a single long integer.

conf() extracts the configuration table from a file saved by the

external memory interface program.  It reads in lines until it finds the

``Page 6'' identifier.  The three subsequent blank lines are skipped and

than the data are read into the array z[] bit by bit.  If an

inconsistency is detected at any point, the routine jumps to the label

conf otherwise the configuration words are output via outword() .

Note that the use of the z[] array as a buffer is necessary as the data

are output in a different order from that in which they appear in the

input file.

openf() calls the external function popen to open the requested file.

The opening is done in text mode for a configuration file and in binary

mode otherwise.

blankconf() produces a null configuration.  This is called instead of

conf() if no configuration file was specified on the command line.

outword() calls outbyte() a number of times appropriate to the word

length of the target processor.

rcode() passes all the remaining bytes from the file whose handle it

is given as argument to the function pointed to by outbyte() , and

closes the file.

instsize() computes the total number of bytes needed to code an

instruction whose operand is passed to the function.  The result is

obtained recursively: if the operand is in the range 0...15 inclusive

(the test op is false) then the result is 1; otherwise the length is one

more than the number to encode the operand, complemented if it is

negative, shifted right four places.

codeop() works by a similar method; it generates the code sequence

required for an instruction by selecting a PFIX or NFIX instruction as

necessary and recursing.  The algorithm is described in ``The Transputer

Instruction Set --- A Compiler Writer's Guide''.

bootcode() generates the transputer initialisation code that is to be

included in the EPROM.  It calls codeop() to assemble each instruction.

The instruction codes are defined in the header file ``m.h''.  If any

changes are contemplated to this routine great care should be taken as

an error in the initialisation code can have disastrous consequences for

the reliability of the target system.  The function instsize() is called

to find the offset required for the forwards jump instructions; the

backward jump offsets are computed by recording the value of counter at

the desired destination and subtracting the value of counter at the

beginning of the jump instruction.  The value is then modified in a

while loop to compensate for the length of the jump instruction itself.

At the end, the code is padded to a word boundary and the sizes of the

various data areas required are calculated.

bounce() is used to insert an extra jump instruction between the

initialisation code and the configuration table.  This will be necessary

if the length of the jump instruction to reach from MostPos INT-1 to the

start of the initialisation code exceeds the two bytes available.  The

routine first checks whether the extra jump is needed; if it is the

correct length is calculated and the code inserted.  The code is padded

to the next word boundary.  The routine returns with jmpdest set to the

value of counter at the start of the initialisation code if no jump was

inserted, and to the inserted jump if one was.

entryjmp() generates the jump from the reset location (MostPos INT-1)

prefixed by two zero bytes.  This ensures correct positioning with both

16- and 32-bit processors.

roundup() pads the code to the next 4-byte boundary with zero bytes.

pass() is the orchestrator of the EPROM code generation.  It calls in

order the functions prog() , sc() , bootcode() , bounce() , conf() and

entryjmp() , recording the correct values of end and start as it goes.

null() is one of the functions to which outbyte() may point.  It has

no action but to increment counter and oadd .

icsum() computes the ``Intel checksum'' of a sequence of bytes.  This

is the two's complement of the modulo 256 sum of the bytes.

send() is the other function to which outbyte() can point.  It checks

whether oadd is between startw and endw (i.e.  whether it is in the

current block) and whether the byte selector of the current address is

one of those chosen in bytesel .  If these conditions are satisfied, a

test is made to see if the current address is in the same segment (64

kilobyte page) as the starting address for the record that is currently

being constructed.  If this is the case and Intel extended hex output

has been selected then the current record must be output by a call to

flush() and a type 02 ``change of segment'' record sent.  Otherwise the

byte to be output is appended to the output buffer obuf .  If the buffer

is now full, it is emptied by calling flush() .  oadd and counter are

both incremented.

flush() writes the current record buffer out to the file specified in

ofp .  If the output mode is one of the Intel hex types, the checksum is

appended to the record and putihln called.  In the simple hex dump mode,

the address of the start of the block, sadd , is printed, followed by

the bytes of data.  The buffer pointer is reset to zero.

endtx() sends an end-of-transmission record appropriate to the output

format in use.  This is null for the simple hex dump mode.

putihln() outputs an Intel hex record.  It adds the opening delimitor

(a colon) and the trailing delimiter ( CRLF ).

it() provides the user interface to the program.  It begins by

finding the total EPROM space required by calling pass() with outbyte()

set to null() .  hicount is set to the highest value that counter

reaches.  At this stage the various ROM and RAM requirements are known

and so are printed out.  If the RAM requirement for the main procedure

is greater than the value given in the

max table entry for the target processor, a warning is given that the

value is too large for the procedure to be used as a loader.  The

selected output format is printed for verification.  The value of tsize

is checked to ensure that the generated code will fit in the final

EPROM; an error is generated if there is not enough space.

If block mode is enabled ( blocks equal to 1) the block number is

printed and the user asked for addresses specifying the area of EPROM to

be programmed and for the byte selection.  These are stored in starto ,

startw , endw and bytesel .  A warning is printed if the selection will

cause an overflow of the addresses available in the chosen output

format.  From these values the correct initial value of oadd is

determined.  A filename is obtained from the user, and ofp set to the

handle for the file.  outbyte() is set to send() so that output actually

occurs and pass() is called.  flush() is called to output the last

record and the output file is closed.  This entire procedure is repeated

until the user elects to stop.

If block mode is disabled, the variables startw and endw are set so that

all the bytes for the EPROM will be output.  Again a test for address

overflow is made and a file handle obtained.  pass() is called to output

the data and flush() is called to write the last record.  The file is

closed if necessary and the routine exits.

Compilation

The program has been compiled on machines with both 16- and 32-bit INTs.

The header file ``m.h'' must be accessible by the compiler.  The

program must be linked with the function popen () whose source is in the

file ``popen.c''.