SUPERVISOR Procedure

--------------------

The purpose of this procedure is to find out  how  "busy"  the

Transputer is.

It returns an INTeger value, the percentage utilization of the

CPU.

To use it  -  this  procedure  is  run  in  parallel  with  an

application, and communicates down two channels, COMMAND (into

SUPERVISOR) and REPLY (out of SUPERVISOR), both CHAN OF INT.

There  are  two  phases  -  a  CALIBRATION  phase,  where  the

procedure  measures  how  fast the scheduling mechanism is - a

function of processor speed  and  whether  it  is  running  in

internal  or  external  RAM.  This  MUST be run when ALL OTHER

PROCESSES are waiting. When SUPERVISOR starts up, it waits for

a signal (an INT) down the COMMAND channel, to signify that it

can start calibration.  It then tells when it is done (approx.

1.6 msec later) by outputting an INT down RESULT.

The  second  phase is the TIMING phase.  During this phase the

COMMAND channel recognises two values -

start (0) This marks the start of the measured period.

read  (1) This  tells  the  SUPERVISOR   to   calculate   the

percentage utilisation since the last start message

is  received,  and  send  the result down the REPLY

channel.

The REPLY channel returns a single integer when prompted to do

so by read. It is the percentage utilisation of the machine.

Some CAVEATs :-

SUPERVISOR MUST be run at low priority.

SUPERVISOR never terminates.  This is done for simplicity,  so

it does not require an ALT in the busy loop.

SUPERVISOR cannot be used to time periods  longer  than  about

2000  seconds.  Otherwise  there is an integer overflow in the

percentage calculation. In the interests of keeping SUPERVISOR

simple,  all calculation was done in integer,  or the accuracy

of the calibration period can be altered.  As a rule of thumb,

the  calibration  period  (in  Ticks) multiplied by the timing

period  (in  seconds)  cannot  be  greater  than  50,000.   If

required, it can be rewritten using floating point.

SUPERVISOR only works on 32 Bit machines,  for the same reason

as above, and also timer range.

SUPERVISOR must have Usage Checking switched off  to  compile.

Again,  this  is  done  to save having to do an ALT.  However,

SUPERVISOR can be separately compiled.

HOW DOES IT WORK ?

------------------

It measures busyness does by measuring the converse - how idle

the machine is.  A process within  SUPERVISOR  reads  the  (Hi

priority)  clock.  It  then  puts itself on the back of the Lo

priority process list.  When it gets to the  front  again,  it

reads  the  time again - and finds out how long it took to get

around again.  If it is less than a certain number of Ticks  -

20  seems  about  right  - SUPERVISOR must be the only process

active. In this instance, it increments an Idle counter.

It times the speed of the loop during the  calibration  phase.

When  asked  to  time  a  period,  it times the length of that

period,  and the rate of increase in  the  Idle  counter,  and

compares  that to the calibration loop,  multiplies it by 100,

and outputs a percentage.